3GPP TSG-RAN WG3 Meeting #124	R3-243348

# Fukuoka, Japan,  20th - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1946 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support User Location Information reporting for NB-IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For NB-IoT NTN, SA2 agreed UE provide the coarse location information to MME via NAS. It is beneficial for MME to provide the coarse location information to eNB, so eNB can determine the right Mapped Cell ID, e.g. to be used as Information on Recommended Cells and eNBs for Paging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new event type and coarse location information in the LOCATION REPORTING CONTROL message. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, the impact can be considered isolated because it only impacts the NB-IoT NTN UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNB cannot know the right Mapped Cell ID for UE, and further operation based on Mapped Cell ID does not work correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.11.1.2, 9.1.12.1, 9.2.1.34, 9.3.3, 9.3.4, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.300 CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: resubmit to RAN3#124. |  |  |  |  |  |  |  |  |

Start of Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.401: "E-UTRAN Architecture Description".

[3]	3GPP TS 36.410: "S1 General Aspects and Principles".

[4]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[5]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[6]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[7]	Void

[8]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[9]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC)".

[10]	3GPP TS 32.422: "Trace control and configuration management".

[11]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for E-UTRAN access".

[12]	3GPP TS 36.414: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 data transport".

[13]	3GPP TS 23.203: "Policy and charging control architecture"

[14]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA), Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[15]	3GPP TS 33.401: "Security architecture".

[16]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRAN); Radio Resource Control (RRC) Protocol Specification".

[17]	3GPP TS 23.272: "Circuit Switched Fallback in Evolved Packet System; Stage 2".

[18]	3GPP TS 48.018: "General Packet Radio Service (GPRS); BSS GPRS Protocol (BSSGP)".

[19]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[20]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA), User Equipment (UE) procedures in idle mode".

[21]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[22]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[23]	3GPP TS 48.008: "Mobile Switching Centre-Base Station System (MSC-BSS) interface; Layer 3 specification".

[24]	3GPP TS 24.301: "Non-Access Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[25]	3GPP2 A.S0008-C: "Interoperability Specification (IOS) for High Rate Packet Data (HRPD) Radio Access Network Interfaces with Session Control in the Access Network".

[26]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[27]	3GPP2 C.S0024-B: "cdma2000 High Rate Packet Data Air Interface Specification".

[28]	3GPP TS 22.220: "Service requirements for Home Node Bs and Home eNode Bs".

[29]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[30] 	3GPP TS 48.016: "General Packet Radio Service (GPRS); Base Station System (BSS) - Serving GPRS Support Node (SGSN) interface; Network service".

[31]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT);Overall description; Stage 2".

[32] 	3GPP TS 29.281: "General Packet Radio Service (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[33]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[34]	3GPP TS 36.455: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)".

[35]	3GPP TS 29.060: "GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[36]	3GPP TS 29.274: "Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[37]	3GPP TS 23.139: "3GPP system – fixed broadband access network interworking".

[38]	3GPP TS 23.007: "Technical Specification Group Core Network Terminals; Restoration procedures".

[39] 	3GPP TS 36.104: "Base Station (BS) radio transmission and reception".

[40]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[41]	3GPP TS 36.306: "User Equipment (UE) radio access capabilities".

[42]	IETF RFC 5905 (2010-06): "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[43]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[44]	3GPP TS 38.413: "NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[45]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[46]	3GPP TS 23.501: "System Architecture for the 5G System".

[47]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[48]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[49]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[50]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[51]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[52]	3GPP TS 38.401: "NG-RAN; Architecture description".

[x]	3GPP TS 37.355: "Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)".

Next Change

### 8.11.1	Location Reporting Control

#### 8.11.1.1	General

The purpose of Location Reporting Control procedure is to allow the MME to request the eNB to report where the UE is currently located. The procedure uses UE-associated signalling.

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Location Reporting Control procedure. Successful operation.

The MME initiates the procedure by sending a LOCATION REPORTING CONTROL message. On receipt of a LOCATION REPORTING CONTROL message, the eNB shall perform the requested location reporting control action for the UE.

The Request Type IE indicates to the eNB whether:

-	to report directly;

-	to report upon change of serving cell, or

-	to stop reporting at change of serving cell.

-	to report the User Location Information.

If reporting upon change of serving cell is requested, the eNB shall report whenever the UE changes its serving cell to another cell belonging to the eNB.

If reporting the User Location Information is requested, the eNB shall, if supported, immediately report the User Location Information based on the UE’s coarse location information in the received Coarse Location Information IE.

If the Additional Location Information IE is included in the LOCATION REPORTING CONTROL message and set to "Include PSCell” then, if EN-DC is activated, the eNB shall include the current PSCell in the report. If a report upon change of serving cell is requested, the eNB shall provide the report also whenever the UE changes the PSCell, and when EN-DC is activated, as specified in TS 23.401 [11].

The Request Type IE also indicates what type of location information the eNB shall report. The location information is E-UTRAN CGI and TAI, or E-UTRAN CGI, PSCell and TAI.

#### 8.11.1.3	Abnormal Conditions

Not applicable.

Next Change

#### 9.1.12.1	LOCATION REPORTING CONTROL

This message is sent by the MME and is used to request the eNB to report where the UE is currently located.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Request Type | M |  | 9.2.1.34 |  | YES | ignore |
| Coarse Location Information | O |  | OCTET STRING | Corresponds to information provided in the Ellipsoid-Point IE as defined in TS 37.355 [x]. | YES | ignore |

Next Change

#### 9.2.1.34	Request Type

The purpose of the Request Type IE is to indicate the type of location request to be handled by the eNB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Request Type |  |  |  |  |  |  |
| >Event Type | M |  | ENUMERATED(Direct, Change of service cell, Stop Change of service cell, …, report ULI) |  |  |  |
| >Report Area | M |  | ENUMERATED (ECGI, …) |  |  |  |
| >Additional Location Information | O |  | ENUMERATED (Include PSCell, ...) |  | YES | ignore |

Next Change

### 9.3.3	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

S1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

SecurityIndication,

SecurityResult,

LTE-NTN-TAI-Information,

CoarseLocationInfo

FROM S1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-SingleContainer{},

S1AP-PRIVATE-IES,

S1AP-PROTOCOL-EXTENSION,

S1AP-PROTOCOL-IES,

S1AP-PROTOCOL-IES-PAIR

FROM S1AP-Containers

id-AssistanceDataForPaging,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-E-RABToBeUpdatedList,

id-E-RABToBeUpdatedItem,

id-CoarseLocationInfo

FROM S1AP-Constants;

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- LOCATION ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- Location Reporting Control

--

-- **************************************************************

LocationReportingControl ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { LocationReportingControlIEs} },

...

}

LocationReportingControlIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID			CRITICALITY reject	TYPE MME-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-eNB-UE-S1AP-ID			CRITICALITY reject	TYPE ENB-UE-S1AP-ID				PRESENCE mandatory	}|

{ ID id-RequestType				CRITICALITY ignore	TYPE RequestType				PRESENCE mandatory	}|

{ ID id-CoarseLocationInfo 		CRITICALITY ignore	TYPE CoarseLocationInfo			PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Location Report Failure Indication

--

-- **************************************************************

LocationReportingFailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { LocationReportingFailureIndicationIEs} },

...

}

Next Change

### 9.3.4	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

CSGMembershipStatus ::= ENUMERATED {

member,

not-member

}

CoarseLocationInfo ::= OCTET STRING

COUNTvalue ::= SEQUENCE {

pDCP-SN			PDCP-SN,

hFN				HFN,

iE-Extensions	ProtocolExtensionContainer { {COUNTvalue-ExtIEs} } OPTIONAL,

...

}

COUNTvalue-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

EPLMNs ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMNidentity

EventType	::= ENUMERATED {

direct,

change-of-serve-cell,

stop-change-of-serve-cell,

...,

Report-ULI

}

E-RAB-ID		::= INTEGER (0..15, ...)

Next Change

### 9.3.6	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

S1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-Constants (4) }

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-M7ReportAmount									ProtocolIE-ID ::= 349

id-TimeBasedHandoverInformation						ProtocolIE-ID ::= 350

id-Bearers-SubjectToDLDiscarding-Item				ProtocolIE-ID ::= 351

id-Bearers-SubjectToDLDiscardingList				ProtocolIE-ID ::= 352

id-CoarseLocationInfo								ProtocolIE-ID ::= x1

END

-- ASN1STOP

End of Change