3GPP TSG-RAN WG3 Meeting #124	R3-243378

# Fukuoka, Japan, 20th – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | <CR#> | rev | <Rev#> | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on inter MN handover without SN change for QMC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current stage 2 description, To ensure QoE measurement continuity during inter-MN handover with SN change, the source SN should provide the information about the SN-associated QMC configurations to the source MN. However, even in case of inter-MN handover without SN change, the source SN needes to provide the the information about the SN-associated QMC configurations to the source MN, since a complete new UE context will be established in the target SN by triggering SN Addition Request procedure by the target MN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that during inter-MN handover without SN change, the source SN should provide the information about the SN-associated QMC configurations to the source MN. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on stage 2 description on inter MN handover without SN change for QMC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether the source SN should provide the the information about the SN-associated QMC configurations to the source MN in case of inter-MN handover without SN change. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 13.4.4	QoE Measurement Continuity for Mobility

For ongoing sessions, QoE measurement continuity is ensured during mobility in NR-DC, e.g., during inter-MN handover (with/without SN change) and SN change scenarios.

To ensure QoE measurement continuity during SN change, the SN-initiated SN modification procedure and/or the MN-initiated SN modification procedure can be used to provide the information about the SN-associated QMC configurations to the MN. The MN can then transfer this information to the new SN during the SN Addition procedure.

To ensure QoE measurement continuity during inter-MN handover with SN change and without SN change, the source SN should provide the information about the SN-associated QMC configurations to the source MN. During the handover procedure, the target MN is provided with all the information that the source MN has about the SN-associated QMC configuration.

If the MN configured the UE with QoE measurements, every subsequent MN serving the UE can configure and release the RAN visible QoE measurements.