3GPP TSG-RAN WG3 Meeting #124	R3-243431

Fukuoka, Japan, 20th – 24th May, 2024

Title:	Discussion on UE Location Information for NB-IoT NTN

Agenda Item:	9.1

Source:	Huawei, Ericsson, Apple

Document for:	Discussion

# 1	Introduction

RAN3 received the Reply LS from SA2 on UE Location Information for NB-IoT NTN in the last meeting [1], with the following question.

To RAN2, RAN3: ACTION: SA2 asks RAN2 and RAN3 whether they think it may be useful for MME to further signal the coarse location information received from the UE in NAS back to eNB e.g. using one of the existing Location Reporting Procedure in order for the eNB to send updated ULI to MME

As observed from [1], SA2 agreed that the NB-IoT UE can report the coarse location information via NAS (SMC) to MME, and from the MME to the E-SMLC. SA2 asked RAN2 and RAN3 whether it may be useful for MME to further signal the coarse location information from the UE in NAS back to eNB.

RAN2 replied an LS to SA2 in the last meeting, with the answer from RAN2 perspective as follows [2].

For the above question, from RAN2 perspective, even if not essential it may be useful for the eNB if the MME further signals the coarse location information received from the UE in NAS back to eNB.

In this contribution, we provide our views on SA2’s question from RAN3 perspective.

# 2	Discussion

## 2.1	Background

In Rel-17, due to the large coverage of NTN cell, RAN2 introduced the coarse location reporting via AS based on SA2 requirements on emergency services and regulatory requirements. RAN3 introduced the mapped cell to inform the MME of the UE location, as request per SA2. However, for NB-IoT UE, RAN2 does not support the coarse location reporting via AS because there is no AS security for NB-IoT CP solution.

In Rel-18, for NB-IoT UEs, the issue was brought up again in RAN2, with the reason that coarse location reporting is a crucial functionally for NTN systems in terms of network efficiency, mobility and even basic operation in regulatory compliance, and LPP is not supported by the vast majority of NB-IoT devices. RAN2 determined the only possible solution is via NAS signalling and an LS was sent to CT1 and SA2.As presented in the introduction part, SA2 sent back the answers in [1], where it mentioned that CRs for TS 23.401 and TS 23.271 are agreed to allow reporting of coarse location information in NAS (SMC) to MME and from MME to E-SMLC.

In the CR, this introduces support for location verification using the EPC-NI-LR LCS procedure based on coarse location information (which corresponds to a granularity of approximately 2 km).  The agreed contents in TS 23.401 is provided as follows.

In case of NB-IoT, the UE and the MME may support reporting of Coarse Location Information from the UE to MME in the NAS Security Mode Command procedure. As described for the Attach procedure (clause 5.3.2.1), TAU procedures (clauses 5.3.3.1 and 5.3.3.2) and Service Request procedure (clause 5.3.4.1), in case of NB-IoT, the MME may, if supported by the UE and if the MME has obtained user consent based on proprietary mechanisms depending on local regulations, request the UE, in a NAS Security Mode Command message, to report its Coarse Location Information. If requested, the UE provides its Coarse Location Information in the NAS Security Mode Complete message. The MME provides the received Coarse Location Information to the E-SMLC as described in clause 9.1.17 of TS 23.271 [57]. The UE indicates its support for providing Coarse Location Information in the UE Network Capability IE, as described in clause 5.11.3. NOTE 1: Provisioning of coarse UE location information in the SMC procedure as described above is intended to enable basic support for UE location verification. If additional accuracy or other positioning features are needed it is assumed that the UE and the network support LPP and the procedures defined in TS 23.271 [57]. Editor’s Note: The Coarse Location Information is analogous to the coarseLocationInfo field defined in 3GPP TS 36.331 [37]. This Editor’s Note is expected to be removed when CT1 has updated the NAS protocol specification.

## 2.2	Answer to the ULI question

As asked in the LS, SA2 asked RAN3 whether it may be useful for MME to further signal the coarse location information from UE in NAS back to eNB, so that the eNB can send the updated mapped cell ID to the CN. The main purpose is copied as follows.

In addition SA2 also observes that the ULI information ("mapped” cell-id and TAC) that will be provided from eNB to MME and stored in CN may potentially not be very accurate given the cells may be quite large and the UE in case of NB-IoT will not be providing coarse location information to eNB. SA2 would therefore like to ask whether RAN2 and RAN3 think it may be useful for MME to further signal the coarse location information received from the UE in NAS back to eNB e.g. using one of the existing Location Reporting Procedure in order for the eNB to send updated ULI to CN. SA2 did not reflect this change in the attached SA2 approved TS 23.401 CR and would like to receive feedback from RAN2 and RAN3.

Basically, the motivation of coarse location information report is to satisfy the SA2 requirements. There are no UE location requirements from RAN side. Although we observe RAN2 replied to SA2 that it may be useful for the eNB if the MME further signals the coarse location information back to the eNB, we doubt whether it is really beneficial. In our understanding, the benefit discussed in RAN2 depends on cases where RAN can know the UE location in appropriated time [4]. But considering this discussion case; the fact is that RAN can know the UE location only when CN sends to it, at Attach. RAN cannot acquire the similar UE location information at any time as the case of AS report. For example, the outdated UE location cannot help the eNB make proper decision on which UE to trigger the ANR measurement or to be redirected as claimed in [4]. The benefit of the UE to eNB relation at one time, the Attach, cannot be claim at anytime in the Network context e.g. with moving mobile, moving cells etc…

In line with the relevance of the “benefit”, we also noticed that the whole proposal depends on an optional UE capability, as UEs are not mandated to support the coarse UE reporting. Then, adding an extra MME to eNB signalling for something that “might” be useful for the eNB, potentially on board with stringent power management, under condition that the UE, subject to low cost with minimum consumption, supports an optional feature, looks weak justification…

Observation: The benefit of a relation between an UE with optimal capability to an eNB, once , at the Attach, cannot be claim at anytime in the Network context e.g. with moving mobile, moving cells etc…

Then again, as far as the RAN is concerned, the UE location is one of the information used to select an appropriate MME (NNSF function); but in this case this has already happened, because the connection with the MME is already successfully established.

Therefore, there is no need for this information in the RAN.

- No need for the MME to signal back the UE coarse location to eNB.

In addition, if the MME has acquired the UE coarse location via the NAS from the UE, then the MME can perform the related actions based on the coarse location directly. Also, if the MME really needs to have the mapped cell ID information, the MME can easily derive it by implementation, e.g., asking the LMF. Therefore, the ULI report does not provide much help.

- No need for the eNB to report any updated ULI to the MME.

In summary, there is no need for the MME to signal back the UE coarse location to the eNB, and report any updated ULI again.

# 3	Conclusion

This document proposes:

Observation: The benefit of a relation between an UE with optimal capability to an eNB, once , at the Attach, cannot be claim at anytime in the Network context e.g. with moving mobile, moving cells etc…

- No need for the MME to signal back the UE coarse location to eNB.

- No need for the eNB to report any updated ULI to the MME.

RAN3 is kindly asked to agreed the response LS in R3-243372 [3].

# 4	References

[1] S2-2403851/R3-241523, Reply LS on UE Location Information for NB-IoT NTN, SA2

[2] R2-2403770/R3-243009, Reply LS on UE Location Information for NB-IoT NTN, RAN2

[3] R3-243372, Reply LS on UE Location Information for NB-IoT NTN, RAN3

[4] R2-2402813, Discussion on reply LS on UE Location Information for NB-IoT NTN