3GPP TSG RAN WG3 Meeting #124	R3-243511

Fukuoka, Japan, 20 - 24 May, 2024

Title:	Reply LS on MDT for NPN

Release:	Rel-18

Work Items:	NR_ENDC_SON_MDT_enh2-Core

Response to:	R3-243011/R2-2403856 LS to RAN3 on MDT for NPN

Source:		RAN3

To:		RAN2

CC:		SA2, SA5

Contact Person:

#### Name:	Angelo Centonza

E-mail Address:	angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachment: None

1. Overall Description:

RAN3 thanks RAN2 for their LS on MDT for NPN.

RAN3 has gone through the questions from RAN2 and it has converged on the following answers.

Answer to questions A), B) and C)

The Mobility Restriction List (see TS 38.413) for a UE allows the UE to move among up to 16 equivalent PLMNs. Additionally, there can be up to 256 unique CAGs per PLMN the UE can be subscribed to.

Thus, to collect MDT measurements for all possible PNI-NPNs the UE may encounter during mobility, the UE would have to be configured with up to 16 PLMN X 256 CAGs. 
However, RAN3 understands that this configuration would be too large to be sent over RRC and for the UE to store and track.

This is why RAN3 has agreed in their specification to a compromise and set the value of 256 PLMN+CAG, while allowing a combination of any PLMN+CAG.

For the above reasons RAN3 suggests that RAN2 increases the number of supported PLMN + CAG configurable at the UE to 16 X 16. This extends the support to 256 PLMN+CAG. 
In the case where all 256 PLMNs are under a single PLMN in the areas scope, the PLMN id in the RRC configuration can be repeated, i.e. the 16 configurable PLMNs would contain the same PLMN ID.

For this solution, RAN3 specifications could be enhanced with amendments in the semantics of the MDT Configuration NR, stating that the PLMN ID + CAG listed in the area scope shall be configurable within the 16 PLMN ID + 16 CAG configuration structure at the UE, where a PLMN ID may be repeated more than once.

Answer to questions D) and E)

RAN3 would like to point out that user consent does not apply to SNPNs, as captured in TS32.422. To RAN3´s understanding, TS32.422 confirms that the configured plmn-IdentityList can be used to indicate the PLMNs serving SNPNs where the UE can collect and report MDT measurements.

2. Actions:

To RAN2

ACTION: RAN3 respectfully asks RAN3 to take the above answers into account and to confirm whether the appropriate changes to their specifications have been applied.

3. Date of Next RAN WG3 Meetings:

TSG-RAN WG3 Meeting RAN3#125			19 - 23 August 2024		Maastricht , NL

TSG-RAN WG3 Meeting RAN3#125bis			14 - 18 October 2024		China (TBC), CN