3GPP TSG-RAN WG3 Meeting #124	R3-243899

Fukuoka, Japan, 20th – 24th May 2024

Agenda item:	  21.2

Source:		Lenovo, Nokia, Nokia Shanghai Bell, Huawei, ZTE

Title:		(TP to BLCR for 37.340) End of Data Burst Indication handling in NR-DC

Document for:		Approval

- Introduction

In the contribution, the TP to BLCR for 37.340 for end of data burst indication handling in NR-DC is provided.

- Text Proposal

### 13.x.2	Awareness

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the PDU Set QoS Parameters as part of the QoS profile to the SN to enable PDU Set based QoS handling.

When the PDCP hosting gNB receives the End of Data Burst Indication of a QoS flow from the UPF, it always provides the End of Data Burst Indication of the QoS flow to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the QoS flow.