3GPP TSG-RAN WG3 Meeting #124	R3-243952

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	11.4

Source:	Nokia, Huawei, Qualcomm, NEC, Samsung, ZTE, CMCC, InterDigital, Ericsson

Title:	(TP for TR 38.743) Continuous Management-based MDT Data Collection

Document for:	Text Proposal

# 1	Introduction

This TP reflects the use case description for Continuous MDT collection targeting the same UE across RRC states.

# 2	Text Proposal for TR 38.743

<<< start of changes >>>

## 5.4	Continuous MDT collection targeting the same UE across RRC states

Editor Note: Capture the description and its potential standard impacts.

## 5.4.1	Use Case Description

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

The Data Collection continuity in this scenario can be split into two tasks as below:

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

<<< end of changes >>>