3GPP TSG-RAN3 Meeting#124 	R3-24xxxx

# Fukuoka City, Fukuoka, Japan,  20th - 24th May. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1163 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on MDT for PNI-NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 sent an LS in R3-243011on the maximum number of PNI NPNs in the Area Scope for MDT, stating that RAN2 supports a maximum of 12 (PLMN) X 12 (CAG) values for UE configuration whereas RAN3 alllows for 256 values of PLMN+CAG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to the semantics description for PNI-NPN based MDT area scope indicating that, for logged MDT, the area scope information should be configurable within the maximum of 12 CAGs per PLMN ID, with a maximum of 12 PLMN IDs, available for UE configuration, where, in such UE configuration, a PLMN ID may be repeated more than once. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mismatch between the RAN3 specifications and the RAN2 specifications for PNI-NPN Area Scope for MDT use in logged MDT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.169 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

#### 9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  | - |  |
| CHOICE Area Scope of MDT | M |  |  |  | - |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - |  |
| >PLMN wide |  |  | NULL |  |  |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAI | M |  | 9.3.3.11 |  | - |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | ignore |
| >>CAG List for MDT |  |  | 9.3.3.65 | The PLMN IDs and CAGs listed in this IE have to be configurable within the maximum of 12 CAGs per PLMN ID, with a maximum of 12 PLMN IDs, available for UE configuration, where, in such UE configuration, a PLMN ID may be repeated more than once. | - |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  | - |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | - |  |
| >SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  | - |  |
| >>>TAI | M |  | 9.3.3.11 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the TAI IE. | - |  |
| >SNPN Based MDT |  |  |  |  | YES | ignore |
| >>SNPN List for MDT |  | 1..<maxnoofMDTSNPNs> |  |  | - |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity IE. | - |  |
| CHOICE MDT Mode | M |  |  |  | - |  |
| >Immediate MDT |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [41]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, other bits reserved for future use. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.171 |  | - |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.172 |  | - |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.173 |  | - |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.174 |  | - |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.175 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>MDT Location Information | O |  | 9.3.1.176 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging Interval | M |  | ENUMERATED (320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 30720ms, 40960ms, 61440ms, infinity, …) | Corresponds to the LoggingInterval IE as defined in TS 38.331 [18]. | - |  |
| >>Logging Duration | M |  | ENUMERATED (10, 20, 40, 60, 90,120, …) | Corresponds to the LoggingDuration IE as defined in TS 38.331 [18]. Unit: [minute]. | - |  |
| >>CHOICE Report Type | M |  |  |  | - |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration | M |  | 9.3.1.180 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >>Area Scope of Neighbour Cells | O |  | 9.3.1.182 |  | - |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [18]. | YES | ignore |
| Signalling Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | - |  |
| PNI-NPN Area Scope of MDT |  | 0..1 |  | This IE is ignored if the Area Scope of MDT IE is set to "PLMN Wide" | YES | ignore |
| >CAG List for MDT | M |  | 9.3.3.65 | The PLMN IDs and CAGs listed in this IE have to be configurable within the maximum of 12 CAGs per PLMN ID, with a maximum of 12 PLMN IDs, available for UE configuration, where, in such UE configuration, a PLMN ID may be repeated more than once. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofMDTSNPNs | Maximum no. of SNPNs in the MDT SNPN list. Value is 16. |

| Condition | Explanation |
| --- | --- |
| C-ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| C-ifM4 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| C-ifM5 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| C-ifM6 | This IE shall be present if the Measurements to Activate IE has the fitth bit set to “1”. |
| C-ifM7 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |

End of Changes