3GPP TSG-RAN WG3 Meeting #124	R3-24xxxx

Fukuoka, Japan, 20 – 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1156 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MDT configuration in MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is not clear that when receiving from AMF, whether the MDT Configuration-NR IE or MDT Configuration-E-UTRA IE should be sent to the SN or not in case of MR-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a MDT Configuration-MRDC IE in MDT Configuration IE to indicates that the MDT Configuration-NR IE is only applicable to MN. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification This CR has an impact under protocol & functional point of view. The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity exists in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.167, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/********************************Start of changes***************************************/

### 8.3.1	Initial Context Setup

<skip unchanged part>

#### 8.3.1.2	Successful Operation

<skip unchanged part>

If the Trace Activation IE is included in the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [11]. In particular, the NG-RAN node shall, if supported:

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT and Trace", initiate the requested trace session and MDT session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT Only", "Logged MDT only", initiate the requested MDT session as described in TS 32.422 [11] and the NG-RAN node shall ignore the Interfaces To Trace IE and the Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling Based MDT PLMN List IE within the MDT Configuration IE, the NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [41].

-	if the Trace Activation IE includes the Bluetooth Measurement Configuration IE within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [41].

-	if the Trace Activation IE includes the WLAN Measurement Configuration IE within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [41].

-	if the Trace Activation IE includes the Sensor Measurement Configuration IE within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [41].

-	if the Trace Activation IE includes the MDT Configuration IE and if the NG-RAN node is a gNB at least the MDT Configuration-NR IE shall be present, while if the NG-RAN node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present.

-	if the Trace Activation IE includes the MDT Configuration-MRDC IE and the MDT Configuration-MRDC IE is set to "MN only" , consider that the MDT Configuration-NR IE or the MDT Configuration-EUTRA IE is only applicable for MN if the UE is configured with MR-DC.

/********************************Next change***************************************/

### 8.4.2	Handover Resource Allocation

<skip unchanged part>

#### 8.4.2.2	Successful Operation

<skip unchanged part>

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [11]. In particular, the NG-RAN node shall, if supported:

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT and Trace", initiate the requested trace session and MDT session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT Only", "Logged MDT only", initiate the requested MDT session as described in TS 32.422 [11] and the target NG-RAN node shall ignore the Interfaces To Trace IE and the Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling Based MDT PLMN List IE within the MDT Configuration IE, the NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [41].

-	if the Trace Activation IE includes the Bluetooth Measurement Configuration IE within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [41].

-	if the Trace Activation IE includes the WLAN Measurement Configuration IE within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [41].

-	if the Trace Activation IE includes the Sensor Measurement Configuration IE within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [41].

-	if the Trace Activation IE includes the MDT Configuration IE and if the NG-RAN node is a gNB at least the MDT Configuration-NR IE shall be present, while if the NG-RAN node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present.

-	if the Trace Activation IE includes the MDT Configuration-MRDC IE and the MDT Configuration-MRDC IE is set to "MN only" , consider that the MDT Configuration-NR IE or the MDT Configuration-EUTRA IE is only applicable for MN if the UE is configured with MR-DC.

/********************************Next change***************************************/

### 8.11.1	Trace Start

<skip unchanged part>

#### 8.11.1.2	Successful Operation

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Trace start

The AMF initiates the procedure by sending a TRACE START message. Upon reception of the TRACE START message, the NG-RAN node shall initiate the requested trace session as described in TS 32.422 [11].

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to "Immediate MDT and Trace", the NG-RAN node shall, if supported, initiate the requested trace session and MDT session as described in TS 32.422 [11].

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to "Immediate MDT Only", "Logged MDT only", the NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [11] and the NG-RAN node shall ignore the Interfaces To Trace IE and the Trace Depth IE.

If the Trace Activation IE includes the MDT Location Information IE within the MDT Configuration IE, the NG-RAN node shall, if supported, store this information and take it into account in the requested MDT session.

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to "Immediate MDT Only", "Logged MDT only" and if the Signalling Based MDT PLMN List IE is included in the MDT Configuration IE, the NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [41].

If the Trace Activation IE includes the Bluetooth Measurement Configuration IE within the MDT Configuration IE, the NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [41].

If the Trace Activation IE includes the WLAN Measurement Configuration IE within the MDT Configuration IE, the NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [41].

If the Trace Activation IE includes the Sensor Measurement Configuration IE within the MDT Configuration IE, the NG-RAN node shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [41].

If the Trace Activation IE includes the MDT Configuration IE and if the NG-RAN node is a gNB at least the MDT Configuration-NR IE shall be present, while if the NG-RAN node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present.

If the PNI-NPN Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the TRACE START message, the NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection in PNI-NPN areas. Upon reception of the PNI-NPN Area Scope of MDT IE, the NG-RAN node shall consider that the area scope for MDT measurement collection in PNI-NPN areas is defined only by the areas included in the PNI-NPN Area Scope of MDT IE.

If the Trace Activation IE includes the MDT Configuration-MRDC IE and the MDT Configuration-MRDC IE is set to "MN only" , the NG-RAN node shall, if supported, consider that the MDT Configuration-NR IE or the MDT Configuration-EUTRA IE is only applicable for MN if the UE is configured with MR-DC.

/********************************Next change***************************************/

#### 9.3.1.167	MDT Configuration

This IE defines the MDT configuration parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT Configuration-NR | O |  | 9.3.1.169 |  |
| MDT Configuration-EUTRA | O |  | 9.3.1.170 |  |
| MDT Configuration-MRDC | O |  | ENUMERATED (MN Only, …) | Indicates that the MDT Configuration-NR IE MDT or the Configuration-EUTRA IE only applies to MN in case of MR-DC. |

/********************************Next change***************************************/

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<skipp unchanged part>

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-ECNMarkingorCongestionInformationReportingStatus,

id-MDT-Configuration-MRDC,

id-XrDeviceWith2Rx,

maxnoofAllowedAreas,

<skipp unchanged part>

MDT-Configuration ::= SEQUENCE {

mdt-Config-NR		MDT-Configuration-NR		OPTIONAL,

mdt-Config-EUTRA	MDT-Configuration-EUTRA		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MDT-Configuration-MRDC		CRITICALITY ignore	EXTENSION MDT-Configuration-MRDC		PRESENCE optional		},

...

}

MDT-Configuration-MRDC ::= ENUMERATED { MN-only,...}

/********************************Next change***************************************/

id-N6JitterInformation									ProtocolIE-ID ::= 424

id-ECNMarkingorCongestionInformationReportingRequest	ProtocolIE-ID ::= 425

id-ECNMarkingorCongestionInformationReportingStatus		ProtocolIE-ID ::= 426

id-ERedCapIndication									ProtocolIE-ID ::= 427

id-XrDeviceWith2Rx										ProtocolIE-ID ::= 428

id-UserPlaneErrorIndicator								ProtocolIE-ID ::= 429

id-SLPositioningRangingServiceInfo						ProtocolIE-ID ::= 430

id-MDT-Configuration-MRDC								ProtocolIE-ID ::= xxx

END

-- ASN1STOP

/********************************End of changes***************************************/