3GPP TSG-RAN WG3 Meeting #124	R3-24xxxx

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	11.4

Source:	Nokia

Title:	(TP for TR 38.743) Continuous Management-based MDT Data Collection

Document for:	Text Proposal

# 1	Introduction

This TP reflects the use case description for Continuous MDT collection targeting the same UE across RRC states.

# 2	Text Proposal for TR 38.743

<<< start of changes >>>

## 5.4	Continuous MDT collection targeting the same UE across RRC states

Editor Note: Capture the description and its potential standard impacts.

## 5.4.1	Use Case Description

In management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state the network does not have standardized means to select a “specific” UE for continuous MDT.

The problem of continuous data collection for management-based MDT can be described as follows: A UE in the RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. The question about maintaining Data Collection continuity in this scenario is two-fold. It is related to:

- Problem A (measurement continuity): How to ensure that the same UE collecting logged MDT in RRC_Idle and RRC_Inactive states will be selected for immediate MDT upon transition to RRC_Connected state.

- Problem B(trace correlation): How to ensure that the TCE that eventually receives the MDT Records can associate the received MDT measurements to a data collection session of measurements from the same UE

<<< end of changes >>>