3GPP TSG-RAN WG3 Meeting #124	R3-243872

Fukuoka, Japan, 20 – 24 May, 2024

Title:	Inventory and Command over CN-RAN interface

Agenda Item:	16.3

Source:	Huawei

Document for:	pCR

# 1	Introduction

This paper provides the pCR to TR 38.769 on Inventory and Command based on the online discussion and offline discussion.

# 2. TP for TR 38.769 (based on R3-243103)

----------------Start of the First Change----------------

## 6.3	Impacts on CN-RAN interface

Editor’s note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

### 6.3.1	General

The purpose of this clause is to identify the functions and procedures supported by CN-RAN interface for AIoT.

### 6.3.1	Information exchanged on the CN-RAN interface for AIoT

#### 6.3.1.1	Inventory

Inventory over CN-RAN interface can be triggered for a single device, or a group of devices, or all devices.

In Topology 1, the Inventory Request from the AIoT CN to the AIoT RAN, may include the following:

(1) AIoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note: It is FFS whether AIoT RAN needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

In both Topology 1 and Topology 2, multiple individual AIoT Device IDs (one ID per device) can be provided to the AIoT CN node via a single Inventory Report.

Editor’s Note: And it is up to SA2 whether device ID is sent transparent or not.

#### 6.3.1.2	Command

Command over CN-RAN interface can be triggered for a single device.

Editor’s Note: it is FFS for command on a group of devices, or all devices.

Editor note: it is FFS whether AIoT RAN can remain agnostic of the type of request over the CN RAN interface (need to differentiate command and inventory)

----------------End of the Changes----------------