3GPP TSG-RAN WG3 Meeting #124	R3-243860

Fukuoka, Japan, 20-24 May, 2024

Agenda Item:	12.2

Source:	Huawei, Xiaomi

Title:	(TP to TR 38.799) On WAB authorization

Document for:	other

1	Introduction

This TP is to introduce the WAB authorization part to TR 38.799 according to following agreements:

WAB Authorization

RAN3 understands that authorization of the WAB-MT is different from the WAB-gNB service authorization/configuration/activation by e.g. OAM/SeGW.

WAB-MT authorization provides the WAB MT with the right to support BH PDU sessions.

In case MT authorization is based on slice, signaling enhancements to the AS layer for the support of WAB-MT authorization are not needed. SA2 to decide whether MT authorization based on slicing is supported.

Authorization of WAB-gNB provides service authorization, i.e., the right to serve UEs.

It is RAN3 understanding that WAB-gNB’s authorization status can change.

The UEs served by a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can either be handed over to other RAN nodes or released.

The NG connection(s) of a WAB-gNB whose authorization status changes from “authorized” to “not authorized” can be removed. It needs to be further studied whether they can be suspended. Any Stage 3 decisions are to be taken during normative work.

CB: # WAB

- Resolve the FFS captured above

- Converge on the TPs below, where agreements taken above will be captured. If any more agreements are taken, they can be included in the TPs below:

  - TP for Architecture (Nokia)

  - TP for Integration procedure (Huawei)

  - TP for Authorization (CATT)

  - TP for Mobility (Ericsson)

  - TP for miscellaneous issues (ZTE)

    - WAB configuration

    - Etc

- SA2 reply LS (Qualcomm)

(Moderator – Docomo)

Summary of offline disc in R3-243844

Annex (TP to TR 38.799)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

4.3	Operational aspects

Editor Note:

- Impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations).

- Inter-gNB- and gNB-to-CN signalling to address the support of WAB.

### 4.3.x	WAB-node authorization

WAB-node authorization inludes the authorization of the WAB-MT and the authorizaiton of WAB-gNB. The authorization of the WAB-MT is different from the WAB-gNB service authorization, configuration or activation by e.g., OAM or SeGW.

Authorization of the WAB-MT provides the WAB-MT with the right to establish BH PDU session. In case WAB-MT authorization is based on slice authorization, signaling enhancements to the AS layer for the support of WAB-MT authorization are not needed.

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The WAB-gNB service authorization may be executed when WAB-gNB connects to the SeGW, or when it connects to the OAM and retrieves the configuration parameters during the plug-and-connect procedure as specified in TS 28.315 [x].

The WAB-gNB’s service authorization status can change. The UEs served by a WAB-gNB whose service authorization status changes from “authorized” to “not authorized” can either be handed over to other RAN nodes or released. And then the NG connection(s) of a WAB-gNB can be removed.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<