3GPP TSG-RAN WG3 Meeting #124	R3-243863

Fukuoka, Japan, 20-24 May, 2024

Agenda Item:	12.2

Source:	Huawei

Title:	(TP to TR 38.799) on integration procedure

Document for:	other

# 1	Introduction

This contribution is to provide TP for the WAB integration according to the following CB:

CB: # WAB

- Resolve the FFS captured above

- Converge on the TPs below, where agreements taken above will be captured. If any more agreements are taken, they can be included in the TPs below:

  - TP for Architecture (Nokia)

  - TP for Integration procedure (Huawei)

  - TP for Authorization (CATT)

  - TP for Mobility (Ericsson)

  - TP for miscellaneous issues (ZTE)

    - WAB configuration

    - Etc

- SA2 reply LS (Qualcomm)

(Moderator – Docomo)

Summary of offline disc in R3-243844

# Annex. TP for TR 38.799 V0.0.1

Start of Change

## 4.3	Operational aspects

Editor Note:

- Impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations).

- Inter-gNB- and gNB-to-CN signalling to address the support of WAB.

### 4.3.A1	WAB-node Integration procedure

Figure 4.3.A1.1 WAB-nodeca integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the way as a UE by performing RRC connection setup procedure with the BH-gNB.The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 steps:

Phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is service authorized by the OAM/SeGW to obtain the right to serve UEs. The OAM can provide the WAB-gNB with the initial configuration to act as a gNB.

Phase 2-2: NG connection setup. The WAB-gNB initiates the NG setup procedure to establish NG connection(s) towards the AMF(s) that will be serving the UEs.

Phase 2-3: Xn connection setup. If needed, the WAB-gNB can initiate the Xn setup procedure towards the neighbouring NG-RAN node(s) to establish the Xn connection(s) towards  the BH-gNB and/or other NG-RAN node(s).

End of Change