3GPP TSG-RAN WG3 #125	R3-244314

Maastricht, NL, 19th – 23rd Aug 2024

Agenda Item:	11.4

Source:	Lenovo

Title: 		Discussion on other leftover issues

Document for:	Discussion and Approval

# Introduction

In this paper, we discuss use case related to left over issues.

# 2	Discussion

## 2.1	Split Architecture

In Release 18 (Rel18) of AI for RAN Work Item (WI), three primary use cases are supported: Load Balancing, Energy Saving, and Mobility Optimization. The following agreements have been captured for E1/F1, but RAN3 didn’t have enough time to thoroughly conclude the standard impact. In this section, we aim to consolidate and discuss the possible effects on the E1/F1 interfaces, taking into account the discussion in Rel18, including those topics that were brought up for discussion.

### 2.1.1	F1 Interface Impacts from Rel18 Use Cases

RAN3 has discussed and concluded in Rel17/18 that the AIML functionality including training and inference will only locate in CU in CU-DU split architecture.

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Energy Saving Enhancements: Measured Energy Cost can be transferred from DU to CU.

In the last RAN3 meeting, it was agreed that the DU will transfer the measured energy cost to CU for AI based network energy saving use case.

Another aspect that RAN3 has not yet discussed is that some prediction information could be transferred from CU to DU as well to optimize the DU decision. Even though the DU does not engage in AI/ML training or inference operations directly, it can still leverage predictions made by the CU through its AI/ML models. These predictions can inform and enhance the operational strategies of the DU. For example,

- Predicted Resource Status per Cell (e.g., predicted radio resource status, number of UEs)

  - In Rel18, CU is capable of predicting the resource status per cell in the future. The prediction result can help DU to proactively handle possible congestion by adjusting its scheduling strategy.

- Predicted UE trajectory

  - In Rel18, CU is capable of predicting the future UE trajectory at cell level. In case of LTM prepared among CU and candidate DUs, the predicted UE trajectory can help the serving DU to optimize its LTM cell switch decision.

RAN3 is suggested to discuss any AIML related information transferred from CU to DU. CU may provide the following predicted information to DU over F1 interface for DU to make use of:

  - Predicted Resource Status per Cell

  - Predicted UE Trajectory

### 2.1.2	E1 Interface Impacts from Rel18 Use Cases

During Rel17 study item, companies agreed that for standalone non-DC scenario, the predicted UE traffic will be used internally at the gNB, i.e., does not send to any neighbour gNB. Thus, RAN3 could at least discuss how to support UE traffic prediction within one gNB, especially between CU-CP and CU-UP considering the CP UP split architecture.

To understand possible E1 interface impact, RAN3 needs to first discuss which node will perform the AI inference and generate the predicted UE data volume, i.e., by CU-CP or CU-UP. Both are possible with different implications in our view:

- Option 1: CU-CP could predict UE data volume based on measured data volume report received from CU-UP

- Option 2: CU-UP could predict UE data volume based on its own measurement and send the predicted UE data volume to CU-CP upon request

Option 1 does not require any AI capability for CU-UP and allows CU-CP to predict the future UE data volume using information collected from other sources other than CU-UP, e.g., traffic characteristics from UE or core network. On the other hand, Option 1 would require CU-UP to provide the measured data volume to CU-CP as demanded for the sake of AI inference. Note that current Data Usage reporting procedure over E1 interface follows instruction from OAM. Thus, to support option 1 enhancement is needed for CU-UP to provide measured data volume as demanded by CU-CP, e.g., upon request from CU-CP.

Comparatively, option 2 would require CU-UP to perform AI inference, and CU-UP has better knowledge of actual UE data volume than CU-CP, which can be used for the prediction. To support option 2 enhancement is needed for CU-UP to send the predicted UE data volume to CU-CP upon request.

No matter which option is supported, it seems some request/response procedure (similar as the resource status request/response procedure) is necessary for the CU-UP to provide the UE data volume measurement or prediction as CU-CP requested. Procedures similar as the new Class 1 and Class 2 procedures over Xn interface can be supported over E1 interface.

RAN3 supports UE traffic (e.g., data volume) prediction with predicted/measured UE traffic (e.g., data volume) transferred between CU-CP and CU-UP with over E1 interface.

RAN3 discusses if UE data volume can be predicted at CU-CP or CU-UP, or both.

A TP in R3-244315 is prepared to capture the analysis above regarding the possible F1/E1 standard impact to support Rel18 use cases in split architecture.

Agree the TP in R3-244315 to capture the F1/E1 standard impact to support Rel18 use cases in split architecture.

## 2.2	Optimization for NR-DC operation

NR Dual Connectivity (DC) in 5G is a technology that allows a device, such as a smartphone or any other user equipment (UE), to be simultaneously connected to two different NG-RAN nodes, namely a Master Node (MN) and a Secondary Node (SN). The primary goal of NR-DC is to enhance network capacity by leveraging the bandwidth from two NG-RAN node, improve the coverage as user could be connected to a macro cell and a small cell at the same time, and strengthen the overall user experience (higher throughput, low latency, low packet loss rate).

However, in the real NR-DC deployment the following challenges are identified especially considering the UE’s mobility:

- Complexity and Energy Consumption: serving the UE with two connections will require close coordination between MN and SN, which will increase the complexity in network management as well as the energy consumption. In some cases, the benefits of employing NR-DC may not justify the effort, especially if the UE exhibits low traffic volume in both uplink (UL) and downlink (DL) and does not have stringent Quality of Service (QoS) requirements.

- Traffic Steering between MN and SN: Establishing NR-DC to serve a UE necessitates a decision from the MN on how to distribute UL/DL traffic between itself and the SN. This distribution must be informed by a thorough understanding of the current resource status of the SN and the anticipated UE UE Traffic (e.g., data volume), ensuring efficient utilization of network resources.

- Service Continuity: In a typical NR-DC configuration, the MN might connect the UE through a macro cell, whereas the SN might use a micro cell. As the UE moves, it may transition between different SNs. The procedure for changing SNs can be initiated by either the MN or SN. However, to maintain optimal service continuity for the UE, the decision to switch SNs must be carefully optimized. This involves not just a seamless transition but also a comprehensive evaluation of the network conditions and the UE's requirements to ensure uninterrupted and high-quality service.

To address the complexities and challenges inherent in NR-DC operations, the deployment of AI/ML-based models presents a promising avenue. ML algorithms can analyze patterns of network usage and UE mobility to anticipate periods of high demand or potential congestion, allowing the network to preemptively adjust resources and optimize traffic distribution between the MN and SN. Furthermore, ML models can facilitate smarter decision-making regarding when and how to engage dual connectivity, thereby minimizing unnecessary energy consumption and extending the battery life of user devices.

From another perspective, in our understanding, NR-DC is only a deployment scenario, and many issues discussed in Rel18 load balancing, energy saving, and mobility optimization are also applicable here in NR-DC. Thus, the AIML models considered in Rel18 load balancing, energy saving and mobility optimization can be used for NR-DC as well. Therefore, it is also suggested to not only consider the mobility optimization issue for NR-DC, but also consider the other issues like load balancing and energy saving in NR-DC scenario.

NR-DC is only a deployment scenario, and many issues discussed in Rel18 load balancing, energy saving, and mobility optimization are also applicable here in NR-DC.

The AIML models considered in Rel18 load balancing, energy saving, and mobility optimization can be used for NR-DC operation as well.

For Rel19 NR-DC scenario, any information/enhancement supported by legacy Rel18 for load balancing and energy saving use cases can be also utilized, e.g., as input/feedback.

Agree to take as baseline for the Mobility Optimization for NR-DC that the use case is studied assuming inference at the MN. The main use case is limited to dual connectivity only (e.g. no conditional procedures are in scope). Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

In the previous RAN3 meeting, it was agreed that RAN3 will focus on the MN initiated SN addition/change scenario, and RAN3 will support the collection of measured UE performance during the procedure.

In the meanwhile, with AIML model deployed at MN, we believe it is also beneficial for MN to provide predicted UE trajectory to the related SN. For example,

- During the SN addition/change, which is similar as the handover procedure, MN can provide any predicted UE trajectory to the target SN.

- MN can also transfer the predicted UE trajectory (that is generated by an AIML model deployed at MN) to the source SN, which can help the source SN for any non AIML based PSCell change (intra-SN or inter-SN).

With AIML model deployed at MN, RAN3 is suggested to support that MN provides predicted information (e.g., predicted UE trajectory) to the related SN (e.g., source SN and target SN).

### 2.2.1 Input/Output/Feedback/Standard Impact

In our understanding, the following are the possible inputs for AIML based NR-DC:

From the serving MN node :

-	UE trajectory prediction (at cell level)

-	Measured/predicted resource status

-	Measured/predicted UE Traffic (e.g., data volume)

-	Measured/Predicted Energy Cost

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

-	UE Mobility History Information.

From the associated SN nodes (can be source SN or target SN):

-	UE’s history information from neighbour

-	Measured/predicted resource status

-	Measured Energy Cost

AIML model for AI/ML-based NR-DC can generate following information as output:

-	UE trajectory prediction (at cell level)

-	UE Traffic (e.g., data volume) prediction

-	SN addition/change recommendation, including recommended candidate PSCells to add/change

-	Traffic steering recommendation, including steering the UE traffic between MN and SN

The following data is required as feedback data for AIML based NR-DC.

-	UE performance from target SN (e.g., throughput, delay, packet loss rate after resource reallocation or after SN addition/change)

-	UE Traffic (e.g., data volume) from target SN

-	Resource status information updates from target SN.

Based on the analysis above, we foresee the following implications for Xn, E1, F1 interfaces

For Xn interface, considering the node level resource status prediction has already been supported in Rel18 Data Collection Initiation/Report procedures, we see some possible impact on DC specific XnAP signaling. For example, RAN3 may enhance the existing SN Addition or SN Change procedure to support:

- Sending the predicted UE Traffic (e.g., data volume) and UE trajectory from MN to the target SN

- MN requesting the target SN to feedback the actual UE Traffic (e.g., data volume), UE trajectory and UE performance after the SN addition/change.

For E1 interface, RAN3 may introduce new procedure to support the exchange of the following between CU-CP and CU-UP:

- Measured/Predicted UE Traffic (e.g., data volume) from CU-UP to CU-CP

- UE performance related measurements from CU-UP to CU-CP

For F1 interface, RAN3 may introduce new procedure to support the exchange of the following between CU and DU:

- UE performance related measurements from DU to CU

A TP in R3-244315 is prepared to capture the analysis above regarding AIML based NR-DC.

Agree the TP in R3-244315 to capture the possible input/output/feedback/standard impact for AIML based NR-DC.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	NR-DC is only a deployment scenario, and many issues discussed in Rel18 load balancing, energy saving, and mobility optimization are also applicable here in NR-DC.

Observation 2	The AIML models considered in Rel18 load balancing, energy saving, and mobility optimization can be used for NR-DC operation as well.

Based on the discussion above, we propose:

Proposal 1	RAN3 is suggested to discuss any AIML related information transferred from CU to DU. CU may provide the following predicted information to DU over F1 interface for DU to make use of:

a.	Predicted Resource Status per Cell

b.	Predicted UE Trajectory

Proposal 2	RAN3 supports UE traffic (e.g., data volume) prediction with predicted/measured UE traffic (e.g., data volume) transferred between CU-CP and CU-UP with over E1 interface.

Proposal 3	RAN3 discusses if UE data volume can be predicted at CU-CP or CU-UP, or both.

Proposal 4	Agree the TP in R3-244315 to capture the F1/E1 standard impact to support Rel18 use cases in split architecture.

Proposal 5	For Rel19 NR-DC scenario, any information/enhancement supported by legacy Rel18 for load balancing and energy saving use cases can be also utilized, e.g., as input/feedback.

Proposal 6	With AIML model deployed at MN, RAN3 is suggested to support that MN provides predicted information (e.g., predicted UE trajectory) to the related SN (e.g., source SN and target SN).

Proposal 7	Agree the TP in R3-244315 to capture the possible input/output/feedback/standard impact for AIML based NR-DC.