3GPP TSG-RAN WG3 Meeting #125	R3-24xxxx

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda item:	11.2

Source:	Nokia, Jio

Title:	(TP to TR 38.743) AI/ML Network Slicing

Document for:	Text Proposal

# 1	Introduction

This paper provides a TP to TR 38.743 to capture the following agreement from the offline session CB: # AIRAN1_Slicing:

Slice UE performance needs to be introduced, while the granularity to support slice UE performance can be further checked in WI phase.

# Annex TP for TR 38.743

The TP is based on TR 38.743 v1.1.0.

<<< start of changes >>>

## 4.1	AI/ML based Network Slicing

### 4.1.1	Use case description

Support of network slicing in NG-RAN is defined in TS38.300 [x].

The NG-RAN plays a key role in taking mobility, load balancing and Radio Resources Management decisions for the purpose of meeting target requirements derived from the SLA of each supported network slice.

AI/ML function can analyze metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements.

### 4.1.2	Solutions and standard impacts

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network slicing:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2	Input data of AI/ML based Network Slicing:

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.1.2.3	Output data of AI/ML based Network Slicing:

AI/ML-based network slicing model in a gNB can generate the following information as output:

- Predicted radio resource status per slice

- Predicted slice available capacity

- Resource management decisions for resources within RRM policies (used by gNB internally)

- Slice aware mobility decisions (used by gNB internally)

#### 4.1.2.4	Feedback of AI/ML based Network Slicing:

To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs:

- Measured Radio resource status per slice

- Measured Slice available capacity

- Legacy UE performance feedback for those UEs handed over from the source gNB

- Finer granularity UE performance feedback for those UEs handed over from the source gNB to determine UE Performance for a certain slice in use by a certain UE.

Note: Exact level of finer granularity to be defined in normative phase.

#### 4.1.2.5	Potential standard impacts:

Following standard impacts is listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

-	Enhanced existing procedure to collect predicted information between gNBs:

■	Predicted radio resource status per slice

■	Predicted slice available capacity

<<< end of changes >>>