3GPP TSG-RAN WG3 Meeting #126				  	                  R3-247854

Orlando, USA, 18th  – 22nd November 2024

Title:	LS on QMC Coordination for RRC Segmentation in NR-DC

Response to:	-

Release:	Rel-18

Work Item:	NR_QoE_enh-Core

Source:	ZTE [shall be RAN3]

To:	RAN2

Cc:

Contact person:	Yansheng Liu

liu.yansheng@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 is currently discussing introduction, as correction to Rel-18, of inter-gNB coordination in support of enabling and disabling of RRC segmentation for QoE reporting on SRB4 and SRB5 in the NR-DC scenario.

In RAN3’s understanding, according to TS 38.331 clause 5.3.5.13d, is that the network performs QMC configuration or reconfiguration by sending the RRC AppLayerMeasConfig-r17 IE to the UE. The UE determines whether to enable or disable RRC segmentation for QoE reporting on SRB4 and SRB5 based on the presence of the rrc-SegAllowedSRB4-r17 IE and the rrc-SegAllowedSRB5-r18 IE within the RRC AppLayerMeasConfig-r17 IE.

With respect to the above, RAN3 kindly asks RAN2 the following:

- Can the presence of the rrc-SegAllowedSRB4-r17 parameter be sent to the UE, by the SN? Correspondingly, can the presence of the rrc-SegAllowedSRB5-r18 parameter be sent to the UE, by the MN?

- If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, how does the UE treat its RRC segmentation state for SRB5 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB5 or does the UE consider that the RRC segmentation function for SRB5 shall be disabled?

- If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, how does the UE treat its RRC segmentation state for SRB4 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB4 or does the UE consider that the RRC segmentation function for SRB4 shall be disabled?

2	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to reply the questions above.

3	Dates of next RAN3 meetings

RAN3#127			17th – 21st  February 2025 		Athens, GR

RAN3#127-bis			7th – 11th April 2025 			TBD, CN