3GPP TSG-RAN WG3 #126	R3-247806

# Orlando, U.S.A., 18th-22nd November 2024

Agenda Item:	17.3

Source:	Ericsson (Moderator)

Title: 	Summary of offline discussions: Rel-19 Network Energy Saving OD-SIB1

Document for:	Discussion and Approval

# Introduction

The CB is related to AI 17.3.

CB: # R19ES

- Discuss the open issues above

(moderator - E///)

- Summary of offline disc R3-245737

# For the Chairman’s Notes

It is proposed to capture the following RAN3 agreements in the chairman’s notes:

Proposal 1:

The purpose of the new XnAP  procedure  is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2

The procedure uses non UE-associated signaling.

Cell A stores the UL WUS configuration information after it has received it.

Proposal 2:

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The procedure uses non UE-associated signalling.

Proposal 3:

Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures,

Proposal 4:

The CRs are agreed as Baseline CR:

XnAP: R3-247857

F1AP: R3-247858

# 3 Discussion

We have already made the following agreements:

- The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

- Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

We first make the agreement for the XnAP and F1AP TPs implementing the above agreements (with or without FFS).

We further discuss what more are needed.

# 4 Discussion on XnAP TP

We have already made the following agreements:

- The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

- Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The below captures the XnAP agreements.

### 8.x.x	UL WUS Configuration Provision (FFS)

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2 and request NG-RAN node2 to transmit UL WUS configuration information (FFS)

The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to discontinue transmission of UL WUS configuration.  (FFS)

The procedure uses non UE-associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

Cell A stores the UL WUS configuration information after it has received it.

Cell A stores the UL WUS configuration information after it has been requested to be discontinued (FFS)

Cell A removes the UL WUS configuration information after it has been requested to be stopped (FFS)

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

#### x.x.x.4	Abnormal Conditions

Void.

### 8.x.y	UL WUS Configuration transmission status Updates (FFS)

#### 8.x.y.1	General

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Provision Change, successful operation

The message name is FFS

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

# 5 Discussion on F1AP TP

Reuse the existing message.

Include UL WUS configuration from gNB-DU to gNB -CU;

# 6 Moderator Summary

The Xn TP in Chapter 4 is captured in the Baseline XnAP CR.

The F1AP in Chapter 5 is included in the Baseline F1AP CR.

The Green text of Chapter 4/5 are captured in the Chairman notes.