3GPP TSG RAN WG3 Meeting #127-bis	R3-251508

Wuhan, China, 07-11 April, 2025

3GPP TSG RAN WG1 # 120		R1-2501523

Athens, Greece, February 17th – 21st, 2025

Title:	Reply LS on LMF-based AI/ML Positioning for Case 2b

Response to:	LS on LMF-based AI/ML Positioning for Case 2b (S2-2501133)

Release:	Rel-19

Work Item:	         AIML_CN

Source:	RAN1

To:	SA2

Cc:	RAN2, RAN3

Contact person:	Huaming Wu

E-mail Address:	huaming.wu @ vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN1 thanks SA2 for the LS on LMF-based AI/ML Positioning for Case 2b.

RAN1 would like to provide the following agreements and working assumptions reached up to RAN1#119 related to Case 2b as response to SA2 on data types and procedures, with reference to RAN1 chairman notes for each.

Agreement [R1-2401937] For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model. Agreement [R1-2401937] For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF: - timing information; - paired timing information and power information. Agreement [R1-2401937] In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement [R1-2401937] For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc. Agreement [R1-2403821] For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption [R1-2403821] For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. Working Assumption [R1-2403821] For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Agreement [R1-2405782] For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed. Agreement [R1-2407587] For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement [R1-2409341] For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information

Note1: the working assumptions and agreements above are based on RAN1 understanding for RAN work item (NR_AIML_air).

Note2: "AI/ML based positioning" by itself refers to all positioning cases, including Case 2b.

Note3: Case 2b was assigned 2nd priority in RAN work item (NR_AIML_air) [RP-240774] and was not discussed during Q4 of 2024.

RAN1 may provide additional input in the future to SA2 for Case 2b if any new agreement reached in RAN1.

# 2	Actions

To SA2:

RAN1 kindly asks SA2 to take the above answer into consideration in future work.

# 3	Dates of next TSG RAN 1 meetings

TSG RAN WG1 Meeting #120bis           7- 11 April,  2025               China

TSG RAN WG1 Meeting #121              19- 23 May,  2025                Malta

# 4	Reference

- R1-2401937, Report of RAN1#116 meeting, Athens, Greece, Feb. 2024.

- R1-2403821, Report of RAN1#116bis meeting, Changsha, China, April 2024.

- R1-2405782, Report of RAN1#117 meeting, Fukuoka, Japan, May 2024.

- R1-2407587, Report of RAN1#118 meeting, Maastricht, Netherlands, Aug. 2024.

- R1-2409341, Report of RAN1#118bis meeting, Hefei, China, Oct. 2024.

- R1-2501525, Reply LS on LMF-based AI/ML Positioning for Case 3b, RAN1, Athens, Feb. 2025