3GPP TSG-RAN WG3#127bis 	R3-251533

Wuhan, China, 7-11 April 2025

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	TP (BL CR TS 38.300) Inter-CU LTM Procedure

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-250339 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

• Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] o Prioritize the case when CU is acting as MN when DC is not configured o When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:  As secondary priority, support the case where CU is acting as SN and MN is unchanged  As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released o Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM  Coordination with SA3 needed with respect to security key handling o Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support • Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] o Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM o Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] o Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] o Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. • Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] o Specify UE evaluated conditions for triggering LTM o Aim to support conditional LTM including subsequent LTM o Limit specifying the conditional LTM to the scenario where the UE is in non-DC • Specify RRM requirements related to the above objectives as necessary [RAN4]

This paper discusses the basic required mobility procedures for inter-CU L1/L2 triggered mobility (LTM) and its potential impact to the disaggregated gNB architecture signalling.

# 2	Inter-CU LTM

In Rel-18, LTM was specified with limitation that LTM mobility could be configured and carried out only to cells within the same gNB-CU (i.e. intra-gNB LTM). In Rel-19, this feature expands to also cover inter-gNB-CU (i.e. inter-gNB LTM) mobility.

For Inter-CU LTM, we foresee the following as necessary to be considered.

- Rel-18 LTM framework and LTM handover phases for intra-gNB-CU LTM are basis also for inter-gNB-CU LTM. That is, inter-CU LTM also has the following phases:

(a) Preparation phase, (b) Early synchronization phase, (c) Execution phase, and (d) Completion phase.

The following are agreements from the previous RAN3 meeting, which are basis for proposals in this paper.

to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. The LTM Configuration IDs are allocated by the source CU. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. Confirm the name of the new procedure as “LTM Configuration Update”. Introduce a new procedure for candidate gNB-initiated LTM cancellation. Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution. The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message. Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch. Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation. Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB. RAN3#127: For both inter-CU and intra-CU cases: For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell. For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one. The candidate gNB-CU responds the full SSB Time/Frequency Configuration (in SSB Information IE) of candidate cells to the source gNB-CU in the Handover Request ACK message. The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration. WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. RAN3 agree the following scenarios to support LTM with NR-DC: 1.SN initiated inter-CU SCG LTM without MCG changes (high priority) 2.Inter-CU MCG LTM without SN release 3.Inter-CU MCG LTM with SN release 4.Inter-CU MCG LTM with SN addition The format of the new introduced IE is same with the gNB-DU ID. The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message. The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message. FFS on whether the source gNB can request a candidate gNB to provide a reference configuration. WA: Use a single UE association for multiple LTM handover request to the same candidate gNB. Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch. Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation. For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN. The MN requests each candidate SN to allocate resources for the UE via SN Addition request message. Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message. The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed. Continue to work on the solutions comparison and details…

## 2.1	Preparation Phase

With respect to the preparation phase, the following aspects require further consideration.

Semi-persistent CSI-RS Support for LTM

At prior meetings, RAN3 had indicated to RAN1 that support for Semi-Persistent CSI-RS was feasible. RAN1 further confirmed the need to support this function in R3-251507. However, RAN3 left open whether to reuse existing messages (in F1AP and XnAP) or introduce new messages. This aspect is discussed separately in R3-25XXXX.

Reference Configuration

RAN3 had earlier concluded to follow approach of single reference configuration. However, at last meeting it was debated whether the single reference configuration must be from a cell from the Source-CU, or whether it could be retrieved from a cell a different Candidate-CU.

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message. The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message. FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

In our view, the single reference configuration to be used to generate the common configuration for all the LTM candidate cells should be maximized to address as much commonalities as possible. Therefore, the Source-CU, who is aware of all candidate LTM cells and their corresponding gNBs, is in best position to determine whether to generate the common configuration based on a cell of its own, or to request a reference configuration from one of the Candidate-CUs. Consider the following scenarios:

- Scenario 1 – Candidate-CU has majority of LTM cells being prepared: In this case, most commonalities would be achieved by using a reference from the Candidate-CU since it will host most LTM cells. E.g., such as if Source-CU is being configured two LTM cells, and in contrast the Candidate gNB-CU is being configured six LTM cells.

- Scenario 1bis – Source-CU has no LTM candidate cells: Similar to the previous case, most commonalities would be achieved from a reference from a Candidate-CU rather than a cell of the Source-CU, which in this scenario, is not hosting any LTM cells.

Proposal 1: Source-CU/Source-gNB can request a Candidate-CU/Candidate-gNB to provide a reference configuration.

Proposal 2: Remove FFS in Stage 3 with respect of request for reference configuration from candidate.

Data Forwarding and TNL Information Update:

To support data forwarding in case of inter-CU LTM, the Source gNB needs to transfer information regarding PDU Session Resource Information List of all other Candidate gNBs to the Target gNB to execute data forwarding for subsequent LTM. Similarly, RAN3 has agreed this information to be included within LTM CONFIGURATION UPDATE message with the exact timing left to implementation. However, some Stage-3 details were left open.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch. Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

In latest BL CR for XnAP 38.423, the existing Data Forwarding Info IE defined in section 9.2.1.16 was introduced to the LTM CONFIGURATION UPDATE message with remark of “FFS if list is needed”. In our view, the IE introduced contains the relevant details for data tunnels for a given target. However, in Inter-CU LTM, data forwarding may be carried out toward multiple targets. Therefore, the IE structure needs to be modified to contain a list of Data Forwarding Info IEs, each corresponding to a different target to where data forwarding can be carried out. With this information, the new Source-gNB can initiate early or normal data forwarding accordingly.

Proposal 3: The Source-gNB indicates via a List, the PDU Session Resource Information of Candidate gNBs.

Given the RAN3 agreement that timing is left to implementation, we believe it is useful to also optionally include the data forwarding related information in the LTM CELL SWITCH NOTIFICATION message. In this way the Source-gNB can utilize this existing signaling step to inform the data forwarding details to the new Source-gNB as well without having to wait for an additional procedure (i.e. a LTM CONFIGURATION UPDATE message afterwards).

Proposal 4: The Source gNB may also provide PDU Session Resource Information List of all Candidate gNBs via the LTM CELL SWITCH NOTIFICATION message.

Subsequent LTM Update Information Mandatory vs. Optional:

In prior RAN3 discussions, it was left open whether Step 6-7 in for LTM CONFIGURATION UPDATE and LTM CONFIGURATION UPDATE ACKNOWLEDGE were mandatory or optional with a corresponding note in the BL CR. In line with Rel-18 and with flexibility for updating candidates at a flexible timing, we think that Steps 6-7 should be indicated as optional and the Note removed.

Further, these additional steps incur unnecessary delay in several scenarios, as the Source gNB-CU could be able to generate the RRC Reconfiguration message toward the UE without waiting for an additional updated response from all the Candidate gNB-CUs. Thus, carrying out any further updates toward the Candidate gNB-CUs at a later time (e.g., to update TNL related information). For example:

Single configuration: if the Source gNB-CU determines that a common LTM CSI-RS resource configuration is used, it could signal this decision to the Candidate gNB-CU(s) in the HANDOVER REQUEST message. Hence, removing the need to consolidate individual reports from each Candidate gNB-CUs prior to generating the RRC Reconfiguration message to the UE.

L3 inter-CU LTM: L3 based LTM is already agreed to be applicable to Inter-CU scenarios. Thus, another related case is when any given Candidate gNB-CU only supports L3 based LTM configuration. Such nodes need not generate a lower layer LTM configuration to be consolidated at the Source gNB-CU and could indicate it accordingly to the Source gNB-CU. The Source gNB-CU would in place proceed to generate the RRC Reconfiguration message to the UE without a second update procedure for these Candidates.

Subsequent LTM not required: Moreover, if subsequent LTM is determined not to be required after Inter-CU LTM operation either by the source or Candidate gNB-CU, these nodes could coordinate whether to restrict Subsequent Inter-CU LTM mobility via Xn signaling. In this case, Steps 6-7 would not be required entirely.

Proposal 5: Remove Note in TS 38.300 BL CR “Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.” And leave as optional procedure.

Proposal 6: RAN3 to study impact and required handling in Inter-CU LTM preparation phase when only L3 based Inter-CU LTM is supported at the Candidate gNB-CU(s).

Proposal 7: RAN3 to study means to simplify Inter-CU LTM preparation by support of using a common CSI-RS configuration, or by limiting execution of subsequent LTM.

## 2.2	Early Synchronization Phase

With respect to the early synchronization phase, the following aspects require further consideration.

TA Information Transfer

RAN3 agreed at prior meetings to “introduce a new procedure on Xn to transfer the TA information”. Further, took a working assumption to be a Class-2 non-UE associated procedure at last RAN3 meeting.

WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

The format of this message simply needs to match to existing F1AP messages introduced in Rel-18 for the same purpose. Hence, the working assumption can be changed to an agreement without further content changes.

Proposal 8: Change from Working Assumption to Agreement the following statement “Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.” With same contents as F1AP existing TA INFORMATION TRANSFER messages.

## 2.3	LTM Execution Phase

With respect to the LTM execution phase, the following aspects require further consideration.

Timing Advance Command (TAC):

Upon cell switch decision, the gNB-DU sends a MAC CE command to the UE which includes the TCI state indicating the UE which beams it should use to access the target cell (applicable for both RACH-less and RACH-based mobility), as well as a Timing Advance Command (TAC). Likewise, the value in the TAC indicates to the UE whether it should perform a RACH-less switch.

However, over F1AP and XnAP interfaces, the Cell Switch Notification messages although it includes the TCI State ID, lacks support for indicating whether RACH-less mobility was triggered to the UE. Therefore, the Candidate gNB-DU/Candidate gNB is unaware whether the upcoming UE access is RACH-less or RACH-based. Consequently, it can also not determine whether it should issue a Dedicated Grant (DG) for DG-based RACH-less cell switch, or whether it should activate a Configured Grant (CG) previously provided in the RRC Configuration for CG-based RACH-less switch. Thus, due to this uncertainty, the target cell is forced to issue a DG, even when the triggered switch by the source was RACH-based, leading to PDCCH and PUSCH resources being wasted. Similarly, to support CG-based RACH-less cell switch, the target cell is also forced to activate and monitor the CG, even for the RACH-based cell switch, leading to PUSCH resource waste. For these reasons, an IE from Source gNB-DU/gNB is required over F1 and Xn interfaces to indicate whether the cell switch triggered was RACH-less based correspondingly.

Proposal 9: Add a “Cell Switch Type (ENUMERATED(RACH-less, RACH-based))” in F1AP and XnAP CELL SWITCH NOTIFICATION messages.

TA Forwarding:

In the Rel.18 DU-CU and CU-DU CELL SWITCH NOTIFICATION messages, the TA information list IE may be included to transfer obtained TA values to the target (i.e., new serving) cell. Similarly, the TA validity was left up to implementation in case of intra-CU LTM.

If TA forwarding is supported for Rel-19 Inter-CU LTM, we believe the TA validity cannot be left up to implementation as the Xn link delay needs to be accounted for. A possible approach which can be linked to UE signal experienced, is to provide information such as the RSRP which the UE experienced when TA was acquired. This would allow the Target-gNB/DU to better understand the signal conditions associated with a TA value being received, rather than a “time” value tied to the TA acquisition itself. Based on the received RSRP value along with TA value(s) and the new RSRP value, the new source DU can determine the validity of the TA value. Note that such scheme is already possible for SDT and should be considered for LTM.

Proposal 10: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP DU-CU CELL SWITCH NOTIFICATION and XnAP CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

Figure 1: Execution Phase Considerations for Subsequent Inter-gNB-CU LTM

## 2.4	LTM Completion Phase

With respect to the LTM completion phase, the following aspects require further consideration.

Handling of Xn UE AP Identifiers:

At prior RAN3 meetings, it was agreed the following.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch. … Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

Hence, there is no need to keep the “Editor’s Note: The new source gNB may initiate to setup the UE associated Xn signalling connection with the candidate gNBs. Details are FFS.”

Proposal 11: Remove editor note on BLCR 38.300 “Editor’s Note: The new source gNB may initiate to setup the UE associated Xn singnalling connection with the candidate gNBs. Details are FFS.”

# 3	Inter-CU LTM with Dual Connectivity

At the last RAN3 meeting, there was progress with respect to Inter-CU SCG LTM. However, Inter-CU MCG LTM has still to be discussed further in RAN3.

## 3.1 Inter-CU MCG LTM Issues

The WI scope indicates both Inter-CU MCG LTM with SN unchanged, and Inter-CU MCG LTM with SN release as in scope of the WID. In the following sections we discuss open issues with respect to these two scenarios.

- When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released.

Issue 1 - Candidate MN prepares a different candidate PSCell at source SN

In Inter-CU MCG LTM with unchanged SN, it is possible that a candidate MN prepares a different PSCell even if it retains the source SN. Thus, both scenarios should be considered and handling would vary as follows.

Scenario 1: Candidate MN retains the source PSCell: Even though the PSCell remains unchanged after Inter-CU MCG LTM execution, the UE must perform reconfiguration with sync as SN security key changes due to MN change.

Scenario 2: Candidate MN prepares a different PSCell: Intra-CU PSCell HO is triggered, as a result of Inter-CU MCG LTM PCell change.

In scenario 2 above, given that the Intra-CU PSCell change (or Intra-CU SCG LTM) may be resulting from Inter-CU MCG LTM execution, the network must ensure it considers the latest PSCell measurements while triggering PSCell HO.

Proposal 12: For Inter-CU MCG LTM where a Candidate MN prepares a different PSCell resulting in an Intra-CU PSCell Handover, the PSCell change is based on latest UE reported radio measurements.

Issue 2 – SCG release and add

Given that it is the decision of candidate MN during HO preparation to retain or release the SCG, some of the Inter-CU MCG LTM candidate nodes during HO preparation may prepare a MCG + SCG configuration (when SCG is retained at source SN) or a MCG only configuration (when SCG is not retained at source SN). Hence, it may result into a combination of inter-CU MCG LTM preparation to MN candidates some with SCG retained and some with SCG released.

In the context of the above stated scenario, RAN2 has confirmed the support of SCG release and subsequently SCG add for Inter-CU MCG LTM, as well as include the SCG configuration in the mrdc-SecondaryCellGroupConfig. Hence, if the Inter-CU MCG LTM results in SCG release at the target, the SCG may be added during a subsequent PCell change if the candidate configuration included the SCG configuration as part of mrdc-SecondaryCellGroupConfig.

Proposal 13: If Inter-CU MCG LTM results in SCG release, then SCG may be added during subsequent LTM PCell change if the Inter-CU MCG LTM candidate configuration includes SCG configuration.

Issue 3 - Avoiding blind SCG or PSCell addition

An SCG Addition or a PSCell Addition during subsequent Inter-CU MCG LTM execution (after SCG release during earlier Inter-CU MCG execution) may result into a potential blind PSCell addition if the network does not consider latest PSCell measurements to identify the suitable PSCell at the time of inter-CU MCG LTM execution. Further, if there is more than one candidate target PSCells prepared associated with the Inter-CU candidate target PCell, considering the latest UE reported radio measurements for candidate PSCells is essential to identify the suitable candidate PSCell and the associated LTM Candidate Configuration ID to avoid user experience and throughput degradation after SCG addition.

Proposal 14: In the case of Inter-CU MCG LTM involving SCG Addition (after SCG release), the PSCell addition is based on latest UE reported radio measurements for candidate PSCells.

## 3.2	Other Issues

Issue 4 - Configuration Mismatch during LTM Cell Switch

During initial preparation of candidate target node, the UE’s source cell configuration is used to prepare the target cell configuration for the prepared candidate target nodes. Given that the configuration at the UE after the initial preparation may vary, by the time of LTM cell switch or HO execution, the candidate target cell configuration prepared during the initial LTM preparation may not be aligned with the latest configuration at the UE. The handling of such a problem may require reconfiguring the UE at the time of cell switch or HO execution. However, to support subsequent LTM, the UE reconfiguration after cell change must be avoided. We propose RAN3 to discuss this issue and consider signalling enhancements for handling mismatch in the PDU session and DRB configuration between the serving and the target cells.

Proposal 15: RAN3 to derive a solution to address configuration mismatch between the serving and the target cells after initial preparation, and at the time of HO execution to support subsequent LTM cell change.

# 4	Conclusions

Preparation Phase

Proposal 1: Source-CU/Source-gNB can request a Candidate-CU/Candidate-gNB to provide a reference configuration.

Proposal 2: Remove FFS in Stage 3 with respect of request for reference configuration from candidate.

Proposal 3: The Source-gNB indicates via a List, the PDU Session Resource Information of Candidate gNBs.

Proposal 4: The Source gNB may also provide PDU Session Resource Information List of all Candidate gNBs via the LTM CELL SWITCH NOTIFICATION message.

Proposal 5: Remove Note in TS 38.300 BL CR “Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.” And leave as optional procedure.

Proposal 6: RAN3 to study impact and required handling in Inter-CU LTM preparation phase when only L3 based Inter-CU LTM is supported at the Candidate gNB-CU(s).

Proposal 7: RAN3 to study means to simplify Inter-CU LTM preparation by support of using a common CSI-RS configuration, or by limiting execution of subsequent LTM.

Early Synchronization Phase

Proposal 8: Change from Working Assumption to Agreement the following statement “Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.” With same contents as F1AP existing TA INFORMATION TRANSFER messages.

LTM Execution Phase

Proposal 9: Add a “Cell Switch Type (ENUMERATED(RACH-less, RACH-based))” IE in F1AP and XnAP CELL SWITCH NOTIFICATION messages.

Proposal 10: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP DU-CU CELL SWITCH NOTIFICATION and XnAP CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

LTM Completion Phase

Proposal 11: Remove editor note on BLCR 38.300 “Editor’s Note: The new source gNB may initiate to setup the UE associated Xn singnalling connection with the candidate gNBs. Details are FFS.”

Inter-CU LTM with Dual Connectivity Issues

Proposal 12: For Inter-CU MCG LTM where a Candidate MN prepares a different PSCell resulting in an Intra-CU PSCell Handover, the PSCell change is based on latest UE reported radio measurements.

Proposal 13: If Inter-CU MCG LTM results in SCG release, then SCG may be added during subsequent LTM PCell change if the Inter-CU MCG LTM candidate configuration includes SCG configuration.

Proposal 14: In the case of Inter-CU MCG LTM involving SCG Addition (after SCG release), the PSCell addition is based on latest UE reported radio measurements for candidate PSCells.

Proposal 15: RAN3 to derive a solution to address configuration mismatch between the serving and the target cells after initial preparation, and at the time of HO execution to support subsequent LTM cell change.

# References

- 3GPP TS 38.401

- RP-250339 Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

# Annex: Text Proposal for TS 38.300

<< First Change >>

##### 9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the CSI-RS configuration.

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request.

6.	The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration.

NOTE :	Step 6 may also be triggered after step 14, or after step 17 by implementation for subsequent LTM.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early synchronization to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB if early TA acquisition is performed to the candidate gNB(s).

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Data forwarding may be initiated as soon as the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 14.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs.

21. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

<< End of Changes >>