3GPP TSG RAN WG3 Meeting #127-bis	R3-251580

Wuhan, China, 07-11 April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0156 | rev | 6 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.425) Support of XR enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T, BT, T-Mobile USA, Charter Communications, Qualcomm, Huawei, Nokia, Nokia Shanghai Bell，China Telecom, CMCC, ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support for XR and L4S in NR-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates for the support of XR and L4S in NR-DC Support for DL PSI based discarding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | XR and L4S in NR-DC is not supported DL PSI based discarding not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.1, 5.5.2.3, 5.5.3.x (new), 5.5.3.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1269 TS 37.340 Draft CR TS 38.473 CR 1485 TS 38.300 Draft CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: update text Rev 2: update cover page Rev 3: implementation of R3-244768 for support of DL PSI based discarding Rev 4: resubmission to RAN3#125bis Rev 5: update the title of the CR. Rev 6: resubmission to RAN3#127bis |  |  |  |  |  |  |  |  |

-------------------------------------- CHANGES START HERE --------------------------------------

### 5.4.3	Transfer of Assistance Information

#### 5.4.3.1	Successful operation

NOTE 1:	In this section, PDCP duplication and delay measurement related information are not applicable to E-UTRA PDCP.

The purpose of the Transfer of Assistance Information procedure is to provide assistance information to the node hosting the NR PDCP entity. Such information may be taken into consideration by the node hosting the NR PDCP entity for UP management and optimisation procedures.

An NR user plane protocol instance making use of the Transfer of Assistance Information procedure is associated to a single data radio bearer only.

The Transfer of Assistance Information procedure may be invoked if

-	the corresponding node decides to send the Radio Quality Assistance Information and/or the PDCP duplication activation suggestion to the node hosting the NR PDCP entity for the concerned data radio bearer or,

-	the corresponding node decides to send the Radio Quality Assistance Information to the node hosting the NR PDCP entity for the concerned RLC entity.

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the QoS monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.

The ASSISTANCE INFORMATION DATA frame may include one or more Radio Quality Assistance Information. The information shall consist of the information indicated in the Assistance Information Type.

The ASSISTANCE INFORMATION DATA shall be sent, if supported, when the corresponding node receives a DL USER DATA PDU including the Assistance Information Report Polling Flag set to 1.

The ASSISTANCE INFORMATION DATA frame may include the PDCP Duplication Activation Suggestion, which informs the node hosting the NR PDCP entity of the suggestion from the corresponding node on whether to activate or not activate DL PDCP duplication. The node hosting the NR PDCP entity may take this information into account to take a decision on whether to activate or not activate PDCP duplication.

The ASSISTANCE INFORMATION DATA frame may include the UL Delay or/and DL Delay measured by the corresponding node. The node hosting the NR PDCP entity may take this information into account to calculate the whole UL or/and DL delay of RAN.

The ASSISTANCE INFORMATION DATA frame may include UL Congestion Information and/or DL Congestion information measured by the corresponding node. The node hosting the NR PDCP entity shall, if supported, take this information into account to perform ECN marking in the NG-RAN node, or it shall take this information into account when sending the UL Congestion Information and/or DL Congestion information  to the UPF for ECN marking or information exposure as specified in TS 23.501 [8].

The ASSISTANCE INFORMATION DATA frame may include the DL PSI Based Discard Activation Suggestion, which informs the node hosting the NR PDCP entity of the suggestion from the corresponding node on whether to activate or deactivate DL PSI based PDCP discard. The node hosting the NR PDCP entity may take this information into account to decide whether to activate or deactivate DL PSI based PDCP discard.

Figure 5.4.3.1-1: Successful Transfer of Assistance Information

NEXT CHANGE

#### 5.5.2.3	ASSISTANCE INFORMATION DATA (PDU Type 2)

This frame format is defined to allow the node hosting the PDCP entity to receive assistance information.

The following shows the respective ASSISTANCE INFORMATION DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=2) |  |  |  | PDCP Dupl. Ind. | Assistance Info. Ind. | UL Delay Ind. | DL Delay Ind. | 1 |
| Spare |  |  | DL PSI Based Discard Ind. | DL PSI Based Discard Activation Suggestion | UL Congestion Information Ind. | DL Congestion Information Ind. | PDCP Duplication Activation Suggestion | 1 |
| Number of Assistance Information Fields |  |  |  |  |  |  |  | 0 or 1 |
| Assistance Information Type |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields) |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |
| UL Delay DU Result |  |  |  |  |  |  |  | 0 or 4 |
| DL Delay DU Result |  |  |  |  |  |  |  | 0 or 4 |
| UL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |

Figure 5.5.2.3-1: ASSISTANCE INFORMATION DATA (PDU Type 2) Format

NEXT CHANGE

#### 5.5.3.x	DL PSI Based Discard Indication

Description: This field indicates the presence of the DL PSI Based Discard Activation Suggestion. This information element is not applicable to E-UTRA PDCP.

Value range: {0= DL PSI Based Discard Activation Suggestion not present, 1= DL PSI Based Discard Activation Suggestion present}.

Field length: 1 bit.

#### 5.5.3.y	DL PSI Based Discard Activation Suggestion

Description: This parameter indicates the suggestion given by the corresponding node on whether DL PSI based PDCP discard should be activated or not. This information element is not applicable to E-UTRA PDCP.

Value range: {0= Not activate, 1= Activate}.

Field length: 1 bit.

CHANGES END