3GPP TSG-RAN3 Meeting #127bis	R3-251589

Wuhan, China, Apr 7th – 11th, 2025

Agenda item:	10.3.1

Source:	Qualcomm Incorporated

Title:	SON MDT for NTN

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss potential SON/MDT enhancements for NTN based on agreements and open issues in the previous meetings.

# Discussion

  - MRO enhancements for NTN

### Time-based CHO

RAN3 discussed and agreed the following failure scenario of Too Late CHO Execution (case 1) when UE is configured with a time-based CHO trigger condition.

Further, it was clarified last meeting that UE can’t attempt CHO execution after T1-Threshold in case of Time-based CHO and therefore there is no need to distinguish whether RLF happens before or after T1-Threshold.

Case 1 (Too Late CHO Execution): RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

In case of Time-based CHO, to distinguish whether RLF occurred before or after T1-Threshold, down-select among below three alternatives:

Alt 1: RLF report includes the Absolute time (measured UTC time) when RLF occurred

Alt 2: RLF Report includes a flag to indicate whether the RLF happens before or after T1-Threshold

Alt 3: based on existing timers in RLF report

Existing timers can be reused to identify whether RLF happens before or after T1 with 100ms accuracy, FFS if the accuracy is a problem to be solved.

No need to identify whether RLF happens before or after T1 for time-based CHO.

So in case of this Too late CHO (RLF before T1-Threshold), if the network is able to determine that a lot of RLFs (e.g., 90%) happen before T1-Threshold, it was suggested that the network can move the T1-Threshold to the left (i.e., decrease its value) so that the UEs can start time-based CHO execution earlier and thereby reducing the possibility of these RLFs.

There were few proposals in the previous meetings to report the absolute timestamp (e.g., timestamp when RLF occurred or timestamp when CHO occurred). But we think there is no need for UE to report the absolute timestamps as existing relative timer (timeSinceCHO-Reconfig) in RLF Report and time points information at the gNB is sufficient.

It is to be noted the gNB knows the time CHO config is sent to the UE and the CHO configuration parameters (T1-Threshold).

Therefore, the network can determine the time RLF occurred by adding the time it sent CHO configuration to the UE with timeSinceCHO-Reconfig received in RLF Report

gNB knows the time CHO configuration is sent to the UE and the CHO configuration parameter T1-Threshold.

timeSinceCHO-Reconfig is reported in RLF Report with a granularity of 100 ms. In the worst case, the RLFs that occur before 100 ms of T1-Threshold can’t be identified via using timeSinceCHO-Reconfig and such a small error can be ignored.

gNB can determine the time RLF occurred via adding the

- time it sent CHO configuration to the UE and timeSinceCHO-Reconfig. Therefore, there is no need for UE to report absolute timestamp when RLF happened in case of time-based CHO

### Location-based CHO

The following location-based CHO trigger conditions are supported for NTN:

CondEvent D1:

Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and

between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2

CondEvent D2:

Distance between UE and a moving reference location determined based on movingReferenceLocation and its corresponding satellite ephemeris and epoch time broadcast in SIB19 for the serving cell becomes larger than configured threshold distanceThreshFromReference1 and

distance between UE and a moving reference location determined based on referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2

In the previous meetings, we discussed a scenario that Location-based CHO parameters in CondEvent D1/D2 (e.g., distanceThreshFromReference1, distanceThreshFromReference2) might not have been configured properly, therefore resulting in a Too early/Too late/wrong cell CHO.

Further, RAN3 agreed the following:

For earth moving case, the accuracy and complexity issue of the network-based solution is acknowledged.

Option 1: to introduce the absolute UTC time of when RLF/HOF happens in the RLF report.

Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell (when RLF occurred).

FFS on option 1 or option 2.

Even for legacy measurement-based CHO conditions (e.g., A3/A5), we never report how “close” the UE was to meeting the CHO threshold, rather just report which condition was met in case multiple thresholds are configured.

For legacy measurement-based CHO conditions (e.g., A3/A5), UE never reports how “close” the UE was to meeting the CHO threshold before RLF, rather just report which condition was met in case multiple thresholds are configured.

For earth moving case, UE just reports which condition (distance between UE and serving cell moving reference location or distance between UE and moving reference location of neighbor cell or neither) was met in CondEventD2. There is no need to report any measured distances.

  - Logged MDT enhancements for NTN

### Location-based event trigger for logged MDT

NTN supports location-based measurements in RRC_IDLE and RRC_INACTIVE. Based on this, the UE will measure in RRC_IDLE and RRC_INACTIVE if it has moved a certain distance larger than a threshold and if the radio signal strength is above a certain threshold.

Current event triggers in logged MDT are only for Out-of-coverage or when signal strength is below a threshold. We propose to introduce a location-based event trigger for logged MDT.

Introduce a new “location-based” event trigger to collect logged MDT only when the UE has moved a certain distance w.r.t. a reference location and if the signal strength is above a certain threshold.

# Conclusion

2.1.1	Time-based CHO

Observation 1.	gNB knows the time CHO configuration is sent to the UE and the CHO configuration parameter T1-Threshold.

Observation 2.	timeSinceCHO-Reconfig is reported in RLF Report with a granularity of 100 ms. In the worst case, the RLFs that occur before 100 ms of T1-Threshold can’t be identified via using timeSinceCHO-Reconfig and such a small error can be ignored.

Proposal 1:	gNB can determine the time RLF occurred via adding the

time it sent CHO configuration to the UE and timeSinceCHO-Reconfig. Therefore, there is no need for UE to report absolute timestamp when RLF happened in case of time-based CHO

2.1.2	Location-based CHO

Observation 3.	For legacy measurement-based CHO conditions (e.g., A3/A5), UE never reports how “close” the UE was to meeting the CHO threshold before RLF, rather just report which condition was met in case multiple thresholds are configured.

Proposal 2:	For earth moving case, UE just reports which condition (distance between UE and serving cell moving reference location or distance between UE and moving reference location of neighbor cell or neither) was met in CondEventD2. There is no need to report any measured distances.

2.2.1	Location-based event trigger for logged MDT

Proposal 3:	Introduce a new “location-based” event trigger to collect logged MDT only when the UE has moved a certain distance w.r.t. a reference location and if the signal strength is above a certain threshold.