3GPP TSG-RAN WG3 Meeting #127-bis	R3-251598

Wuhan, China, 7-11 April, 2025

Title:	Consideration on (S-)CPAC cancel and SN Release

Agenda Item:	9.2

Source:	Huawei, Lenovo, LG Electronics, Google, China Telecom

Document for:	Discussion

# Introduction

In this contribution, we discuss the issues found about interactions for (S-)CPAC and SN Release procedures.

# 2	Discussion

## 2.1	Rel-17 CPAC

In RAN3#115 chairman notes, the following agreements can be found in the agreement list of CPAC:

In CPA and MN/SN initiated inter-SN CPC, MN can:

Update/modify previous CPAC configurations provided in CPAC addition using MN initiated SN modification procedure

Cancel all prepared PSCells at target SN and release the target SN using MN initiated SN release procedure

In CPA and MN/SN initiated inter-SN CPC, target SN can:

Update/modify previous CPAC configurations provided in CPAC addition using SN initiated SN modification procedure

Add prepared PSCells within the limit given by the MN or source SN using SN initiated SN modification procedure

Cancel some of the prepared PSCells using SN initiated SN modification procedure.

Cancel all prepared PSCells using SN initiated SN release procedure

In SN initiated inter-SN CPC, source SN can:

Update/modify previous CPC configurations provided in CPC preparation using SN change required procedure

Cancel all prepared PSCells at target SN and release the target SN using SN change required procedure

New IEs are introduced in SN change required message indicating CPC configuration modification and target SN release.

In MN/SN initiated inter-SN CPC, MN can inform source SN about the triggered target SN release or some prepared PSCells cancellation at a target SN using:

A new class2 XnAP procedure

A new class2 X2AP procedure

Observation 1: The cancellation of the prepared CPA or inter-SN CPC, can either be triggerred by the MN via MN initiated SN Release procedure, or triggerred by the target SN via SN initiated SN Release procedure.

Observation 2: In case source SN would like to cancel all prepared PSCells at target SN, the SN change required procedure should be used instead of SN initiated SN Release procedure.

In R3-226795/ R2-2213371, the interaction between CPC Cancel and SN Release were added to “clarify that CPA/CPC with target SN(s) should be cancelled when the MN receives SN Release Request Ack message or SN Release Required message”. And the following changes, with the single message related to multiple target candidate SNs, it should be understood that these NOTEs are only appliable to the message from source SN:

For EN-DC the following were added:

In case of SN Release procedure – MN initiated:

NOTE 0:	If CPA or inter-SN CPC is configured, upon reception of the SgNB Release Request Acknowledge message the MN cancels all CPAC with the target candidate SN(s).

In case of SN Release procedure – SN initiated:

NOTE 2a:	If CPA or inter-SN CPC is configured, upon reception of the SgNB Release Required message the MN cancels all CPAC with the target candidate SN(s).

For MR-DC the following wereadded:

In case of SN Release procedure – MN initiated:

NOTE 00:	If CPA or inter-SN CPC is configured, upon reception of the SN Release Request Acknowledge message the MN cancels all CPAC with the target candidate SN(s).

In case of SN Release procedure – SN initiated:

NOTE 1b:	If CPA or inter-SN CPC is configured, upon reception of the SN Release Required message the MN cancels all CPAC with the target candidate SN(s).

Observation 3: The current NOTEs in 10.4.1 and 10.4.2 which related to CPAC cancellation and SN release, should aim to the scenario/interactions between MN and source SN, and only related to inter-SN CPC not related CPA.

Based on the discussion above, it is proposed to:

Proposal 1: For CPAC, in 10.4.1 and 10.4.2, it is needed to update the exisitng NOTEs which related to CPAC cancellation and SN release, clarifying that the message is received from the source SN, and removing “CPA” as it only applies to inter-SN CPC.

Proposal 2: For CPAC, in 10.4.1 and 10.4.2, it is needed to add description about that, the cancellation of the prepared CPA or inter-SN CPC, can either be triggerred by the MN via MN initiated SN Release procedure, or triggerred by the target SN via SN initiated SN Release procedure.

On the other hand, the Conditional PSCell Change Cancel procedure was intended to be used by the MN to inform the source SN about the cancellation of all the prepared PSCells in a target SN, but there is no procedure interaction in stage 3 spec, no description in stage 2 spec, it is needed to add the missing interactions about the usage of Conditional PSCell Change Cancel procedure.

Proposal 3: For CPAC, in 10.4.1 and 10.4.2, it is needed to add the interaction of using Conditional PSCell Change Cancel procedure.

## 2.2	Rel-18 S-CPAC

During S-CPAC discussion the following agreement was achieved:

Reuse the following messages to update/modify/cancel the prepared candidate PSCells for SCG Selective Activation:

- SN Modification Request/ SN Modification Request Acknowledge

- SN Modification Required/ SN Modification Confirm

- Conditional PSCell Change Cancel

- SN Change Required/ SN Change Confirm

- SN Release Request / SN Release Request Acknowledge

But in either stage 3 or stage 2 TSs, there is no clue (e.g., no description, no interaction, no S-CPAC related IE) of “whether the Conditional PSCell Change Cancel procedure is applicable to S-CPAC” and “how and when to used it in case of S-CPAC”.

Observation 4: “whether the Conditional PSCell Change Cancel procedure is applicable to S-CPAC” and “how and when to used it in case of S-CPAC” are missing in all current specifications.

Proposal 4: For S-CPAC, in 10.4.2, it is needed to add description about that, the cancellation of the prepared S-CPAC, can either be triggerred by the MN via MN initiated SN Release procedure, or triggerred by the target SN via SN initiated SN Release procedure.

Proposal 5: For S-CPAC, in 10.4.2, it is needed to add the interaction of using Conditional PSCell Change Cancel procedur.

# 3	Proposals

This paper discusses the issues found for (S-)CPAC cancellation and SN release, got the following observations and proposals:

For CPAC:

Proposal 1: For CPAC, in 10.4.1 and 10.4.2, it is needed to update the exisitng NOTEs which related to CPAC cancellation and SN release, clarifying that the message is received from the source SN, and removing “CPA” as it only applies to inter-SN CPC.

Proposal 2: For CPAC, in 10.4.1 and 10.4.2, it is needed to add description about that, the cancellation of the prepared CPA or inter-SN CPC, can either be triggerred by the MN via MN initiated SN Release procedure, or triggerred by the target SN via SN initiated SN Release procedure.

Proposal 3: For CPAC, in 10.4.1 and 10.4.2, it is needed to add the interaction of using Conditional PSCell Change Cancel procedure.

For S-CPAC:

Proposal 4: For S-CPAC, in 10.4.2, it is needed to add description about that, the cancellation of the prepared S-CPAC, can either be triggerred by the MN via MN initiated SN Release procedure, or triggerred by the target SN via SN initiated SN Release procedure.

Proposal 5: For S-CPAC, in 10.4.2, it is needed to add the interaction of using Conditional PSCell Change Cancel procedur.

The CPAC related Rel-17 and Rel-18 CRs are provided in [1] and [2].

The S-CPAC related Rel-18 CR is provided in [3].

# 4	Reference

- R3-251599	Correction of CPC Cancel and SN Release,	Huawei, Lenovo, LG Electronics, Google, ZTE, China Telecom, Rel-17 draftCR to TS 37.340, Cat F

- R3-251600	Correction of CPC Cancel and SN Release,	Huawei, Lenovo, LG Electronics, Google, Nokia, Ericsson, ZTE, China Telecom, Rel-18 draftCR to TS 37.340, Cat A

- R3-251601	Correction of Conditional PSCell Change Cancel for S-CPAC,	Huawei, Lenovo, LG Electronics, Google, Nokia, Ericsson, ZTE, China Telecom, Rel-18 draftCR to TS 37.340, Cat F