3GPP TSG-RAN WG3 Meeting #127bis	R3-251616

Wuhan, China, 7-11 April 2025

Agenda Item:	11.4

Source:	ZTE Corporation, China Telecom, China Unicom

Title:	(TP to BLCR for TS 37.483) Support of UE performance in case of split architecture

Document for:	Text Proposal

1	Introduction

This TP follows discussions in R3-251614.

2	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| gNB-CU-UP E1 Setup | GNB-CU-UP E1 SETUP REQUEST | GNB-CU-UP E1 SETUP RESPONSE | GNB-CU-UP E1 SETUP FAILURE |
| gNB-CU-CP E1 Setup | GNB-CU-CP E1 SETUP REQUEST | GNB-CU-CP E1 SETUP RESPONSE | GNB-CU-CP E1 SETUP FAILURE |
| gNB-CU-UP Configuration Update | GNB-CU-UP CONFIGURATION UPDATE | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU-UP CONFIGURATION UPDATE FAILURE |
| gNB-CU-CP Configuration Update | GNB-CU-CP CONFIGURATION UPDATE | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU-CP CONFIGURATION UPDATE FAILURE |
| E1 Release | E1 RELEASE REQUEST | E1 RELEASE RESPONSE |  |
| Bearer Context Setup | BEARER CONTEXT SETUP REQUEST | BEARER CONTEXT SETUP RESPONSE | BEARER CONTEXT SETUP FAILURE |
| Bearer Context Modification (gNB-CU-CP initiated) | BEARER CONTEXT MODIFICATION REQUEST | BEARER CONTEXT MODIFICATION RESPONSE | BEARER CONTEXT MODIFICATION FAILURE |
| Bearer Context Modification Required (gNB-CU-UP initiated) | BEARER CONTEXT MODIFICATION REQUIRED | BEARER CONTEXT MODIFICATION CONFIRM |  |
| Bearer Context Release (gNB-CU-CP initiated) | BEARER CONTEXT RELEASE COMMAND | BEARER CONTEXT RELEASE COMPLETE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| IAB UP TNL Address Update | IAB UP TNL ADDRESS UPDATE | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE | IAB UP TNL ADDRESS UPDATE FAILURE |
| BC Bearer Context Setup | BC BEARER CONTEXT SETUP REQUEST | BC BEARER CONTEXT SETUP RESPONSE | BC BEARER CONTEXT SETUP FAILURE |
| BC Bearer Context Modification (gNB-CU-CP initiated) | BC BEARER CONTEXT MODIFICATION REQUEST | BC BEARER CONTEXT MODIFICATION RESPONSE | BC BEARER CONTEXT MODIFICATION FAILURE |
| BC Bearer Context Modification Required (gNB-CU-UP initiated) | BC BEARER CONTEXT MODIFICATION REQUIRED | BC BEARER CONTEXT MODIFICATION CONFIRM |  |
| BC Bearer Context Release (gNB-CU-CP initiated) | BC BEARER CONTEXT RELEASE COMMAND | BC BEARER CONTEXT RELEASE COMPLETE |  |
| MC Bearer Context Setup | MC BEARER CONTEXT SETUP REQUEST | MC BEARER CONTEXT SETUP RESPONSE | MC BEARER CONTEXT SETUP FAILURE |
| MC Bearer Context Modification (gNB-CU-CP initiated) | MC BEARER CONTEXT MODIFICATION REQUEST | MC BEARER CONTEXT MODIFICATION RESPONSE | MC BEARER CONTEXT MODIFICATION FAILURE |
| MC Bearer Context Modification Required (gNB-CU-UP initiated) | MC BEARER CONTEXT MODIFICATION REQUIRED | MC BEARER CONTEXT MODIFICATION CONFIRM |  |
| MC Bearer Context Release (gNB-CU-CP initiated) | MC BEARER CONTEXT RELEASE COMMAND | MC BEARER CONTEXT RELEASE COMPLETE |  |
| Data Collection Reporting Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| Bearer Context Release Request (gNB-CU-UP initiated) | BEARER CONTEXT RELEASE REQUEST |
| Bearer Context Inactivity Notification | BEARER CONTEXT INACTIVITY NOTIFICATION |
| DL Data Notification | DL DATA NOTIFICATION |
| UL Data Notification | UL DATA NOTIFICATION |
| Data Usage Report | DATA USAGE REPORT |
| gNB-CU-UP Counter Check | GNB-CU-UP COUNTER CHECK REQUEST |
| gNB-CU-UP Status Indication | GNB-CU-UP STATUS INDICATION |
| MR-DC Data Usage Report | MR-DC DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Early Forwarding SN Transfer | EARLY FORWARDING SN TRANSFER |
| GNB-CU-CP Measurement Results Information | GNB-CU-CP MEASUREMENT RESULTS INFORMATION |
| IAB PSK Notification | IAB PSK NOTIFICATION |
| BC Bearer Context Release (gNB-CU-UP initiated) | BC BEARER CONTEXT RELEASE REQUEST |
| MC Bearer Context Release (gNB-CU-UP initiated) | MC BEARER CONTEXT RELEASE REQUEST |
| MC Bearer Notification | MC BEARER NOTIFICATION |
| Data Collection Reporting | DATA COLLECTION UPDATE |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.2.xx	Data Collection Reporting Initiation

8.2.xx.x1	General

This procedure is used by an gNB-CU-CP to request to gNB-CU-UP the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

8.2.xx.x2	Successful Operation

Figure 8.2.9.2-1: Data Collection Reporting Initiation, successful operation

The procedure is initiated with a DATA COLLECTION REQUEST message sent from gNB-CU-CP to gNB-CU-UP to start a measurement or stop a measurements.

If gNB-CU-UP is capable to provide all requested information, it shall initiate the measurement as requested by gNB-CU-CP, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the DATA COLLECTION REQUEST indicates the type of objects gNB-CU-UP shall perform measurements on. The gNB-CU-UP shall include in the DATA COLLECTION UPDATE message:

-	the Average Packet Delay IE, if the first bit, "Average Packet Delay" of the Report Characteristics IE included in the DATA COLLECTION REQUEST message is set to "1".

If the Reporting Periodicity IE is included in the DATA COLLECTION REQUEST message, this indicates the periodicity for the reporting of periodic measurements. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity IE.

8.2.xx.x3	Unsuccessful Operation

Figure 8.2.9.3-1: Data Collection Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, gNB-CU-UP shall send a DATA COLLECTION FAILURE message with an appropriate cause value.

8.2.xx.x4	Abnormal Conditions

If the initiating gNB-CU-CP does not receive either DATA COLLECTION RESPONSE message or DATA COLLECTION FAILURE message, the gNB-CU-CP may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-CU-UP, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message with the same Transaction ID.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the DATA COLLECTION REQUEST message then gNB-CU-UP shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-CU-UP receives a DATA COLLECTION REQUEST message which includes the Registration Request IE set to "start" and the gNB-CU-CP Measurement ID IE corresponding to an existing on-going load measurement reporting, for which a different Transaction ID is used, then gNB-CU-UP shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

8.2.yy	DATA COLLECTION Reporting

8.2.yy.y1	General

This procedure is initiated by an gNB-CU-UP to report information accepted by the gNB-CU-UP following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

8.2.yy.y2	Successful Operation

Figure 8.2.10.2-1: Data Collection Reporting, successful operation

The gNB-CU-UP shall report the results of the admitted measurements in DATA COLLECTION UPDATE message. The admitted measurements are the measurements that were successfully initiated during the preceding Data Collection Initiation procedure.

8.2.yy.y3	Unsuccessful Operation

Not applicable.

8.2.yy.y4	Abnormal Conditions

Void.

9.2.1.x	DATA COLLECTION REQUEST

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(36)) | Each position in the bitmap indicates measurement object the gNB-CU-UP is requested to report. First Bit = Average UE Throughput DL, Second Bit = Average UE Throughput UL, Third Bit = Average Packet Delay, Forth Bit = Average Packet Loss DL Other bits shall be ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 120000ms, …) | Periodicity that can be used for reporting. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value “stop” |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

9.2.1.yy	DATA COLLECTION RESPONSE

This message is sent by the gNB-CU-UP to indicate that the requested measurement, for all the measurement objects included in the measurement is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.1.mm	DATA COLLECTION FAILURE

This message is sent by the gNB-CU-UP to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | O |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.1.nn	DATA COLLECTION UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | OCTET STRING (SIZE(8)) |  | – |  |
| >>UE Performance | O |  | 9.3.1.x |  | – |  |

9.3.1.x	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.3.1.20 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.3.1.20 |  |
| Average Packet Delay | O |  | 9.3.1.y |  |
| Average Packet Loss | O |  | Packet Loss Rate 9.3.1.46 |  |

9.3.1.y	Average Packet Delay

This IE indicates the RAN part of the average packet delay in the UL and DL directions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Delay UL | M |  | INTEGER (0.. 10000) | Defined in TS 38.314 [42]. Unit: 0.1 millisecond |
| Average Packet Delay DL | M |  | INTEGER (0.. 10000) | Defined in TS 38.314 [42]. Unit: 0.1 millisecond |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>