3GPP TSG-RAN WG3 Meeting #127-bis	R3-251678

Wuhan, China, 7th – 11st April 2025

Agenda Item:		11.3

Source:		NEC

Title:	Open issues for AI/ML-based CCO

Document for:		Discussion and decision

# Introduction

BLCR to TS 38.423 [1] and to TS 38.473 [2] were agreed to exchange Future Coverage Modification List over Xn and Future Coverage Modification Notification and predicted CCO assistance information over F1 respectively.

In RAN3#127 meeting, further agreements for F1 interface were achieved as below:

Each Future Coverage Modification Notification Item included in the Future Coverage Modification Notification List IE has a Future Coverage Modification Cause IE associated to it from DU to CU.

The Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only.

However, the following issues are still FFS:

Whether to exchange time for predicted CCO issue over XnAP needs further discussion.

Whether gNB-CU provides to the gNB-DU not only the predicted CCO issue but also the corresponding future CCO state as a recommendation?

UE performance feedback for CCO? Legacy UE performance feedback is sufficient or other finer granularity, e.g., cell-/SSB-level UE performance feedback?

The maximum value of time for predicted CCO issue and time for future CCO state needs further discussion.

In this contribution, we further discuss the abovementioned open issues.

# Discussion

2.1	Time information over Xn

For CCO, the following two timing information were introduced:

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn.

In XnAP NG-RAN NODE CONFIGURATION UPDATE message, a Time for Future Coverage State IE can be included for each Future Coverage Modification Item. As explained in the semantics description, this time information indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information [1].

As for timing information for predicted CCO issue, it indicates the time when the predicted CCO issue will happen in the gNB who performed this prediction. gNB uses this timing information and the predicted CCO issue to generate the corresponding future coverage configuration and timing information for future coverage state in legacy way or using AI/ML model. If in CU/DU split, gNB-CU will send this timing information together with the predicted CCO issue to gNB-DU, which can assist gNB-DU to select corresponding future coverage configuration and time for future coverage modification.

As described above, timing information for predicted CCO issue is used to generate corresponding future coverage configuration and time for future coverage state. For neighbour nodes, this timing information for predicted CCO issue is useless. The neighbour nodes only need to know when the future coverage state(s) will be applied, which can allow the neighbour nodes to adapt to this CCO coverage change.

Proposal 1: Timing information for predicted CCO issue is NOT needed to exchange over Xn.

2.2	Recommended future CCO state

Furthermore, in the receiving side, after after gNB-CU receives future coverage configuration and related Coverage Modification Cause from a neighbour gNB (gNB-CU), there can be two options:

Option 1: gNB-CU directly forwards the received future coverage configuration to its gNB-DU, and gNB-DU can adapt to this CCO coverage change by itself.

Option 2: gNB-CU uses this future information as input to generate a recommend future CCO state, e.g. using its own AI/ML-based CCO model, and it sends to recommend future CCO state to its gNB-DU for further coverage adaption.

As we agreed in the study phase, future CCO state can be generated by AI/ML-based CCO model, and future CCO state can be derived based on the predicted CCO issue and other information.

Considering gNB-CU holds the AI/ML-based CCO model and it collects all input data AI/ML model, it is more knowledgeable on the neighbour node status and UE performance status. Based on the received future CCO state and predicted CCO issue, also together with other information, e.g. current/predicted radio resource status, gNB-CU can take advantage of AI/ML tool and coordinate all gNB-DUs connected to it to generate a more propitiate future CCO state. Therefore, we think gNB-CU can generate a recommended future CCO state and send it to gNB-DU as assistance information.

Proposal 2: Based on the received future CCO state, the receiving gNB-CU can generate a recommended future CCO state for its gNB-DU.

Proposal 3: gNB-CU can also provide to gNB-DU a recommended future CCO state as assistance information.

2.3	UE performance feedback for CCO

UE performance feedback is used to evaluate the AI/ML model by comparing UE performance before and after a CCO action that is triggered based on the output from AI/ML Model Inference.

CCO is to optimize the network by adjusting cell coverage due to coverage and capacity problems. For AI/ML based CCO, since the predicted CCO issue can be avoided or mitigated by advance CCO action triggered by the AI/ML Model Inference output, UE performance should be not (much) impacted due to the coverage adjustment, especially for the UE in the cell border area.

As specified in TS38.423, legacy UE performance measurements collected by gNB includes Average UE throughput DL, Average UE throughput UL Average packet delay and Average packet loss DL. gNB should get and evaluate UE performance for UEs that may be affected by the CCO action, e.g. UEs in the cell border area. Therefore, UE location information, if available, is suggested to be reported together with UE performance in the DATA COLLECTION UPDATE message, or neighbour gNBs perform UE selection based on UE location and only report the UE performance of the UEs in the cell border area.

Proposal 4: Legacy UE performance measurement metrics can be reused for CCO.

Proposal 5: RAN3 to discuss the following two options for UE performance feedback for CCO:

- To report UE location information together with UE performance in the DATA COLLECTION UPDATE message.

- Neighbour gNBs perform UE selection based on UE location and only report the UE performance of the UEs in the cell border area.

2.4	IE value for predicted CCO issue and predicted future CCO state

We already agreed that Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds, but the maximum value for the time interval is still FFS.

Considering this time interval is one of the predicted information and it needs to align with the predicted time for others, we suggest to follow the defined value for Requested Prediction Time IE, i.e. 1 to 60 seconds.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |

Proposal 6: The maximum value of the Time interval for predicted CCO issue and future CCO state is 60 seconds.

# Proposals

In this contribution, we provide our analysis on open issues for AI/ML-based CCO and made the following proposals:

Proposal 1: Timing information for predicted CCO issue is NOT needed to exchange over Xn.

Proposal 2: Based on the received future CCO state, the receiving gNB-CU can generate a recommended future CCO state for its gNB-DU.

Proposal 3: gNB-CU can also provide to gNB-DU a recommended future CCO state as assistance information.

Proposal 4: Legacy UE performance measurement metrics can be reused for CCO.

Proposal 5: RAN3 to discuss the following two options for UE performance feedback for CCO:

- To report UE location information together with UE performance in the DATA COLLECTION UPDATE message.

- Neighbour gNBs perform UE selection based on UE location and only report the UE performance of the UEs in the cell border area.

Proposal 6: The maximum value of Time interval for predicted CCO issue and future CCO state is 60 seconds.

# Reference

[1] R3-250924, (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN

[2] R3-250925, (BL CR to TS 38.473) Support of enhancements on AI/ML for NG-RAN