3GPP TSG-RAN WG3 #127bis	R3-251687

Wuhan, China, April 7th – 11th, 2025

Agenda item:		16.2

Title:		Discussion on AIoT architecture aspects

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The following progress was made during the last RAN3 meeting:

RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1. WA: Take Option1 (Including AIoTF information in NGAP). WA: The AIoT RAN node may only provide the AIoT radio. AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list. The location of reader is FFS. FFS on whether signaling is needed, further discussion on the detail information and necessity. AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN. AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN. The behavior on the AIoT RAN on how to select the proper readers needs to be further discussed? How to encode the requested service area information is FFS. Focus on the basic signaling procedures approved in SI. Architecture aspects: - AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio. - AIOTF may be aware of the location of reader via OAM configuration. - FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support. - FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT? - Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area. - Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers. - WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list. - FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list. About Inventory Procedures: - WA: Introduce a NGAP Class 1 Inventory Request procedure. - Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE. - Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE. - Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE. - Introduce a NGAP Class 2 Inventory Report procedure. - Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE. - The above Transfer IEs are transparent to AMF in case of indirect communication. - FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier. About Command Procedure: - WA: Introduce a NGAP Class 1 Command Request procedure - Introduce COMMAND REQUEST message, which includes Command Request Transfer IE. - Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE. - Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE. - The above Transfer IEs are transparent to AMF in case of indirect communication. - FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier. - FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure? - FFS: Command request procedure is a per device procedure or to be used for multiple devices? - FFS: The need for a Command Report procedure? - FFS: Device association between gNB and AIOTF is used in Command procedure. About Interface management procedures: - FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF. - FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF. - FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

This contribution will discuss the open issues of Ambient IoT.

# 2. Discussion

2.1	How AIoTF obtain the AIoT RAN information

This question is triggered by the LS R3-250075:

[Question to RAN3] SA2 has concluded for topology 1, the AIOTF selects the AIOT RAN node(s) and optionally provides a list of the BS readers to AIOT RAN. SA2 assumes that OAM can configure the AIOTF with needed information for this selection. SA2 have further discussed whether the AIOT RAN may provide the AIOT RAN information e.g., supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area to the core network. SA2 kindly asks RAN3 to clarify whether RAN3 plan to support the above AIOT RAN information exchange with AIOTF. If so, SA2 kindly ask RAN3 to provide the definition of the appropriate parameters.

During the last meeting, the OAM-based solution was agreed as the baseline approach for the AIoTF to acquire the AIoT RAN information. The agreed AIoT RAN information elements include: the supported AIoT serving area, served reader ID list, and reader location. Several aspects remain for further discussion: whether the AIoTF can obtain AIoT RAN information through RAN-AIoTF signaling (i.e., signaling-based solution); the definition of service area (whether to create a new AIoT-specific area or reuse existing Tracking Areas); and the definition of reader location (e.g., room numbers, GNSS coordinates).

Regarding AIoT service area definition, both legacy Tracking Areas and new AIoT-specific areas are available. However, it should be noted that Tracking Areas were originally designed for UE location and mobility management, typically covering areas ranging from several to hundreds of km2. In contrast, AIoT service areas are intended to describe the service coverage provided by AIoT readers, with expected service areas ranges comparable to cell sizes (tens of m2 to several kilometers km2). This difference in granularity is the reason why we prefer to defining a dedicated AIoT service areas to better accommodate AIoT service requirements.

Proposal 1: We prefer to define a new AIoT service area to better accommodate AIoT service requirements.

For reader location in Topology 1, the reader location is decided by the gNB TRP location. Current NR positioning systems already utilize geographical coordinates (latitude and longitude) to specify TRP location, and this same approach can precisely describe reader location in AIoT service. Additionally, the AIoT service area (as mentioned above) may additionally serve as an approximate location reference.

Proposal 2: The geographical coordinates can precisely describe the reader location, and the AIoT service area can roughly represent the reader location.

Form technique point of view, the OAM-based solution is always workable to exchange the AIoT RAN information exchange, while we have identified no strong advantages to implementing a signaling-based solution.

Proposal 3: The signaling-based solution for AIoT RAN information exchange is not needed.

2.2 Reader selection

A working assumption and a FFS is left in the last meeting.

WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

When the inventory request contains only a reader list from AIoTF to AIoT-enabled gNB, this implies the AIoTF has pre- knowledge of available readers (the AF indicates which reader to use, or the AF preconfigures the AIoTF which reader to use, or the AIoT RAN provides the AIoT service before). In such cases, the gNB shall accept and utilize all the indicated readers for AIoT service.

Proposal 4: Turning “WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list” to agreement.

In cases where the AIoT-enabled gNB receives both area and reader list in the Inventory Request, this is a combined scenario of:

- The existing agreement: "Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area".

- The current working assumption: "Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list".

Therefore, the AIoT-enabled gNB should:

- Select all readers specified in the reader list.

- Select all readers located within the AIoT service area.

Proposal 5: Turning “FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list” to agreement.

2.3 Interface management

RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1.

FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF.

FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF.

FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

Direction connection:

The existing NG interface management procedures can be utilized without modification for indirect connections between AIoT enabled gNB and AIoTF. This is because the NG interface connection between gNB and AMF is established after the TNL association becomes operational, which is independent to the AIoT service requirements.

Proposal 6: The legacy NG interface management procedures should be used for indirection connection between AIoT enabled gNB and AIoTF.

Indirection connection:

The NG interface between AIoT-enabled gNB and AIoTF is established only for AIoT services. As previously discussed, the legacy NG management procedures (including NG SETUP/RESPONSE, RAN CONFIGURATION UPDATE/ACKNOWLEDGE/FAILURE, NG RESET/ACKNOWLEDGE, and ERROR INDICATION) contain some mandatory IEs that may not be relevant for AIoT services.

Before determining whether to reuse existing procedures or define new ones for AIoT services, we should perform a thorough analysis to identify which mandatory IEs can be considered necessary or unnecessary for AIoT service.

NG SETUP mandatory IEs are:

| IE name | If useful for AIoT service |
| --- | --- |
| Message Type | Yes |
| Global RAN Node ID | Yes |
| Supported TA List | No, if the tracking area is not agreed to represent the AIoT service. |
| Default Paging DRX | No |

NG SETUP RESPONSE mandatory IEs are:

| IE name | If useful for AIoT service |
| --- | --- |
| Message Type | Yes |
| AMF Name | No |
| Served GUAMI List | No |
| Relative AMF Capacity | No |
| PLMN Support List | Yes |

RAN CONFIGURATION UPDATE mandatory IEs are:

| IE name | If useful for AIoT service |
| --- | --- |
| Message Type | Yes |

RAN CONFIGURATION UPDATE ACKNOWLEDGE mandatory IEs are:

| IE name | If useful for AIoT service |
| --- | --- |
| Message Type | Yes |

RAN CONFIGURATION UPDATE FAILURE mandatory IEs are:

| IE name | If useful for AIoT service |
| --- | --- |
| Message Type | Yes |
| Cause | Yes |

NG RESET mandatory IEs are:

| IE name | If useful for AIoT service |
| --- | --- |
| Message Type | Yes |
| Cause | Yes |
| CHOICE Reset Type | Yes |

NG RESET ACKNOWLEDGE mandatory IEs are:

| IE name | If useful for AIoT service |
| --- | --- |
| Message Type | Yes |

ERROR INDICATION mandatory IEs are:

| IE name | If useful for AIoT service |
| --- | --- |
| Message Type | Yes |

Based on the analysis above, we observe that:

- NG SETUP/RESPONSE messages contain certain mandatory IEs that are unnecessary for AIoT services.

- The mandatory IEs in RAN CONFIGURATION UPDATE/ACKNOWLEDGE/FAILURE, NG RESET/ACKNOWLEDGE, and ERROR INDICATION messages are useful for AIoT services.

Observation 1: While NG SETUP/RESPONSE messages include unnecessary mandatory IEs for AIoT services, the mandatory IEs in other NG management messages (RAN CONFIGURATION UPDATE/ACKNOWLEDGE/FAILURE, NG RESET/ACKNOWLEDGE, ERROR INDICATION) are applicable to AIoT services.

Therefore, we propose that:

- Introducing new AIoT-specific NG SETUP and NG SETUP RESPONSE messages

- Reusing and enhancing existing RAN CONFIGURATION UPDATE/ACKNOWLEDGE/FAILURE, NG RESET/ACKNOWLEDGE, and ERROR INDICATION messages

Proposal 7: Introduce new AIoT NG SETUP and NG SETUP RESPONSE messages to support direct connections between AIoT-enabled gNB and AIoTF.

Proposal 8: Enhance and reuse the existing RAN CONFIGURATION UPDATE/ACKNOWLEDGE/FAILURE, NG RESET/ACKNOWLEDGE, and ERROR INDICATION messages for AIoT services in direct connection scenarios.

For the direct connection AIoT NG interface between gNB and AIoTF, the interface may be established prior to AIoT service initiation and maintained during service operation. Since the AIoT services are always initiated by the AIoTF, we propose the AIoT NG interface establishment should be initiated by the AIoTF.

Also, considering the network resource consumption and signaling overhead of maintaining the NG interface, it should be released after AIoT service completion, and the NG REMOVAL REQUEST/RESPONSE procedure (already agreed for NR NTN) can be reused for AIoT NG interface release

Proposal 9: The AIoTF should initiate establishment of the direct connection AIoT NG interface.

Proposal 10: The NG REMOVAL REQUEST/RESPONSE procedure should be used for AIoT NG interface release.

# 3. Conclusion

Observation 1: While NG SETUP/RESPONSE messages include unnecessary mandatory IEs for AIoT services, the mandatory IEs in other NG management messages (RAN CONFIGURATION UPDATE/ACKNOWLEDGE/FAILURE, NG RESET/ACKNOWLEDGE, ERROR INDICATION) are applicable to AIoT services.

Proposal 1: We prefer to define a new AIoT service area to better accommodate AIoT service requirements.

Proposal 2: The geographical coordinates can precisely describe the reader location, and the AIoT service area can roughly represent the reader location.

Proposal 3: The signaling-based solution for AIoT RAN information exchange is not needed.

Proposal 4: Turning “WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list” to agreement.

Proposal 5: Turning “FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list” to agreement.

Proposal 6: The legacy NG interface management procedures should be used for indirection connection between AIoT enabled gNB and AIoTF.

Proposal 7: Introduce new AIoT NG SETUP and NG SETUP RESPONSE messages to support direct connections between AIoT-enabled gNB and AIoTF.

Proposal 8: Enhance and reuse the existing RAN CONFIGURATION UPDATE/ACKNOWLEDGE/FAILURE, NG RESET/ACKNOWLEDGE, and ERROR INDICATION messages for AIoT services in direct connection scenarios.

Proposal 9: The AIoTF should initiate establishment of the direct connection AIoT NG interface.

Proposal 10: The NG REMOVAL REQUEST/RESPONSE procedure should be used for AIoT NG interface release.

# References