3GPP TSG-RAN WG3 Meeting #127bis														R3-251733

Wuhan, China, 7th – 11th April 2025

Agenda item:	21.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the new RAN3 related objectives

Document for:	Discussion and Decision

# 1	Introduction

RAN#107 agreed new objectives for RAN3 XR:

-	Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]:

-	Specify uplink congestion signalling [RAN2, RAN3]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

-	Support of PDU set based QoS handling enhancement [RAN3]:

-	Support of DL PDU Set marking without PDU Set QoS

-	Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

-	QoS Handling when Traffic Characteristics Change Dynamically [RAN3]

-	TTNB and burst size to be provided over GTP-U

-	Support of exposure of available data rate [RAN3]

Note: Coordination with SA2, as needed

This contribution analyses these objectives, and proposal a way forward.

# 2	Support and specify multi-modality awareness for QoS flows in both DL and UL RAN

SA2 has agreed CR to provide the MMSID for a QoS flow to NG-RAN. The MMSID information also need to be provided to gNB-DU and gNB-CU-UP, and to target NG-RAN during handover procedure. A set of TPs have been co-sourced by many companies during offline discussion.

Proposal 1-1: agree to introduce MMSID in NGAP, F1AP, E1AP, and XnAP.

For the usage of MMSID, RAN2 LS ([2]) states:

- RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

However, SA4 reply LS ([3]) states:

1. Overall Description: SA4 thanks RAN2 LS in S4-241802/R2-2409272 on multi-modality awareness, and would like to provide the feedback as following. - RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation). Answer: From an XR service perspective, it is generally desirable that the gNB should try to grant all associated QoS Flows for the XR service whenever possible. However, not all applications require all QoS flows to be granted. If the gNB cannot grant all associated QoS Flows, only the application layer can determine how to handle the case when the QoS flow for one modality cannot be established e.g. due to RAN resource limitations. For instance, it may be acceptable to an application if the QoS flow for video cannot be established with the requested QoS requirements but it may not be acceptable if the QoS flow for audio cannot be established. The conditions under which particular QoS flows are considered essential for an application to continue are application specific and cannot be determined by 5GS. - For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. - RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the mutli-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.. Answer: For DL, from the application perspective, there are well-defined mechanisms to support synchronized playout among different media flows, e.g. RTP timestamps, jitter buffering mechanism in the RTP client. These mechanisms guarantee frame level synchronization but do not provide packet level synchronization among different media flows. There is no evidence to suggest 5GS per packet inter-modal synchronization is needed for XR services, as the very low round trip latency requirements will prevent the system from introducing unacceptable inter-modal skew. Regarding PDU Set discard, it is recommended that the 5GS not jointly discard packets across multi-modal flows. Receiving packets for some modalities is preferred than receiving none at all as this enables the application to use the received modalities to better manage the user experience (e.g., pause the video while rendering received audio packets, or render received video packets while concealing an audio packet drop).

SA4 LS indicates the gNB should not perform join discard. So joint discard should not be specified for the NG-RAN.

For admission control, it is up to the gNB’s implementation. So it is proposed to capture following in Stage-2 TP:

The Multi-modal Service ID may also be provided by the SMF to the gNB as part of the QoS parameters of the QoS flow. It is up to the gNB’s implementation to use this information, e.g., joint admission control and QoS flow to DRB mapping.

Proposal 1-2: agree Stage-2 TP ([5]) for MMSID.

# 3	Specify uplink congestion signalling

RAN3 received a LS ([4]) from RAN2.

1. Overall Description: For XR rate control, RAN2 discussed how the gNB gets the information of QoS flows which are subject to uplink rate control, i.e., whether the information is from CN or UE, and agreed that: - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. 2. Actions: To SA2, RAN3. ACTION: RAN2 respectfully asks SA2 and RAN3 to take the above decision into consideration for further work on XR uplink rate control and provide feedback.

Per RAN2 LS, it is assumed the gNB receive this information from CN. This is similar to other XR related control information that gNB received from the CN, e.g. ECN marking or congestion information reporting, MMSID, available data reporting, etc. It is straightforward that same approach is reused for this case, i.e. gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control. Even it is SA2 to make the final decision, it is necessary to inform SA2 that RAN3 agree with RAN2 assumption.

Proposal 2-1: agree a reply LS to RAN2 and SA2 that RAN3 agree with RAN2 assumption, i.e. the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control.

The draft Reply LS can be found in ([6]).

Before we discuss the two options on how gNB informs UE on UL rate control, it is necessary to first discuss how the gNB know which QoS flow requires UL rate control. In this meeting, RAN3 received a LS ([4]) from RAN2 on uplink rate control states:

1. Overall Description: For XR rate control, RAN2 discussed how the gNB gets the information of QoS flows which are subject to uplink rate control, i.e., whether the information is from CN or UE, and agreed that: - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. 2. Actions: To SA2, RAN3. ACTION: RAN2 respectfully asks SA2 and RAN3 to take the above decision into consideration for further work on XR uplink rate control and provide feedback.

The previous RAN2 LS listed two options:

- Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

From the RAN2 agreement above, there are two options for rate control signalling:

- Option 1: QoS flow information for rate control is included in the rate control MAC CE.

- Option 2: The QoS flow information for rate control is provided via RRC, while the DRB information is included in the rate control MAC CE.

Regarding whether the XR related control is per DRB or per QoS flow, RAN3 have discussed similar issues in Rel-18, and agreed the XR related control is performed per DRB. For example,

- TS 38.473:

If the ECN Marking or Congestion Information Reporting Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it accordingly for the specific DRB.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

- TS 38.425:

5.5.3.61 UL Congestion Information

Description: For the cases of ECN marking Request, this field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked for a DRB.

For the case of Congestion Information Request, this field should be interpreted as a percentage of congestion level in UL up to two decimal points for a DRB.

5.5.3.62 DL Congestion Information

Description: For the cases of ECN marking Request, this field indicates the percentage of DL IP packets up to two decimal points that should be ECN marked for a DRB.

For the case of Congestion Information Request, this field should be interpreted as a percentage of congestion level in DL up to two decimal points for a DRB.

The RAN2 agreement about per QoS flow Rate indication requires a different handling than current XR related control in F1 and gNB-DU.

Observation 2-1: in current F1, the control of ECN marking or congestion information reporting, and PSI based SDU discard UL is performed per DRB.

Regarding the two options from RAN2, both options have their own pros and cons. In the following we consider following aspects to analyze each option:

- Flexibility of QoS flow level rate control

- Impact on protocol stack separation

- Spec and cross WGs impact

In option 1, gNB-CU inform gNB-DU for the QoS Flow requiring rate control. The gNB-DU further send the rate control MAC CE directly indicating the index of any QoS flow to which the rate control is applied, offering more flexibility, especially when multiple QoS flows are subject to rate control. Currently, the maximum number of QFI is 63 as defined in TS 38.331, so QoS flow index field could use 6 bits in the MAC CE instead of LCID. However, exposing QoS flow level information at MAC may violate of the current principle of protocol stack separation. In addition, it will be a major change to current gNB-DU implementation.

Observation 2-2: Option 1 have major impact to gNB-DU implementation.

In option 2, gNB-CU first configure via RRC signaling to which QoS flow(s) the RB level MAC CE is to be applied when there are multiple QoS flows mapped to a single DRB. The gNB-CU also need to inform gNB-DU for the DRB requiring rate control, so the gNB-CU can send the MAC CE. Current F1AP cannot provide the information to gNB-DU, so this option requires enhancement to F1AP.

It is worthy to also note that current MAC CE provide the Recommended Bit Rate to UE. TS38.300 states:

For uplink or downlink bit rate adaptation, gNB may send a recommended bit rate to the UE to inform the UE on the currently recommended transport bit rate on the local uplink or downlink, which the UE may use in combination with other information to adapt the bit rate, e.g. the UE may send a bit rate request to the peer UE via application layer messages as specified in TS 26.114 [24], which the peer UE may use in combination with other information to adapt the codec bit rate.

When multiple QoS flows are multiplexed to same DRB, the UE already have the capability to derive the bit rate request for a specific QoS flow. So Option 2 can continue to reuse this UE capability. Again, this is in RAN2 scope and no need to further discuss the detail in RAN3.

Observation 2-3: It is possible to perform QoS flow level rate control using the RRC configurations and legacy RB level MAC CE.

Based on above analysis, Option 2 is slightly better than Option 1. We suggest adopt Option 2.

Proposal 2-2: RAN3 agree Option 2 is preferrable.

# 4	Support of PDU set based QoS handling enhancement

Support of DL PDU Set marking without PDU Set QoS

SA2 has agreed following:

If no DL PDU Set QoS parameters are included in the PCC rule, but a DL Protocol Description is included in the PCC rule received by the SMF and the PSA UPF supports PDU Set Handling (i.e. supports to perform PDU Set Information marking), the SMF may, based on the local operator policy, send to the NG-RAN, a DL PDU Set Information Marking Support Indication indicating that the DL PDU Set Information marking is supported.

If the DL PDU Set Information Marking Support Indication has been sent to the 5G-AN (as described in clause 5.37.5.1) and the 5G-AN has provided a PDU Set based handling support Indication indicating that PDU Set handling is supported, the SMF instructs PSA UPF to perform PDU Set marking and provides the PSA UPF the DL Protocol Description used by the service data flow.

This requires following enhancement to NGAP:

* From SMF to RAN: the SMF provide an indication that DL PDU Set Information marking is supported. This requires a new indication in NGAP, e.g. a new Indication IE in QoS Flow Level QoS Parameters IE.

* From RAN to SMF: the RAN provide an indication so that CN perform PDU Set marking. This can be supported by current PDU Set based Handling Indicator IE.

Proposal 3-1: introduce a new indication IE in QoS Flow Level QoS Parameters IE.

Alternative PDU Set QoS

SA2 has agreed following：

The Alternative QoS Profile(s) shall also contain the PDU Set QoS parameters PSDB and PSER in the respective direction(s) when the QoS Profile contains PDU Set QoS parameters PSDB and PSER in UL and/or DL as described in clause 5.7.7; otherwise, the corresponding Alternative QoS Profile(s) shall not contain the PDU Set QoS parameters PSDB and PSER.

If the NG-RAN determines that only in one direction the GFBR, the PDB or the PER cannot be fulfilled, the NG-RAN shall indicate that "GFBR can no longer be guaranteed" together with the corresponding direction.

One option is to enhance current Alternative QoS Parameters Set List IE. For example, the Alternative QoS Parameters Set Item IE includes new PSDB Downlink/Uplink and PSER Downlink/Uplink. The behavior text remains unchanged.

The PDU Session Resource Notify Transfer IE also need to be enhanced to indicate the direction for the notification. For example, add new codepoints “not fulfilled DL” and “not fulfilled UL” in the Notification Cause IE.

Proposal 3-2: introduce PSDB and PSER in NGAP Alternative QoS Parameters Set Item IE, and introduce new codepoints “not fulfilled DL” and “not fulfilled UL” in the Notification Cause IE.

TP for TS 38.413 can be found in ([7])

# 5	QoS Handling when Traffic Characteristics Change Dynamically

According to SA4 TS 26.522, the TTNB and Burst size are defined as below:

-	Burst Size [BSSize] (24 bits): The Burst Size indicates the total size of the burst to be transmitted (in bytes (including the overhead of the RTP Header).). If the burst size is not known it shall be set to 0

-	Time To Next Burst [TTNB] (16 bits): Indicates the approximate time in tenth of milliseconds to the next burst  If the time to next burst is not known, it shall be set the reserved value 65535.

Both BSSize and TTNB need to be added in DL PDU SET INFORMATION (PDU Type 0) frame. TS 38.300 BL CR also need to be updated to add the BSSize and TTNB.

Proposal 4-1: Add BSSize and TTNB in DL PDU SET INFORMATION (PDU Type 0) frame.

Proposal 4-2: Agree TP to BL CR for TS 38.300 in [9]

# 6	Support of exposure of available data rate

In last meeting, RAN3 agreed

… the available data rate reported to CN refers to the data rate that gNB is currently able to provide for a QoS Flow with GFBR, and it is up to gNB's implementation to derive the value of available data rate.

The RAN3 LS listed some aspects requiring confirmation by SA2. For example, RAN3 assumes that the SMF will not request Alternative QoS Profiles together with available data rate reporting for one QoS flow.

While the detail needs to be further confirmed by SA2, SA2 has agreed the general principal for this feature. The RAN3 related aspects are listed below:

For a GBR QoS Flow, based on the PCC rule from PCF, the SMF requests the NG-RAN to report the UL and/or DL available bitrate. The NG-RAN reports the UL and/or DL available bitrate information to the PSA UPF via the GTP-U header of UL packets.

In detail, the required enhancement in RAN3 is listed below:

- For the control plane, the 5GC need to request the gNB about which DRB QoS flow requires the available data reporting. Then the gNB-CU need to forward the request to the gNB-DU and gNB-CU-UP.  During the mobility, target gNB also need to know it either from source gNB over XnAP, or from AMF over NGAP.

- For the user plane, the gNB-DU need to report the available data report to gNB-CU-UP, which is then further reported to the gNB-CU.

Proposal 5: RAN3 agree following enhancement to support exposure of available data rate

- For NGAP/XnAP/F1AP/E1AP, introduce a new “request” IE to inform the peer node that exposure of available data rate for a QoS flow is requested.

- For F1-U/NG-U, introduce the available date report for UL, or DL, or both UL and DL.

TP for TS 38.413 can be found in ([8]).

# 7	Conclusion

In this contribution, we have analyzed the new RAN3 related objectives. Our proposals are:

For multi-modality awareness

Proposal 1-1: agree to introduce MMSID in NGAP, F1AP, E1AP, and XnAP.

Proposal 1-2: agree Stage-2 TP ([5]) for MMSID.

For Specify uplink congestion signalling:

Proposal 2-1: agree a reply LS to RAN2 and SA2 that RAN3 agree with RAN2 assumption, i.e. the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control.

The draft reply LS can be found in ([6])

Proposal 2-2: RAN3 agree Option 2 is preferrable.

For Support of PDU set based QoS handling enhancement

Proposal 3-1: introduce a new indication IE in QoS Flow Level QoS Parameters IE.

Proposal 3-2: introduce PSDB and PSER in NGAP Alternative QoS Parameters Set Item IE, and introduce new codepoints “not fulfilled DL” and “not fulfilled UL” in the Notification Cause IE.

TP for TS 38.413 can be found in ([7])

For QoS Handling when Traffic Characteristics Change Dynamical:

Proposal 4-1: Add BSSize and TTNB in DL PDU SET INFORMATION (PDU Type 0) frame.

Proposal 4-2: Agree TP to BL CR for TS 38.300 in [9]

For Support of exposure of available data rate:

Proposal 5: RAN3 agree following enhancement to support exposure of available data rate

- For NGAP/XnAP/F1AP/E1AP, introduce a new “request” IE to inform the peer node that exposure of available data rate for a QoS flow is requested.

- For F1-U/NG-U, introduce the available date report for UL, or DL, or both UL and DL.

TP for TS 38.413 can be found in ([8]).

# References

- RP-250107	Revised WID on XR (eXtended Reality) for NR Phase 3

- R2-2409272	Reply to SA2 LS on multi-modality awareness

- R3-250027	LS Reply on multi-modality awareness	SA4

- R3-251513	LS on uplink rate control  RAN2

- R3-251734 	(TP to BL CR for TS 38.300) Addition of MMSID

- R3-251737	[Draft]Reply LS on uplink rate control

- R3-251735	(TP to BL CR for TS 38.413) enhancement for DL PDU Set marking without PDU Set QoS, and Alternative PDU Set QoS with PSDB and PSER

- R3-251736	(TP to BL CR for TS 38.413) enhancement for available data rate report

- R3-251738	(TP to BL CR for TS 38.300) Support for Burst Size and TTNB