3GPP TSG-RAN WG3 Meeting #127-bis	R3-251742

Wuhan, China, April 7th – 11st, 2025

Agenda item:	13.2

Source: 	LG Electronics Inc.

Title: 	Discussions on key remaining aspects of Inter-CU LTM

Document for:	Discussion and Decision

# Introduction

The time for Rel-19 Inter-CU LTM feature completion is approaching (June 2025). The relevant agreements made in RAN3 and RAN2 so far are provided in Annex A and B, respectively.

In this contribution, we provide our views on key remaining aspects of Rel-19 Inter-CU LTM design.

# Discussion

## Per-cell LTM preparation: avoiding multiple UE-associations

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

While RAN3 moved forward with per-cell HO preparation, we still have WA on how to avoid multiple UE-associations between two CUs over Xn.

If we do nothing, then the current XnAP signalling may result in multiple UE-associations for the same UE between two CUs for different candidate cells. In previous releases, this was allowed by the source CU and the target CU treated each UE-association separately, as if they belonged to different UEs. Also, managing these multiple UE-associations was left to the source CU's implementation.

This was acceptable in previous releases because CHO was designed for a one-time cell switch, after which candidate cell configurations were released in both the UE and NW following a successful execution. However, regarding UE-association, Rel-19 Inter-CU LTM introduces more complex interactions over XnAP, not only during the preparation phase but also after each cell switch (for subsequent LTM, shown below):

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

As a result, allowing multiple UE-associations for Inter-CU LTM would create several challenges: (a) It would complicate the identification of the same UE when establishing UE-association in-between C-CUs for subsequent LTM, due to each C-CU having multiple UE associations and multiple pair of UE XnAP IDs associated with S-CU; (b) Needless to say, multiple UE-associations (each tied with different candidate cells) would complicate error handlings throughout LTM operations.

Given these issues, we think it is critical to prevent multiple UE-associations from the initial LTM preparation phase. As observed during online discussions [1], the best and simplest solution would be to enforce a single UE-association between CUs during Inter-CU LTM preparation phase. This approach would ensure that (a) the risk of multiple UE-associations is blocked from the start; (b) No additional UE XnAP ID needs to be encoded into HO REQ message (avoiding unnecessary stage-3 impact); and (c) Any CU can easily identify and maintain a single UE-association with other CU throughout LTM operations. By adopting this approach, we can effectively eliminate unnecessary complexities and streamline UE-association management over Xn for Inter-CU LTM.

Proposal 1: To simplify XnAP signalling management and improve error handling between CUs throughout LTM operations, turn WA into agreement that enforces a single UE association for parallel transactions during LTM preparation.

Proposal 2: As a solution, allow parallel transactions using the same source UE AP ID only, with each identified by a unique target cell ID and only one UE-associated signaling connection being established/maintained (as illustrated below). FFS where to capture (either in TS 38.300 or TS 38.423).

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM, parallel transactions are allowed using the same source UE AP ID only. In this case, possible parallel requests are identified by the target cell ID, and only one UE-associated signaling connection is established and maintained between the source NG-RAN node and the target NG-RAN node. The procedure uses UE-associated signalling.

## Direct UE-associated Xn signalling connections between C-CUs for subsequent LTM

This is also an important and unique aspect of LTM in the context of Inter-CU Xn signaling design, but in the last meeting, this discussion was skipped [1] due to its inter-dependency with WA on enforcing a single UE association during the LTM preparation phase. We re-iterated our views and analysis for proper discussions.

The Inter-CU LTM can be viewed as a series of NW-triggered conditional handovers executed sequentially. In Rel-16, CHO was designed only for single execution from the source currently serving the UE toward one of the candidate targets. If executed serially, the selected target becomes the new radio access management point of the UE, and subsequently determines, as a “source”, for the next handover of the served UE toward another RAN node, and so on. This nature inherently allows the XnAP descriptions based on the "source" and "target" to be applied successively for subsequent Inter-CU LTM.

From this sense, RAN3 acknowledged the fact that, once we have direct UE-associations in-between C-CUs, any procedure triggered by a C-CU at any point of time can be made to work as effective as from S-CU for initial PCell switch, and our specification efforts and signaling overhead can be greatly reduced as their stage-2/3 descriptions can remain universally applied regardless of how many CUs are involved for LTM:

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

So far, what RAN3 has agreed is (a) the need of direct UE-association between C-CUs; (b) when it should be established and via which Xn signalling (i.e. after each Inter-CU LTM cell switch, using LTM Configuration Update – the first message expected in-between). This is a great milestone indeed, but we still have leftover issues of

- How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

- How to identify the same UE?

### (1) How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution from S-CU) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

First, we would like to point out that this issue doesn’t exist when only one C-CU (other than S-CU) is involved from the beginning (i.e. a total of two CUs). In this case, there is already a direct UE-association between two CUs (S-CU <> C-CU) over Xn, and it can be re-used for subsequent LTM. Only when the number of CUs involved exceeds two (i.e. more than one C-CU), this issue becomes evident for establishing direct UE-associations in-between C-CUs.

Having that in mind, once an initial Inter-CU PCell switch happens from S-CU toward a candidate PCell in one C-CU, we first observe that this C-CU needs to know the existence of another CU(s) involved for the same UE’s LTM. Otherwise, establishing direct UE-associations (or even first seeking for Xn connectivity if not connected beforehand) would not be a question.

Overall, we think that leveraging the LTM Configuration ID Mapping List IE is the best way. This IE is used for assigning the unique identifiers for each candidate cell configuration, allocated and managed by S-CU (RAN3-125bis already agreed: The LTM Configuration IDs are allocated by the source CU). Moreover, similar to the (common) CSI resource configurations of candidate cells, this IE shall also be maintained up-to-date (whenever update happens) and synced to all the involved C-CUs for correct LTM operations. Currently, this IE is captured at bare minimum in the current BL CR [2], i.e. a list of IDs (1 to 8) for each candidate cell. But if we can extend this IE to also include the information of which candidate cell belongs to which CU, any recipient CU of this IE would be able to know which CUs are involved for the UE’s LTM and which candidate cells are currently planned.

In fact, making involved entities visible with each other is nothing new. In Rel-18 of (Intra-CU) Inter-DU LTM, each involved DU was also visible to other DUs based on early UL sync information handling. The preamble indexes for early UL synchronization are allocated and shared per DU basis, and the early UL sync resource request from CU can indicate a list of involved DU IDs [3]. Though this aspect was not addressed by F1AP LTM Configuration ID Mapping List IE, at least we can say that the similar handling of letting each involved DU know the existence of other involved DU(s) was already in place in Rel-18.

Moreover, one advantage of this approach observed is that timing of information update (i.e. “when”) doesn’t need to be worried about. As mentioned above, the LTM Configuration ID Mapping List IE shall anyway be maintained up-to-date and synched to all the involved entities throughout LTM operations of a UE. Whenever there is a need for update, it will be updated in NW side and UE side (though synch to the UE may not be always required case-by-case).

### (2) How to identify the same UE?

A reference to a UE context should be included in the LTM Configuration Update message (first message for direct UE-association) triggered from the new serving C-CU so that the recipient C-CU can associate with the same UE.

In the past, identifying the same UE context already established in one entity upon request from other entity was addressed in RAN3 based on “UE XnAP ID” [4].

- During Inter-MN HO without SN change, the same SN identifies the already established UE context when the target MN decides to add the same SN, by the “SN UE XnAP ID” included in SN ADD REQ, which is forwarded from the source MN via HO REQ.

- Another practice was at Rel-17 CHO with NR-DC, where the same target SN identifies the same UE SN addition request from different target MN(s), based on the pair of the source MN node ID and the MN UE XnAP ID included in each SN ADD REQ, which is also forwarded from the source MN via HO REQ.

Two slightly different approaches were taken for each practice:

- In the first case, an entity requesting for the same UE identification directly provided the UE XnAP ID assigned by the recipient before.

- For the second case, the requesting entities provide the common UE XnAP ID for the recipient to identify that multiple requests (from different entities) are indeed for the same UE, where the common ID was selected as the identifier assigned at the source side.

When it comes to having direct UE-associations in-between C-CUs, the first approach can be applied as:

- Each C-CU would have already established a UE-association with S-CU and the corresponding “Target UE XnAP ID” has been assigned for the interaction with S-CU (during the first round-trip HO preparation). When a new serving C-CU initiating to establish direct UE-association with other C-CU, the requesting C-CU can use the “Target UE XnAP ID” (assigned by the receiving C-CU for interaction with S-CU), to be used as a reference to the UE context established in the receiving C-CU.

- If we extend this approach successively for subsequent LTM, the pre-requisite will be for a new serving C-CU to know the “Target UE XnAP IDs” assigned by other C-CUs (for interaction with the old serving CU), before it initiates direct UE-association with other C-CU(s).

The second approach can be worked out by:

- A new serving C-CU who initiating to establish direct UE-association with other C-CU uses the “Source UE XnAP ID” (assigned by S-CU toward each receiving C-CU during initial preparation).

- If we extend this approach successively for subsequent LTM, then the pre-requisite will be for the new serving CU to know the “Source UE XnAP IDs” assigned by the old serving CU toward other C-CUs. In addition, this may require to include the old serving CU’s node ID as well to refer to the right UE context, because from the receiving CU point of view, the reference to a UE context is the “Source UE XnAP ID” assigned by the other node (i.e. by the old serving CU).

Or, both approaches can be mixed up to provide both UE XnAP IDs assigned by the peer CUs.

But, regardless of which UE XnAP IDs to take as a reference, the reference info shall be provisioned to a new serving CU (after each Inter-CU LTM cell switch), so that it can use those references to make the receiving CU to identify the same UE when initiating direct UE-association in-between.

For that, we think that Cell Switch Notification is a natural choice as it is triggered from the old serving CU to the new serving CU whenever Inter-CU LTM cell switch happens.

However, this alone is not enough due to a failure/re-attempt scenario for which the UE may fail cell switch to the indicated CU and access to another CU. In this case, the old serving CU receives HO SUCCESS from a CU that it didn’t send the Cell Switch Notification to. Moreover, Cell Switch Notification would not be applicable for Conditional LTM, for which the old serving CU knows the Inter-CU LTM execution only when it receives HO SUCCESS from the new serving CU. For provision of UE context reference to this unintended new serving CU, the LTM Configuration Update shall also be able to carry the references.

Considering above analysis and observations, we propose the followings:

Proposal 3: For direct UE-associations between C-CUs, the leftover issues are on

- How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

- How to identify the same UE?

Proposal 4: For (2) to work, after each Inter-CU LTM cell switch, the old serving CU shall provision to the new serving CU, UE context “reference” to be included in the subsequent LTM Configuration Update procedure (first message) toward other CU for establishing direct UE-association in-between.

Proposal 5: For provisioning “reference” from the old serving CU to the new serving CU,

- Cell Switch Notification to include such “reference”, for normal NW-triggered LTM.

- LTM Configuration Update to include such “reference”, for failure/re-attempt and Conditional LTM.

Proposal 6: Considering that the LTM Configuration ID Mapping List IE shall be maintained up-to-date (whenever update happens) and synced to all the involved CUs for correct LTM operations,

- Address both (1) and (2) by including the IE into Cell Switch Notification (Note that this LTM Configuration ID Mapping List IE is already included inside the LTM Configuration Update message).

- Enhance the IE to include

- The information of which candidate cell belongs to which CU (for (1)).

- UE context “reference” associated with each candidate cell, which can be used by the new serving CU for establishing direct UE-association with other CUs (for (2)).

Proposal 7: FFS on which UE XnAP IDs to take as reference, either “Source UE XnAP ID” assigned by the old serving CU, or “Target UE XnAP ID” assigned for the old serving CU, or both.

## Requesting reference configuration from C-CU

For Inter-CU LTM, RAN3-126 (Orlando) discussed the (non-technically agreed in RAN2) single reference configuration option vs the per-CU multiple reference configuration approach. However, per-CU reference option was not pursued with no consensus, largely because many companies didn’t fully recognize the benefits of having multiple references or its underlying design concept for enabling RRC signalling efficiency/flexibility in subsequent mobility scenarios [5][6][7].

So, following the single reference configuration, RAN3-127 (Athens) progressed with one FFS:

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message.

The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message.

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

Considering these situations, regarding FFS, we don’t see a clear reason for S-CU to select a specific C-CU (out of potentially many) and request a “single” reference to be used for all the candidate cell configurations.

As already agreed in RAN3, S-CU can provide a reference to a recipient C-CU which must either apply delta or complete configurations based on the received reference. But if S-CU doesn’t provide a reference (anyway optional), a recipient C-CU can independently decide to generate its candidate cell configurations based on its own reference and send it back to S-CU. Please note that S-CU has no idea whether and how the candidate cell(s) requested for a given C-CU will be admitted or not. Before admission result is known, requesting a reference from a specific C-CU from the beginning (i.e. in the initial HO REQ message) doesn’t seem to align with its design purpose and concept to optimize/reduce RRC signalling overhead.

If S-CU receives one or more reference configuration(s) from one or more C-CU(s), then it can decide which one to adopt (including its own configuration as a reference), based on factors such as # of candidate cells associated per C-CU, the UE’s mobility pattern, etc. If no reference is generated and supplied from any C-CU, then S-CU can choose its own configuration as a reference for all the involved C-CU(s) to follow. The selected reference can be provided for C-CU(s) to update their configurations accordingly (as agreed).

In sum, there is no clear reason or benefit for S-CU to “explicitly” request a reference from a specific C-CU. If S-CU doesn’t provide a reference, each recipient C-CU can decide whether to generate one for its candidate cells and, if so, provide it back to the S-CU for subsequent handling. Anyway, the current UE configuration at S-CU is always provided in initial HO REQ (UE Context Information > RRC Context = HandoverPreparation). Each C-CU can evaluate, considering admission status, whether generating a reference is beneficial. For example, if there is only one candidate cell associated with a specific C-CU, having a reference for that cell would not provide any benefit from C-CU point of view. Since the benefit of generating a reference depends on actual admission at each C-CU, we think it is logical for C-CU to determine on its own (if no reference was provided) and generate/provide back to S-CU for further discretion, without explicit request from S-CU.

Proposal 8: No clear reason or benefit is observed for S-CU to “explicitly” request a reference configuration from a specific C-CU, especially given that S-CU has no idea whether and how the candidate cell(s) requested for a given C-CU will be admitted or not.

Proposal 9: Without explicit request from S-CU, the single reference can work for Inter-CU LTM by

- S-CU can provide a reference configuration to a recipient C-CU which must apply delta or complete configurations based on the received reference (as already agreed).

- If S-CU doesn’t provide a reference (anyway optional), a recipient C-CU can independently decide to generate its own reference and send it back to S-CU.

- S-CU can decide which one to adopt (including its own configuration) as a reference, and the chosen reference can be shared with C-CU(s) for updating the candidate cell configurations accordingly (as already agreed).

## Semi-Persistent CSI-RS support

For SP CSI-RS coordination, RAN3-127 (Athens) made a good progress:

For both inter-CU and intra-CU cases:

For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell.

For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

With respect to whether to introduce a new class-1 procedure or re-use the existing one, we think a new procedure is better and cleaner. The serving cell’s activation/deactivation decision of CSI-RS transmissions in a candidate cell may involve up to 3 round-trip singallings across the serving DU, serving CU, candidate CU, and candidate DU. These interactions are complicated and tightly inter-dependent, requiring coordinated class-1 request/response handlings from end to end. Having dedicated class-1 procedures over Xn and F1 would be cleaner and future-proof rather than re-using the existing UE-associated procedure which have been abused and complicated already.

Proposal 10: Adopt new dedicated class-1 procedures over Xn and F1 to support the serving cell’s activation/deactivation decision of CSI-RS transmissions in a candidate cell, which may involve up to 3 round-trip signallings across the serving DU, serving CU, candidate CU, and candidate DU, and require inter-dependent class-1 request/response from end to end.

With that, another important aspect is the possibility that a candidate cell (who requested by the serving cell to start or stop CSI-RS transmissions for the UE) may not follow as exactly the serving cell wants. Even if CSI-RS transmission is initiated as requested, it may be stopped unexpectedly (or may have to be stopped for various reasons) before de-activation is requested by the serving cell. To ensure a future-proof and robust design, we believe it is important to specify a mechanism that a candidate cell who inevitably stopped the requested CSI-RS transmissions for the UE notifies the serving cell, so that the serving cell can take subsequent actions (e.g. de-activate the UE’s CSI-RS measurement for that candidate cell via MAC CE).

Proposal 11: For SP CSI-RS, also consider a mechanism for a candidate cell’s DU to notify the serving DU if the CSI-RS transmissions requested from the serving DU are (or has to be) stopped.

## Others

Regarding TA handling, there were several additions worked out in Rel-18 that CU can share with a DU: (a) a known TA value (i.e. TA value = 0) between the current serving cell and a candidate cell; and (b) whether UE-based TA measurement can be applied (and configured to the UE) in-between the serving and candidate cells. The former was an optimization to cater a special deployment (co-located cells), while the latter was one of early UL synchronization mechanism specified that is utilized only when DU knows so.

While those TA handling perfectly fits within a single CU scope (as specified in Rel-18), a question is whether they can be applicable between cells under different CUs.

It is hard to imagine that either co-located cells are managed under different CUs, or the 260ns TAE (timing alignment error) requirement could be satisfied between cells under different CUs [8].

Before extending, the usage of those TA handlings across CUs shall be validated.

Proposal 12: Do not consider additional TA handlings specified in Rel-18 across CUs (a known TA value and UE-based TA measurement, both of which were specified applicable within a single CU), unless their usage across CUs is validated.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

## Per-cell LTM preparation: avoiding multiple UE-associations

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

Proposal 1: To simplify XnAP signalling management and improve error handling between CUs throughout LTM operations, turn WA into agreement that enforces a single UE association for parallel transactions during LTM preparation.

Proposal 2: As a solution, allow parallel transactions using the same source UE AP ID only, with each identified by a unique target cell ID and only one UE-associated signaling connection being established/maintained (as illustrated below). FFS where to capture (either in TS 38.300 or TS 38.423).

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM, parallel transactions are allowed using the same source UE AP ID only. In this case, possible parallel requests are identified by the target cell ID, and only one UE-associated signaling connection is established and maintained between the source NG-RAN node and the target NG-RAN node. The procedure uses UE-associated signalling.

## Direct UE-associated Xn signalling connections between C-CUs for subsequent LTM

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

Proposal 3: For direct UE-associations between C-CUs, the leftover issues are on

- How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

- How to identify the same UE?

Proposal 4: For (2) to work, after each Inter-CU LTM cell switch, the old serving CU shall provision to the new serving CU, UE context “reference” to be included in the subsequent LTM Configuration Update procedure (first message) toward other CU for establishing direct UE-association in-between.

Proposal 5: For provisioning “reference” from the old serving CU to the new serving CU,

- Cell Switch Notification to include such “reference”, for normal NW-triggered LTM.

- LTM Configuration Update to include such “reference”, for failure/re-attempt and Conditional LTM.

Proposal 6: Considering that the LTM Configuration ID Mapping List IE shall be maintained up-to-date (whenever update happens) and synced to all the involved CUs for correct LTM operations,

- Address both (1) and (2) by including the IE into Cell Switch Notification (Note that this LTM Configuration ID Mapping List IE is already included inside the LTM Configuration Update message).

- Enhance the IE to include

- The information of which candidate cell belongs to which CU (for (1)).

- UE context “reference” associated with each candidate cell, which can be used by the new serving CU for establishing direct UE-association with other CUs (for (2)).

Proposal 7: FFS on which UE XnAP IDs to take as reference, either “Source UE XnAP ID” assigned by the old serving CU, or “Target UE XnAP ID” assigned for the old serving CU, or both.

## Requesting reference configuration from C-CU

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message.

The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message.

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

Proposal 8: No clear reason or benefit is observed for S-CU to “explicitly” request a reference configuration from a specific C-CU, especially given that S-CU has no idea whether and how the candidate cell(s) requested for a given C-CU will be admitted or not.

Proposal 9: Without explicit request from S-CU, the single reference can work for Inter-CU LTM by

- S-CU can provide a reference configuration to a recipient C-CU which must apply delta or complete configurations based on the received reference (as already agreed).

- If S-CU doesn’t provide a reference (anyway optional), a recipient C-CU can independently decide to generate its own reference and send it back to S-CU.

- S-CU can decide which one to adopt (including its own configuration) as a reference, and the chosen reference can be shared with C-CU(s) for updating the candidate cell configurations accordingly (as already agreed).

## Semi-Persistent CSI-RS support

For both inter-CU and intra-CU cases:

For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell.

For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

Proposal 10: Adopt new dedicated class-1 procedures over Xn and F1 to support the serving cell’s activation/deactivation decision of CSI-RS transmissions in a candidate cell, which may involve up to 3 round-trip signallings across the serving DU, serving CU, candidate CU, and candidate DU, and require inter-dependent class-1 request/response from end to end.

Proposal 11: For SP CSI-RS, also consider a mechanism for a candidate cell’s DU to notify the serving DU if the CSI-RS transmissions requested from the serving DU are (or has to be) stopped.

## Others

Proposal 12: Do not consider additional TA handlings specified in Rel-18 across CUs (a known TA value and UE-based TA measurement, both of which were specified applicable within a single CU), unless their usage across CUs is validated.

# Reference

[1] R3-250807, “Summary of offline discussion on MobilityEnh_LTM”, China Telecom (moderator)

[2] R3-251558, ”(BL CR to 38.423) Xn support for inter-CU LTM”, Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno

[3] TS 38.473, NG-RAN; F1 Application Protocol (F1AP), v18.5.0

[4] TS 37.340, E-UTRA and NR; Multi-connectivity; Overall Description; Stage 2, v18.5.0

[5] R3-247112, “Support of Multiple Reference Configurations”, Nokia. KDDI, LG Electronics, ZTE, Rakuten, Fujitsu

[6] R3-247762, “Response to R3-247111 and R3-247112”, LG Electronics Inc, ZTE, Fujitsu, KDDI, KT Corp., ITRI, Rakuten, ETRI, SK Telecom, LG Uplus

[7] R3-247824, “Summary of offline discussion on MobilityEnh”, China Telecom (moderator)

[8] R3-235019/R4-2314455, Reply LS on beam application time and UE based TA measurement for LTM, RAN4

# Annex A (RAN3 agreement for Inter-CU LTM)

## RAN3-127 (Athens, February 2025)

For both inter-CU and intra-CU cases:

For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell.

For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

The candidate gNB-CU responds the full SSB Time/Frequency Configuration (in SSB Information IE) of candidate cells to the source gNB-CU in the Handover Request ACK message.

The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration.

The format of the new introduced IE is same with the gNB-DU ID.

WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message.

The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message.

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

RAN3 agree the following scenarios to support LTM with NR-DC:

1.SN initiated inter-CU SCG LTM without MCG changes (high priority)

2.Inter-CU MCG LTM without SN release

3.Inter-CU MCG LTM with SN release

4.Inter-CU MCG LTM with SN addition

For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

The MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

## RAN3-126 (Orlando, November 2024)

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB.

Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?

RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution.

## RAN3-125bis (Hefei, October 2024)

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

The LTM Configuration IDs are allocated by the source CU.

WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

## RAN3-125 (Maastricht, August 2024)

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

## RAN3-124 (Fukuoka, May 2024)

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

## RAN3-123bis (Changsha, April 2024)

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

# Annex B (RAN2 agreement for Inter-CU LTM)

## RAN2-129 (Athens, February 2025)

Agreements on RRC issues:

- For inter-CU SCG LTM, introducing the SK counter list in LTM-ConfigSCG-r19, each list associated with one ltm-NoSecurityChangeID.

- Reuse the selectedSK-Counter-r18 to indicate the selected SK counter upon the execution of inter-CU SCG LTM.

- The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field.

- UE does not combine reference configurations from ltm-Config and ltm-ConfigSCG.

- FFS whether only one indication is needed to indicate LTM configuration as complete configuration.

- The existing field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an MCG LTM execution. Can be revisited if there is any issue.

- The release of the LTM configuration (MCG or SCG) follows the Rel-18 principles.

- A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured.

- The LTM-Candidate IE is re-used to model an CLTM candidate cell configuration.

- Re-use existing L1 events for CLTM.

- For field ltm3-Offset-r19, the granularity is 0.5 dB.

- Add extension mark “…” to LTM-ConfigSCG-r19, adopt the principle that fields only applicable to inter-CU SCG change should be defined in the root of LTM-ConfigSCG-r19, not LTM-Config-r18.

Agreements on MAC issues:

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

- If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

- If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

- The CLTM TAT is per candidate configuration.

- FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

- Absolute TA value is included in the MAC CE for CLTM early TA acquisition.

- The candidate configuration ID is included in the MAC CE for CLTM early TA acquisition. And one configuration ID for one TA in one MAC CE.

- RAN2 assumes the CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE and in the candidate cell TCI state Activation/Deactivation MAC CE, pending RAN1 confirmation.

- The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'.

- The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.

- The max number of N (included in the MR MAC CE) is the total number of all candidate beams without counting current serving beam.

- There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on, i.e. not limited to SpCell.

- Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader.

Agreements on inter-CU LTM:

- Using the term “LTM” to represent “intra-CU LTM” and “inter-CU LTM” in MAC specification, and introduce separate term for “conditional intra-CU LTM” in the specification(s).

- Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

- Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM.

- For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2.

- For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

- How to support shared RACH resources for inter-CU LTM is left to RAN3.

## RAN2-128 (Orlando, November 2024)

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

7.	In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

8.	RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

9.	RAN2 to support intra-CU MCG LTM with SCG configuration.

10.	It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

11.	RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

## RAN2-127bis (Hefei, October 2024)

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

Agreements related to Inter-SN LTM

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

## RAN2-127 (Maastricht, August 2024)

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

## RAN2-126 (Fukuoka, May 2024)

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

## RAN2-125bis (Changsha, April 2024)

Agreements on scenarios:

- RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios.

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

- UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

Agreements on latency analysis:

- Mobility latency analysis of rel-18 intra-CU LTM is reused for Rel-19 inter-CU LTM.

Agreements on early sync phase:

- Early DL and UL sync is also supported for inter-CU LTM.  Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.