3GPP TSG-RAN WG3 Meeting #127-bis	R3-251743

Wuhan, China, April 7th – 11st, 2025

Agenda item:	13.3

Source: 	LG Electronics Inc.

Title: 	(TP for TS 38.401 and TS 38.473) Discussions on RAN3 impacts from Conditional Intra-CU LTM

Document for:	Other

# Introduction

For Rel-19 Conditional Intra-CU LTM, RAN3-127 (Athens) has progressed as follows:

## RAN3-127 (Athens, February 2025) The CU decides to configure C-LTM. The CU determines which type of C-LTM to initiate: L1 event-triggered C-LTM or L3 event-triggered C-LTM. Reuse the current LTM preparation procedure (i.e. UE Context Setup/Modification procedure) for DU to generate the C-LTM L1 execution condition. UE Context Setup procedure for the candidate DU to generate the subsequent C-LTM L1 execution condition. UE Context Modification procedure for the source DU to generate the C-LTM L1 execution condition. Indicator for C-LTM shall be added in the UE CONTEXT SETUP/MODIFICATION REQUEST message. FFS for the detail. For L1 measurement based C-LTM, to support Early Sync, the existing F1: DU-CU/ CU-DU TA Information Transfer is reused.

RAN3 also touched upon baseline CU-DU interactions for Intra-CU C-LTM [1]. In the meantime, RAN2-129 (Athens) also progressed as follows:

RAN2-129 (Athens) agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

Considering the progresses so far, we further discuss the remaining aspects and impacts our existing RAN3 Intra-CU LTM designs for C-LTM.

# Discussion

## Harmonization with traditional LTM

The CU decides to configure C-LTM.

The CU determines which type of C-LTM to initiate: L1 event-triggered C-LTM or L3 event-triggered C-LTM.

Reuse the current LTM preparation procedure (i.e. UE Context Setup/Modification procedure) for DU to generate the C-LTM L1 execution condition.

UE Context Setup procedure for the candidate DU to generate the subsequent C-LTM L1 execution condition.

UE Context Modification procedure for the source DU to generate the C-LTM L1 execution condition.

Indicator for C-LTM shall be added in the UE CONTEXT SETUP/MODIFICATION REQUEST message. FFS for the detail.

For L1 measurement based C-LTM, to support Early Sync, the existing F1: DU-CU/ CU-DU TA Information Transfer is reused.

As seen from the agreements above and the agreed TP [1], RAN3 already assumed to leverage the existing F1AP signaling framework designed at Rel-18 LTM as the foundation for Rel-19 C-LTM. Given this situation, we believe there is no reason not to harmonize C-LTM with the existing LTM and allow flexibility to configure a UE such that traditional LTM is applied for one candidate cell, while C-LTM operates for another candidate cell, all through a single RRC configuration upfront.

Moreover, RAN2 has already agreed on the use of both L1-based and L3-based execution conditions for LTM. The L1 execution condition is evaluated using certain L1 measurement results, measured from the (common) CSI resources configured for LTM. Similar to CSI report configuration, it is the DU’s responsibility to generate L1 execution condition(s).

L3 execution condition (generated by CU) is also possible. If L3-based condition is prepared for a candidate cell, L1-based condition for the same cell may no longer be necessary. Or, both can be configured and evaluated simultaneously, which appears feasible according to RAN2’s progress so far.

In Rel-18, LTM F1AP signaling was designed so that DU (always) generates and provides the cell-specific CSI report configuration whenever it receives the (common) CSI resource configuration from CU. As a result, for C-LTM, CU needed to explicitly instruct a DU to generate L1 execution conditions instead of CSI report configuration when providing the CSI resource configuration, for which RAN3 already agreed to do so (new “C-LTM” indicator). On the other hand, if CU decides to use only L3 execution condition for a certain cell, then L1 execution condition from the corresponding DU is unnecessary. In this case, the CSI resource configuration (necessary for building L1 execution conditions) can be simply not provided to the DU during preparation phase, ensuring that DU neither generates CSI report configuration nor L1 execution conditions.

This leads to the conclusion that, from DU point of view, what is different to the traditional LTM (non-Conditional LTM) during the preparation phase is that, essentially, CSI “report” configuration generated by DU is replaced by L1 execution conditions when CSI resource configuration is provided, for which the agreed “C-LTM” indicator can control per candidate cell basis.

Proposal 1: Allow flexibility for CU to prepare both traditional LTM and C-LTM for different candidate cells within the same UE (i.e. configure traditional LTM for some candidate cells while enabling C-LTM for others) by

- When the agreed “C-LTM” is indicated for a candidate cell, this is for C-LTM, and L1 execution conditions are requested from DU if CSI resource configuration is provided.

- When the agreed “C-LTM” is not indicated for a candidate cell, this is for traditional LTM, and CSI report configuration is requested from DU if CSI resource configuration is provided.

## Impacts from RAN2 RRC design

RAN2-128 agreed the following:

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

The first bullet needs a special attention as it requires the C-LTM configuration of each candidate cell include something generated by the other cell, i.e. initial source cell.

If C-LTM is configured for a UE, an initial cell switch from the UE’s current serving cell can be based on C-LTM. In this case, the first bullet means that the execution conditions evaluated at the current serving cell are NOT configured via the current serving cell’s configuration (who generated those initial execution conditions). Instead, they are configured via each candidate cell’s configuration (i.e. LTM-Candidate-r18) separately [2]:

We provide the Figure 2 as an illustration: In Figure 2, there are three CLTM configurations. Each CLTM configuration includes the initial execution condition (blue font) and a list of subsequent execution conditions (green font). Taking CLTM configuration 1 as an example, when the candidate cell 1 fulfils the Condition 1, UE performs CLTM and cell 1 becomes the serving cell. Then UE evaluates the Condition 1-2 for the candidate cell 2 and the Condition 1-3 for the candidate cell 3 for subsequent conditional LTM. Figure 2. RRC structure for the configuration of execution conditions A potential ASN.1 structure is provided in Annex.

This means that, if L1-based C-LTM is decided at the initial source cell, those L1 execution conditions generated by S-DU needs to be provided to the CU separately for each candidate cell, so that CU can compile each initial execution condition into each candidate cell’s LTM configuration.

In case L3-based C-LTM is decided at the initial source cell, we see no F1 impact as it is the CU who generates initial L3 execution conditions and compiles final LTM configurations of candidate cells.

Proposal 2: L1-based C-LTM from the initial source cell (i.e. current serving cell) requires the S-DU to supply the generated L1 execution conditions for each candidate cell separately to the CU, so that CU can compile each condition into each candidate cell’s LTM configuration accordingly.

Proposal 3: For that, enhance the F1AP UE CONTEXT MODIFICATION RESPONSE message > LTM Configuration IE to include a list of execution conditions generated from the current serving cell for each candidate cell.

The second bullet does not require special attentions, as it is typical for a cell’s LTM configuration to include conditions that it generates for the UE. However, if RAN2's RRC structure places the execution conditions for subsequent C-LTM (i.e., those that must be evaluated when a candidate cell becomes the serving cell) separately under the LTM-Candidate-r18 of that candidate cell, a special attention is required for L1  execution conditions, because the candidate cell’s DU must also provide these conditions explicitly to the CU (not hidden under the candidate cell’s CellGroupConfig).

Currently, there is no requirement for a candidate cell’s DU to supply those subsequent C-LTM L1 execution conditions for other candidate cells individually. The DU just needs to provide the whole for inclusion into its LTM-Candidate-r18. However, to maintain consistency in the F1AP signaling structure, we think it is better to harmonize the stage-3 designs of delivering initial and subsequent C-LTM L1 execution conditions to CU. In fact, Proposal 3, which propose to enhance the F1AP UE CONTEXT MODIFICATION RESPONSE message to carry L1 execution conditions from the initial source cell, can also be applied for carrying subsequent C-LTM L1 execution conditions from a candidate cell. Then, the gap is on the F1AP UE CONTEXT SETUP RESPONSE message for which we can enhance similarly as in Proposal 3.

Proposal 4: To ensure F1AP signalling consistency in delivering L1 execution conditions for subsequent C-LTM, also enhance the F1AP UE CONTEXT SETUP RESPONSE message > LTM Configuration IE to include a list of execution conditions generated by a candidate cell for other candidate cells.

## Impacts from C-LTM early synchronization

RAN2-128 (Orlando) agreements on C-LTM:

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

From the above RAN2-128 agreements, we can see that the traditional LTM mechanisms, where early UL synchronization is triggered by PDCCH and early DL synchronization by MAC CE, are re-used for C-LTM. As in traditional LTM, these triggers normally come from a DU serving the UE. However, one key difference in C-LTM is that CU can also trigger early UL synchronization based on L3 measurement reports. This requires new F1AP signaling, as CU needs to instruct the serving DU to initiate PDCCH-ordered early RACH for the UE.

Moreover, in C-LTM, enabling RACH-less LTM requires that a TA value obtained by a candidate cell and forwarded to the serving cell be configured in the UE before it executes the cell switch toward that candidate cell. This is different from the traditional NW-triggered LTM, where a TA value is configured in the UE only when NW instructs a cell switch (i.e., via the MAC CE Cell Switch Command). In contrast, C-LTM requires that only a TA value be pre-configured in the UE before the UE executes cell switching. To support this, RAN2 has agreed on a new MAC CE, and for this to work, no further F1AP impact is foreseen:

For L1 measurement based C-LTM, to support Early Sync, the existing F1: DU-CU/ CU-DU TA Information Transfer is reused.

It is still unclear whether early DL synchronization (TCI state activation/deactivation) can be triggered by CU based on L3 measurement reports for C-LTM. But, RAN3 can address once RAN2 makes more progress. In the meantime, our focus can remain on the CU to DU F1AP signaling for triggering early TA acquisition.

Proposal 5: For C-LTM, define a new class-2 F1AP signaling from CU to command the serving DU to trigger early TA acquisition.

Proposal 6: For C-LTM, FFS whether early DL synchronization can also be triggered from CU, pending RAN2 progress.

## Impacts from C-LTM execution

The main impact of UE’s autonomous execution is when a C-LTM execution happens across DUs.

In C-LTM, the existing Cell Switch Notification mechanism cannot be applicable as it is, because the serving DU does not know when the UE will execute a cell switch. The main purpose of Cell Switch Notification was to support DG (dynamic grant) based RACH-less procedure, by informing the target cell that a cell switch has been triggered for the UE and on which beams, so that it can start DG scheduling toward those beams accordingly. In C-LTM, unless RAN2 defines a new mechanism to “notify” the serving DU of its cell switch and the selected target cell right after execution (a.k.a. “bye” signal from the UE), the serving DU remains unaware of the UE’s execution. Otherwise, the source DU would have to abuse the existing Cell Switch Notification mechanism toward any plausible target cells to make them initiate their DG scheduling in advance, “just in case” the UE may execute and switch to one of them (using DG).

In fact, RAN2-129 discussed this matter and agreed not to support DG-based RACH-less C-LTM, due to its complicated nature in C-LTM and also considering that its main objective is to enhance robustness.

RAN2-129 (Athens) agreements on C-LTM:

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

This was expected, as Cell Switch Notification was originally designed for NW-triggered mobility, where the NW entity initiating a cell switching informs the corresponding target. On the other hand, C-LTM follows a fundamentally different philosophy where the UE (not NW) autonomously decides when/which cell to switch.

In the last RAN3 meeting, we intentionally skipped discussing this aspect due to pending discussions in RAN2 and left this part FFS in the agreed TP [1]. Given RAN2 concluded not to support DG-based RACH-less C-LTM, there is no reason to keep this FFS.

Instead, as illustrated in Figure, 1, the previous serving DU, which is unaware that an Inter-DU C-LTM has been executed by the UE, requires additional notification from the CU to properly stop data transmission for the UE, revert the previous serving cell to a prepared state (if it remains one of the configured LTM candidate cells), and inform any DL packets not successfully delivered to the UE for retransmissions by the CU:

Figure 1: Inter-DU C-LTM execution procedure

Such additional Step 5-7 handling (instead of the existing Cell Switch Notification) is not new, which has been well-documented in our RAN3 specifications for conditional mobilities of previous releases [3]:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### 8.2.1.3	Inter-gNB-DU Conditional Handover or Conditional PSCell Change or subsequent CPAC

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for conditional handover or conditional PSCell change or subsequent CPAC. Figure 8.2.1.3-1 shows the inter-gNB-DU conditional mobility procedure for intra-NR.

Figure 8.2.1.3-1: Inter-gNB-DU Conditional Handover or Conditional PSCell Change or subsequent CPAC for intra-NR

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

13.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU and indicates to stop the data transmission for the UE. The source gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the target gNB-DU.

NOTE 1:	The step 13 may happen before step 12, as soon as the gNB-CU knows which cell the UE has successfully accessed.

NOTE 2:	The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate target gNB-DUs, if any, to cancel conditional handover or conditional PSCell change for the UE.

14.	The source gNB-DU responds to the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Proposal 7: For C-LTM, do not consider the existing Cell Switch Notification mechanism (designed for NW-triggered mobility) any longer. RAN2-129 agreed not to support DG (dynamic grant) based RACH-less procedure for C-LTM.

Proposal 8: Instead, for C-LTM, after CU becomes aware of the UE’s Inter-DU cell switch execution through ACCESS SUCCESS, informs the previous serving DU, similarly following TS 38.401 Figure 8.2.1.3-1 Steps 13-14. No stage-3 F1AP impact is foreseen.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

## Harmonization with traditional LTM

Proposal 1: Allow flexibility for CU to prepare both traditional LTM and C-LTM for different candidate cells within the same UE (i.e. configure traditional LTM for some candidate cells while enabling C-LTM for others) by

- When the agreed “C-LTM” is indicated for a candidate cell, this is for C-LTM, and L1 execution conditions are requested from DU if CSI resource configuration is provided.

- When the agreed “C-LTM” is not indicated for a candidate cell, this is for traditional LTM, and CSI report configuration is requested from DU if CSI resource configuration is provided.

## Impacts from RAN2 RRC design

Proposal 2: L1-based C-LTM from the initial source cell (i.e. current serving cell) requires the S-DU to supply the generated L1 execution conditions for each candidate cell separately to the CU, so that CU can compile each condition into each candidate cell’s LTM configuration accordingly.

Proposal 3: For that, enhance the F1AP UE CONTEXT MODIFICATION RESPONSE message > LTM Configuration IE to include a list of execution conditions generated from the current serving cell for each candidate cell.

Proposal 4: To ensure F1AP signalling consistency in delivering L1 execution conditions for subsequent C-LTM, also enhance the F1AP UE CONTEXT SETUP RESPONSE message > LTM Configuration IE to include a list of execution conditions generated by a candidate cell for other candidate cells.

## Impacts from C-LTM early synchronization

Proposal 5: For C-LTM, define a new class-2 F1AP signaling from CU to command the serving DU to trigger early TA acquisition.

Proposal 6: For C-LTM, FFS whether early DL synchronization can also be triggered from CU, pending RAN2 progress.

## Impacts from C-LTM execution

Proposal 7: For C-LTM, do not consider the existing Cell Switch Notification mechanism (designed for NW-triggered mobility) any longer. RAN2-129 agreed not to support DG (dynamic grant) based RACH-less procedure for C-LTM.

Proposal 8: Instead, for C-LTM, after CU becomes aware of the UE’s Inter-DU cell switch execution through ACCESS SUCCESS, informs the previous serving DU, similarly following TS 38.401 Figure 8.2.1.3-1 Steps 13-14. No stage-3 F1AP impact is foreseen.

The corresponding 38.401 and F1AP TPs are provided in Section 5 and 6, respectively.

# Reference

[1] R3-250915, “TP to TS 38.401 on conditional intra-CU LTM”, ZTE Corporation, NEC, China Telecom, Samsung, Nokia, Google, Huawei, Ericsson, LG Electronics

[2] R2-2410022, “Discussion on conditional LTM”, Xiaomi

[3] TS 38.401, NG-RAN; Architecture description (Release 18)

# TP for TS 38.401 BL CR (R3-251557)

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.2.1.x Conditional intra-CU LTM (Intra-gNB-DU)

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for Conditional LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU Conditional LTM procedure for intra-NR.

Figure 8.2.1.x-1: Conditional intra-CU LTM (Intra-gNB-DU)

1.	The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing measurements of neighbouring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate Conditional LTM configuration.

3.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU for each candidate cell, containing one candidate cell ID. The gNB-CU may provide the LTM configuration ID mapping list to the gNB-DU. The gNB-CU may request PRACH resources from the gNB-DU. The gNB-CU may request the gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the gNB-DU. The gNB-CU may inform the gNB-DU about intra-DU L2 reset configuration. The gNB-CU may request the gNB-DU to generate the L1-based execution conditions for subsequent Conditional LTM by indicating that the request is for Conditional LTM and providing the CSI resource configuration.

4.	If the gNB-DU accepts the request of Conditional LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations for the accepted candidate cell. If the L1-based execution conditions are requested, the gNB-DU also provides a list of execution conditions generated for other candidate cells.

NOTE 1:	Steps 3 and 4 may be initiated multiple times for Conditional LTM candidate cell preparation of multiple cells including the source cell.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU which may include the LTM configuration ID mapping list. The gNB-CU may request the source gNB-DU to generate the L1-based execution conditions for initial Conditional LTM by indicating that the request is for Conditional LTM and providing the CSI resource configuration. The gNB-CU may inform the gNB-DU about intra-DU L2 reset configuration.

6.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration. If the L1-based execution conditions are requested, the source gNB-DU also provides a list of execution conditions generated for the candidate cells

NOTE 2:	In case of subsequent Conditional LTM, the CU-initiated UE Context Modification procedure may be invoked per each candidate cell to transfer to the gNB-DU the updated CSI resource configuration.

7.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the Conditional LTM configuration.

8.	The gNB-DU forwards the received RRCReconfiguration message to the UE.

9.	The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

10.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

11.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2]. The gNB-CU may request the gNB-DU to trigger early TA acquisition for a specific candidate cell.

12.	The gNB-DU sends the MAC CE to the UE.

13.	The execution condition(s) to trigger initiation of Conditional LTM is fulfilled in the UE.

14. Conditional LTM is executed. The gNB-DU detects the UE access as specified in TS 38.300 [2]. The gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID. The gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source cell, are sent from the gNB-CU to the gNB-DU.

15.	The UE sends an RRCReconfigurationComplete message to the gNB-DU.

16.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

17.	The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

18.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

#### 8.2.1.y Conditional intra-CU LTM (Inter-gNB-DU)

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for Conditional LTM. Figure 8.2.1.y-1 shows the inter-gNB-DU Conditional LTM procedure for intra-NR.

Figure 8.2.1.y-1: Conditional intra-CU LTM (Inter-gNB-DU)

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate Conditional LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one candidate cell ID. The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to generate the L1-based execution conditions for subsequent Conditional LTM by indicating that the request is for Conditional LTM and providing the CSI resource configuration..

4.	If the candidate gNB-DU accepts the request of Conditional LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell. If the L1-based execution conditions are requested, the candidate gNB-DU also provides a list of execution conditions generated for other candidate cells.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU Conditional LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration ID mapping list for the accepted target candidate cell(s). The gNB-CU may request the source gNB-DU to generate the L1-based execution conditions for initial Conditional LTM by indicating that the request is for Conditional LTM and providing the CSI resource configuration.. The gNB-CU may inform the source gNB-DU about intra-DU L2 reset configuration.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration of the source cell. If the L1-based execution conditions are requested, the source gNB-DU also provides a list of execution conditions generated for the candidate cells.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message for each candidate cell accepted in the candidate gNB-DU(s), containing the information for subsequent conditional LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may inform the candidate gNB-DU(s) about intra-DU L2 reset configuration. The gNB-CU may request the candidate gNB-DU(s) to generate the L1-based execution conditions for subsequent Conditional LTM.

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration of the requested candidate cell. If the L1-based execution conditions are requested, the candidate gNB-DU also provides a list of execution conditions generated for other candidate cells

NOTE 2: Step 7 may also be triggered after step 18 for subsequent Conditional LTM.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the Conditional LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2]. The gNB-CU may request the source gNB-DU to trigger early TA acquisition for a specific candidate cell.

14. The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the gNB-CU, which includes the TA values, and the associated PRACH resource information.

15.	The gNB-CU forwards the TA value and the associated PRACH resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

Editor’s Note: Details are FFS on step 14 and 15 on how to handle the TAT.

16.	The source gNB-DU sends the MAC CE to the UE.

17. The execution condition(s) to trigger initiation of Conditional LTM is fulfilled in the UE.

18. Conditional LTM is executed. The target gNB-DU detects the UE access as specified in TS 38.300 [2]. The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID. The target gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

20.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

21.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU to inform that the UE executed an inter-DU Conditional LTM and indicate to stop the data transmission for the UE. The source gNB-DU revert the previous serving cell to a prepared state, if it remains one of the configured LTM candidate cells, and sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the target gNB-DU.

NOTE 3:	The step 21 may happen before step 20, as soon as the gNB-CU knows which cell the UE has successfully accessed.

22.	The source gNB-DU responds to the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

23.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

24.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# TP for TS 38.473 BL CR (R3-251559)

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 8.3	UE Context Management procedures

### 8.3.1	UE Context Setup

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.1.2	Successful Operation

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the LTM Indicator IE set to "true" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, provide the lower layer configuration in the Reference Configuration Information IE in the LTM ConfigurationIE in the UE CONTEXT SETUP RESPONSE message for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration(s) in the CellGroupConfig IE for the requested LTM candidate cell.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, include the Early Sync Information IE of the accepted candidate cell for early TA acquisition (early UL synchronisation), in the UE CONTEXT SETUP RESPONSE message. If the Early UL Sync Configuration IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information from the accepted candidate cell in the gNB-DU. If the Early UL Sync Configuration for SUL IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information for SUL from the accepted candidate cell in the gNB-DU.

If the LTM Configuration IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the candidate gNB-DU.

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the LTM Indicator IE set to "C-LTM" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns Conditional LTM for the included SpCell ID IE. If the CSI Resource Configuration is also contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the Conditional LTM execution conditions from the requested LTM candidate cell toward the LTM candidate cells indicated by the LTM Configuration ID Mapping List IE and provide the Conditional LTM Execution Condition List IE in the UE CONTEXT SETUP RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.4	UE Context Modification (gNB-CU initiated)

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.4.2	Successful Operation

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the LTM Indicator IE set to "true" is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT MODIFICATION RESPONSE message.

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include the Reference Configuration Information IE in the LTM Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message to provide lower layer configuration for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message and the SpCell ID IE is also included, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the requested LTM candidate cell identified by the SpCell ID IE.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message while the SpCell ID IE is absent, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the serving cell.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include Early Sync Information IE of the accepted candidate cell for early TA acquisition (early UL synchronisation) in the UE CONTEXT MODIFICATION RESPONSE message. If the Early UL Sync Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information from the accepted candidate cell in the gNB-DU. If the Early UL Sync Configuration for SUL IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information for SUL from the accepted candidate cell in the gNB-DU.

If the Early Sync Candidate Cell Information List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the UE Based TA Measurement Configuration IE is contained in the Early Sync Candidate Cell Information List IE for some candidate cell, the gNB-DU shall, if supported, take them into account for UE based TA measurement during LTM cell switch as specified in TS 38.331 [8].

If the Early Sync Serving Cell Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the UE Based TA Measurement Configuration IE is contained in the Early Sync Serving Cell Information IE, the gNB-DU shall, if supported, take it into account for UE based TA measurement during LTM cell switch as specified in TS 38.331 [8].

If the LTM CFRA Resource Config List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [16].

If the LTM Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the gNB-DU.

If the LTM Cells to be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the configured candidate cells in the list.

If the LTM Reset Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account for L2 reset (i.e., RLC re-establishment) during an intra-DU LTM cell switch as specified in TS38.331 [8].

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the LTM Indicator IE set to "C-LTM" is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns Conditional LTM. If the CSI Resource Configuration IE is also contained in the LTM Information Modify IE and the SpCell ID IE is also included, the gNB-DU shall, if supported, use it to generate the Conditional LTM execution conditions from the requested LTM candidate cell toward the LTM candidate cells indicated by the LTM Configuration ID Mapping List IE and provide the Conditional LTM Execution Condition List IE in the UE CONTEXT MODIFICATION RESPONSE message. If the CSI Resource Configuration IE is also contained in the LTM Information Modify IE while the SpCell ID IE is absent, the gNB-DU shall, if supported, use it to generate the Conditional LTM execution conditions from the serving cell toward the LTM candidate cells indicated by the LTM Configuration ID Mapping List IE and provide the Conditional LTM Execution Condition List IE in the UE CONTEXT MODIFICATION RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | reject |
| gNB-CU UE F1AP ID |  | M |  |  |  | 9.3.1.4 |  |  |  | YES |  | reject |
| gNB-DU UE F1AP ID |  | O |  |  |  | 9.3.1.5 |  |  |  | YES |  | ignore |
| SpCell ID |  | M |  |  |  | NR CGI 9.3.1.12 |  | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. |  | YES |  | reject |
| ServCellIndex |  | M |  |  |  | INTEGER (0..31,...) |  |  |  | YES |  | reject |
| SpCell UL Configured |  | O |  |  |  | Cell UL Configured 9.3.1.33 |  |  |  | YES |  | ignore |
| CU to DU RRC Information |  | M |  |  |  | 9.3.1.25 |  |  |  | YES |  | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |  |  |  |  |  |  |
| LTM Information Setup |  |  | 0..1 |  |  |  |  |  | YES |  | reject |  |
| >LTM Indicator | M |  |  |  | ENUMERATED (true, …, C-LTM) |  |  |  | - |  |  |  |
| >Reference Configuration | O |  |  |  | 9.3.1.292 |  |  |  | - |  |  |  |
| >CSI Resource Configuration | O |  |  |  | 9.3.1.330 |  |  |  | - |  |  |  |
| LTM Configuration ID Mapping List | O |  |  |  | 9.3.1.294 |  |  |  | YES |  | reject |  |
| Early Sync Information Request |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >Request for RACH Configuration | M |  |  |  | ENUMERATED (true, …) |  |  |  | - |  |  |  |
| >LTM gNB-DUs List |  |  | 1 |  |  |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). |  | YES |  | reject |  |
| >>LTM gNB-DUs Item IEs |  |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |  | - |  |  |  |
| >>>LTM gNB-DU ID | M |  |  |  | gNB-DU ID 9.3.1.9 |  |  |  | - |  |  |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell. | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >Conditional LTM Execution Condition List | O |  | 9.3.1.XXX | FFS | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | reject |  |
| gNB-CU UE F1AP ID |  | M |  |  |  | 9.3.1.4 |  |  |  | YES |  | reject |  |
| gNB-DU UE F1AP ID |  | M |  |  |  | 9.3.1.5 |  |  |  | YES |  | reject |  |
| SpCell ID |  | O |  |  |  | NR CGI 9.3.1.12 |  | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. |  | YES |  | ignore |  |
| ServCellIndex |  | O |  |  |  | INTEGER (0..31, ...) |  |  |  | YES |  | reject |  |
| SpCell UL Configured |  | O |  |  |  | Cell UL Configured 9.3.1.33 |  |  |  | YES |  | ignore |  |
| DRX Cycle |  | O |  |  |  | 9.3.1.24 |  |  |  | YES |  | ignore |  |
| CU to DU RRC Information |  | O |  |  |  | 9.3.1.25 |  |  |  | YES |  | reject |  |
| Transmission Action Indicator |  | O |  |  |  | 9.3.1.11 |  |  |  | YES |  | ignore |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LTM Information Modify |  |  | 0..1 |  |  |  |  |  | YES |  | reject |  |
| >LTM Indicator | M |  |  |  | ENUMERATED (true, …, C-LTM) |  |  |  | - |  |  |  |
| >Reference Configuration | O |  |  |  | 9.3.1.292 |  |  |  | - |  |  |  |
| >CSI Resource Configuration | O |  |  |  | 9.3.1.330 |  |  |  | - |  |  |  |
| LTM CFRA Resource Config List |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >LTM CFRA Resource Config Item IEs |  |  | 1 .. <maxnoofLTMCells> |  |  |  |  |  | EACH |  | ignore |  |
| >>Cell ID | M |  |  |  | NR CGI 9.3.1.12 |  |  |  | - |  |  |  |
| >>LTM CFRA Resource Configuration | O |  |  |  | OCTET STRING |  | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. |  | - |  |  |  |
| >>LTM CFRA Resource Configuration for SUL | O |  |  |  | OCTET STRING |  | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. |  | - |  |  |  |
| LTM Configuration ID Mapping List | O |  |  |  | 9.3.1.294 |  |  |  | YES |  | reject |  |
| Early Sync Information Request |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >Request for RACH Configuration | M |  |  |  | ENUMERATED (true, …) |  |  |  | - |  |  |  |
| >LTM gNB-DUs ID List |  |  | 1 |  |  |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). |  | YES |  | reject |  |
| >>LTM gNB-DUs Item IEs |  |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |  | - |  |  |  |
| >>>LTM gNB-DU ID | M |  |  |  | gNB-DU ID 9.3.1.9 |  |  |  | - |  |  |  |
| Early Sync Candidate Cell Information List |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >Early Sync Candidate Cell Information Item IEs |  |  | 1 .. <maxnoofLTMCells> |  |  |  |  |  | EACH |  | ignore |  |
| >>Cell ID | M |  |  |  | NR CGI 9.3.1.12 |  |  |  | - |  |  |  |
| >>TCI States Configurations List | O |  |  |  | OCTET STRING |  | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. |  | - |  |  |  |
| >>Early UL Sync Configuration | O |  |  |  | 9.3.1.328 |  |  |  | - |  |  |  |
| >>Early UL Sync Configuration for SUL | O |  |  |  | Early UL Sync Configuration 9.3.1.328 |  | This IE applies for SUL carrier. |  | - |  |  |  |
| >>TA Assistance Information | O |  |  |  | ENUMERATED (zero, …) |  | The value "zero" corresponds to TA value of the cell being equal to zero. |  | - |  |  |  |
| >>UE Based TA Measurement Configuration | O |  |  |  | OCTET STRING |  | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. |  | - |  |  |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  |  |  | 9.3.1.138 |  | This IE applies to early TA acquisition. |  | YES |  | ignore |  |
| Early Sync Serving Cell Information |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >UE Based TA Measurement Configuration | O |  |  |  | OCTET STRING |  | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. |  | - |  |  |  |
| LTM Cells To Be Released List | O |  |  |  | 9.3.1.291 |  |  |  | YES |  | reject |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >Conditional LTM Execution Condition List | O |  | 9.3.1.XXX | FFS | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.3.1	Radio Network Layer Related IEs

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.3.1.XXX		Conditional LTM Execution Condition List

This IE indicates the list of conditional LTM execution conditions to be used by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Conditional LTM Execution Condition Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >Execution Condition | M |  | OCTET STRING (FFS) | Up to RAN2 | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# Annex A (RAN3 agreement for Conditional Intra-CU LTM)

## RAN3-127 (Athens, February 2025)

The CU decides to configure C-LTM.

The CU determines which type of C-LTM to initiate: L1 event-triggered C-LTM or L3 event-triggered C-LTM.

Reuse the current LTM preparation procedure (i.e. UE Context Setup/Modification procedure) for DU to generate the C-LTM L1 execution condition.

UE Context Setup procedure for the candidate DU to generate the subsequent C-LTM L1 execution condition.

UE Context Modification procedure for the source DU to generate the C-LTM L1 execution condition.

Indicator for C-LTM shall be added in the UE CONTEXT SETUP/MODIFICATION REQUEST message. FFS for the detail.

For L1 measurement based C-LTM, to support Early Sync, the existing F1: DU-CU/ CU-DU TA Information Transfer is reused.

# Annex B (RAN2 agreement for Conditional Intra-CU LTM)

## RAN2-129 (Athens, February 2025)

RAN2-129 (Athens) agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

## RAN2-128 (Orlando, November 2024)

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

## RAN2-127bis (Hefei, October 2024)

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.