3GPP TSG-RAN WG3 Meeting #127-bis	R3-251760

Wuhan, China, Apr 7th - Apr 11th, 2025

Agenda item:	13.3

Source:	Qualcomm Incorporated

Title:	Signalling enhancements for Conditional Intra-CU LTM

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #127 [1]), discussion on conditional Intra-CU LTM was initiated. The following set of basic agreements were made.

RAN3 #127 agreements:

The CU decides to configure C-LTM.

The CU determines which type of C-LTM to initiate: L1 event-triggered C-LTM or L3 event-triggered C-LTM.

Reuse the current LTM preparation procedure (i.e. UE Context Setup/Modification procedure) for DU to generate the C-LTM L1 execution condition.

UE Context Setup procedure for the candidate DU to generate the subsequent C-LTM L1 execution condition.

UE Context Modification procedure for the source DU to generate the C-LTM L1 execution condition.

Indicator for C-LTM shall be added in the UE CONTEXT SETUP/MODIFICATION REQUEST message. FFS for the detail.

For L1 measurement based C-LTM, to support Early Sync, the existing F1: DU-CU/ CU-DU TA Information Transfer is reused.

In this contribution, we continue the discussion on the signalling aspects of the LTM preparation phase of intra-CU conditional LTM (C-LTM).

# 2 Signalling aspects of the LTM preparation phase of intra-CU conditional LTM (C-LTM)

We discuss the two cases when the gNB-CU decides to configure the UE with L3 execution conditions and when it decides to configure the UE with L1 execution conditions.

Figure 1: Signalling flow for Intra-CU C-LTM with L3 execution conditions configured

Proposal 1. The gNB-CU initiates intra-CU C-LTM preparation, decides to configure L3 execution conditions, and transmits to an underlying candidate gNB-DU UE Context Setup Request including a suggested candidate cell for LTM preparation. The UE Context Setup Request message includes:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication.

If an LTM candidate cell belongs to the source gNB-DU, the gNB-CU also transmits to the source gNB-DU the UE Context Modification Request message including a suggested candidate cell for LTM preparation. The UE Context Modification Request message includes:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication.

Proposal 2. The preparation of candidate cells in intra-CU C-LTM can take place in parallel, similar as in Rel-18 intra-CU LTM.

If the candidate gNB-DU or the source gNB-DU accepts the LTM configuration request, it generates and transmits the lower layer configuration of the prepared LTM candidate cell in its response to the gNB-CU in UE Context Setup Response.

When the parallel preparation of LTM candidate cells is completed, i.e., upon completion of Steps 4a and 4b in Figure 1, the gNB-CU knows about all the prepared candidate cells. The gNB-CU can then generate the L3 execution conditions based on this information and the UE RRC measurement reports.

Proposal 3. Upon receiving information regarding the prepared LTM candidate cells from its underlying DUs, the gNB-CU generates the L3 execution conditions for C-LTM:

- The gNB-CU generates the L3 execution conditions for cell switch from the source cell to other LTM candidate cells, as well as the execution conditions for cell switch from an LTM candidate cell to other candidate cells, i.e., for subsequent LTM.

Figure 1 below shows an example signalling flow of intra-CU C-LTM when the gNB-CU decides that L1 execution conditions should be configured.

Figure 2: Signalling flow for Intra-CU C-LTM with L1 execution conditions configured

Proposal 4. The gNB-CU initiates intra-CU C-LTM preparation, decides that L1 execution conditions should be configured, and transmits to an underlying candidate gNB-DU UE Context Setup Request including:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication,

- List of candidate cells belonging to other candidate DUs and source DU.

If an LTM candidate cell belongs to the source gNB-DU, the gNB-CU transmits to the source gNB-DU UE Context Modification Request including:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication,

- List of candidate cells belonging to other candidate DUs and source DU.

If the candidate gNB-DU accepts the LTM configuration request, it generates and transmits the lower layer configuration of the prepared LTM candidate cell in its response to the gNB-CU.

Proposal 5. If the candidate gNB-DU accepts the LTM configuration request, it generates and transmits in UE Context Setup Response:

- The lower layer configuration of prepared LTM candidate cell,

- C-LTM execution conditions from prepared candidate cell to other candidate cells.

Similarly, if the source gNB-DU accepts the LTM configuration request, it transmits the same information as above in UE Context Modification Response.

When the parallel preparation of LTM candidate cells is completed, i.e., upon completion of Steps 4a and 4b in Figure 2, the gNB-CU knows about the prepared candidate cells in the candidate gNB-DUs and the source gNB-DU. The gNB-CU forwards information regarding the prepared LTM candidate cells to each candidate gNB-DU and the source gNB-DU using UE Context Modification Request.

Proposal 6. Based on information regarding all the prepared LTM candidate cells:

- The source gNB-DU generates the L1 execution conditions for the initial LTM cell switch and may also generate the L1 execution conditions for a subsequent LTM cell switch.

- A candidate gNB-DU generates the L1 execution conditions for subsequent LTM cell switch.

Proposal 7. Each candidate gNB-DU and the source gNB-DU transmit the L1 execution conditions for the initial LTM cell switch and subsequent LTM cell switch to the gNB-CU in UE Context Modification Response.

The gNB-CU transmits the L3 execution conditions or the L1 execution conditions to the UE in the RRC reconfiguration message containing the LTM configuration for intra-CU C-LTM.

5. RACH-less Conditional intra-CU LTM is supported.

7. UE based TA measurement mechanism is supported for conditional intra-CU LTM.

8. PDCCH ordered early TA acquisition is supported for conditional LTM.

8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

10. Candidate cell TA is maintained by a new timer.

RAN2 made the following agreements regarding Early TA acquisition for intra-CU C-LTM.

Based on the agreements above, we think that for the Early TA RACH configuration procedure, the related Rel-18 intra-CU LTM procedures can be simply reused.

Proposal 8. The Rel-18 intra-CU LTM Early TA RACH configuration procedure can be reused for Early TA acquisition for intra-CU conditional LTM.

Similarly, the Rel-18 CSI configuration procedure can be reused for providing the UE with configuration for performing L1 measurements and reporting.

Proposal 9. For Intra-CU conditional LTM, the Rel-18 LTM CSI configuration procedure can be reused for providing the UE with configuration based on SSBs for performing L1 measurements and reporting.

Proposal 10. For Intra-CU conditional LTM, the Rel-18 LTM CSI configuration procedure is enhanced to provide the UE with configuration based on CSI-RSs for performing L1 measurements and reporting, and the procedure consists of:

- The CU requests a candidate gNB-DU and the source gNB-DU to provide the CSI-RS resources and configuration for the UE, for a suggested candidate cell, in UE Context Setup Request and UE Context Modification Request, respectively.

- A candidate gNB-DU and the source gNB-DU provide the CSI-RS resources and configuration for the UE for the prepared candidate cell, in UE Context Setup Response and UE Context Modification Response, respectively.

- The CU generates the list of CSI-RS resources for all the LTM candidate cells and provide them to a candidate gNB-DU and the source gNB-DU, which then respond with the corresponding CSI reporting configurations for each LTM candidate cell.

# 3 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

Proposal 1. The gNB-CU initiates intra-CU C-LTM preparation, decides to configure L3 execution conditions, and transmits to an underlying candidate gNB-DU UE Context Setup Request including a suggested candidate cell for LTM preparation. The UE Context Setup Request message includes:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication.

If an LTM candidate cell belongs to the source gNB-DU, the gNB-CU also transmits to the source gNB-DU the UE Context Modification Request message including a suggested candidate cell for LTM preparation. The UE Context Modification Request message includes:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication.

Proposal 2. The preparation of candidate cells in intra-CU C-LTM can take place in parallel, similar as in Rel-18 intra-CU LTM.

If the candidate gNB-DU or the source gNB-DU accepts the LTM configuration request, it generates and transmits the lower layer configuration of the prepared LTM candidate cell in its response to the gNB-CU in UE Context Setup Response.

Proposal 3. Upon receiving information regarding the prepared LTM candidate cells from its underlying DUs, the gNB-CU generates the L3 execution conditions for C-LTM:

- The gNB-CU generates the L3 execution conditions for cell switch from the source cell to other LTM candidate cells, as well as the execution conditions for cell switch from an LTM candidate cell to other candidate cells, i.e., for subsequent LTM.

Proposal 4. The gNB-CU initiates intra-CU C-LTM preparation, decides that L1 execution conditions should be configured, and transmits to an underlying candidate gNB-DU UE Context Setup Request including:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication,

- List of candidate cells belonging to other candidate DUs and source DU.

If an LTM candidate cell belongs to the source gNB-DU, the gNB-CU transmits to the source gNB-DU UE Context Modification Request including:

- Suggested candidate cell ID,

- Intra-CU C-LTM indication,

- List of candidate cells belonging to other candidate DUs and source DU.

Proposal 5. If the candidate gNB-DU accepts the LTM configuration request, it generates and transmits in UE Context Setup Response:

- The lower layer configuration of prepared LTM candidate cell,

- C-LTM execution conditions from prepared candidate cell to other candidate cells.

Similarly, if the source gNB-DU accepts the LTM configuration request, it transmits the same information as above in UE Context Modification Response.

Proposal 6. Based on information regarding all the prepared LTM candidate cells:

- The source gNB-DU generates the L1 execution conditions for the initial LTM cell switch and may also generate the L1 execution conditions for a subsequent LTM cell switch.

- A candidate gNB-DU generates the L1 execution conditions for subsequent LTM cell switch.

Proposal 7. Each candidate gNB-DU and the source gNB-DU transmit the L1 execution conditions for the initial LTM cell switch and subsequent LTM cell switch to the gNB-CU in UE Context Modification Response.

Proposal 8. The Rel-18 intra-CU LTM Early TA RACH configuration procedure can be reused for Early TA acquisition for intra-CU conditional LTM.

Proposal 9. For Intra-CU conditional LTM, the Rel-18 LTM CSI configuration procedure can be reused for providing the UE with configuration based on SSBs for performing L1 measurements and reporting.

Proposal 10. For Intra-CU conditional LTM, the Rel-18 LTM CSI configuration procedure is enhanced to provide the UE with configuration based on CSI-RSs for performing L1 measurements and reporting, and the procedure consists of:

- The CU requests a candidate gNB-DU and the source gNB-DU to provide the CSI-RS resources and configuration for the UE, for a suggested candidate cell, in UE Context Setup Request and UE Context Modification Request, respectively.

- A candidate gNB-DU and the source gNB-DU provide the CSI-RS resources and configuration for the UE for the prepared candidate cell, in UE Context Setup Response and UE Context Modification Response, respectively.

- The CU generates the list of CSI-RS resources for all the LTM candidate cells and provide them to a candidate gNB-DU and the source gNB-DU, which then respond with the corresponding CSI reporting configurations for each LTM candidate cell.

# References

[1] Chair notes, RAN3 #127 meeting.