3GPP TSG-RAN WG3 #127bis                                              R3-251778

Wuhan, China, 7th – 11th April, 2025

Source:	CATT

Title:	Discussion on Evolution of Duplex Operation

Agenda Item:	19.2

Document for:	Discussion and decision

- Introduction

This document discusses Evolution of Duplex Operation base on WID [1].

- Information exchange among gNBs, including [RAN3]

  - Semi-static cell-specific SBFD time and frequency location configuration

  - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

  - Strongest DL beam information

  - CLI-mitigation request

- Discussion

  - Semi-static cell-specific SBFD time and frequency location configuration

gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message.

The SBFD time and frequency configuration is encoded as container defined by RAN2.

For F1AP, the above SBFD time and frequency location configuration can be provided during GNB-CU CONFIGURATION UPDATE in Neighbour Cell Information List IE from CU to DU.

Proposal 1: CU provides SBFD time and frequency location configuration to DU in Neighbour Cell Information List IE in GNB-CU CONFIGURATION UPDATE.

  - Measurement resource configuration and Strongest DL beam information

LS [1] received from RAN1 as below

From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

For beam pairing, victim gNB measures NZP CSI-RS which is provided by neighbour aggressor cell. After measurement, victim gNB sends measurement result i.e., the strongest DL beam information back to neighbour aggressor cell. According to the strongest DL beam information, neighbour aggressor cell will try to reduce interference by implementation.

It can be found that there are two steps need to be supported

- Step 1: neighbour aggressor cell sends a set of one or more periodic NZP CSI-RS resources to victim cell

- Step 2: victim cell provides strongest DL beam information in the context of NZP-CSI-RS/SSB to neighbour aggressor cell

For step 1, NZP CSI-RS resource defined in TS38.331 is used to configure reference signal Non-Zero-Power (NZP) CSI-RS for measurement. It can be added in Served Cell Information NR.

For CU/DU split, aggressor gNB-DU sends NZP CSI-RS resources to aggressor gNB-CU for further transfer from aggressor gNB to victim gNB. Victim gNB-CU sends received NZP CSI-RS resources to Victim gNB-DU in Neighbour Cell Information List.

Proposal 2: Add NZP CSI-RS resource Set and NZP CSI-RS resource in Served Cell Information NR in XnAP, and Neighbour Cell Information List in F1AP

For step 2, the strongest DL beam information is defined as CRI and SSB index.

The question is that if the reporting of the strongest DL beam is periodic or one-off.

- In case of periodic response, the strongest DL beam information can be added in the RESOURCE STATUS UPDATE base on the request in Report Characteristics from aggressor gNB.

- In case of one-off response, the strongest DL beam information can be added in NG-RAN NODE CONFIGURATION UPDATE or a new Xn signalling.

From technically point view, whether perform interference mitigation depends on aggressor gNB. If victim gNB sends strongest DL beam too often which may cause singling overhead and aggressor gNB cannot reduce interference so quick. If victim gNB sends strongest DL beam too lazy, the interference cannot be quick solved by aggressor gNB.

Observation 1: It is up to aggressor gNB decides whether to mitigate interference. The aggressor gNB should control how often receiving strongest DL beam from victim to avoid signalling overhead or miss some strongest DL beam information.

For CU/DU split, victim gNB-DU provides strongest DL beam to victim gNB-CU for further transfer from victim gNB to aggressor gNB. The same question as XnAP also exists in F1AP i.e., whether the reporting of the strongest DL beam from victim gNB-DU is periodic or one-off.

Proposal 3: RAN3 further discuss how to response the strongest DL beam in XnAP and F1AP i.e., periodic response or one-off response. We slightly prefer periodic response.

After aggressor gNB-CU receiving strongest DL beam information in the context of NZP-CSI-RS/SSB from victim gNB-CU, neighbour aggressor gNB-CU need to send strongest DL beam information to aggressor gNB-DU e.g., for beam adjustment.

The strongest DL beam information can be added in Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE message.

Proposal 4: Add strongest DL beam information in a Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE, or new F1AP signalling, sending from aggressor gNB-CU to aggressor gNB-DU.

  - CLI-mitigation request

- Notes: Following are assumed for information exchange among gNBs

  - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3

  - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

The victim gNB sends the CLI-mitigation handling request aggressor gNB performs the CLI-mitigation. The NG-RAN NODE CONFIGURATION UPDATE can be reused.

For CU/DU split, the CLI-mitigation handling request can be sent from victim gNB-DU to victim gNB-CU first.

For beam paring, we do not need to specify when to send CLI-mitigation request in specification. The NZP CSI-RS exchange can be happened at any time. For example, CLI-mitigation request send before NZP CSI-RS exchange which can be considered as a trigger for aggressor gNB to send NZP CSI-RS to victim gNB to try to reduce CLI. Or the NZP CSI-RS exchange happen before real interface occurs.

For beam nulling, the CLI-mitigation request can trigger aggressor gNB starts measuring to reduce CLI base on the NZP CSI-RS provided from victim gNB. If NZP CSI-RS of victim did not send yet, the aggressor can wait for receiving NZP CSI-RS of victim and then start measuring.

Proposal 5: For both beam paring and beam nulling, CLI-mitigation request can be send either before or after receiving/sending NZP CSI-RS. There is no need to add order restrictions to the specification.

Proposal 6: RAN3 agree to introduce CLI-mitigation request in XnAP NG-RAN NODE CONFIGURATION UPDATE and F1AP gNB-DU/CU CONFIGURATION UPDATE.

- Conclusion

Proposal 1: CU provides SBFD time and frequency location configuration to DU in Neighbour Cell Information List IE in GNB-CU CONFIGURATION UPDATE.

Proposal 2: Add NZP CSI-RS resource Set and NZP CSI-RS resource in Served Cell Information NR in XnAP, and Neighbour Cell Information List in F1AP.

Observation 1: It is up to aggressor gNB decides whether to mitigate interference. The aggressor gNB should control how often receiving strongest DL beam from victim to avoid signalling overhead or miss some strongest DL beam information.

Proposal 3: RAN3 further discuss how to response the strongest DL beam in XnAP and F1AP i.e., periodic response or one-off response. We slightly prefer periodic response.

Proposal 4: Add strongest DL beam information in a Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE, or new F1AP signalling, sending from aggressor gNB-CU to aggressor gNB-DU.

Proposal 5: For both beam paring and beam nulling, CLI-mitigation request can be send either before or after receiving/sending NZP CSI-RS. There is no need to add order restrictions to the specification.

Proposal 6: RAN3 agree to introduce CLI-mitigation request in XnAP NG-RAN NODE CONFIGURATION UPDATE and F1AP gNB-DU/CU CONFIGURATION UPDATE, or new XnAP/F1AP signalling.

- Reference

[1] R1-2407533, LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation, RAN1.