3GPP TSG-RAN3 Meeting #127bis	R3-251784

Wuhan, China, Feb 07-11, 2025

Agenda Item:	14.3

Source: 		Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Xiaomi, China Telecom

Title: 	(TP for TS 38.300) Discussion on Hard FLSO and RRC Inactive state in NR NTN Regenerative Payload

Document for:	Discussion/Decision

# Background

In this paper we provide our views on Inactive UE support and hard feeder link switch over due to moving gNB on satellite.

RAN3#126 Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Any standard impact? Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

# Discussion

## Feeder Link Switch Over

In the previous RAN3 meetings it was agreed to consider only the scenario where FLSO happens without AMF relocation. In the last meeting it was agreed that Legacy NG HO can be used if AMF also changes with Feeder Link switch over which does not have any specification impacts.

For soft and hard feeder link without AMF relocation the AMF and the gNB remains the same. Hence the UE context and NG context in AMF and gNB remains intact. Just the NTN GW IP address changes due to FLSO. Since the Feeder link and the NTN GW is out of 3GPP scope, the NTN operators can handle it via existing mechanisms like mobike, SCTP multihoming etc.

For soft and hard feeder link switch over without AMF relocation, the context in gNB and AMF remains intact. For soft FLSO, since AMF and gNB remains connected, current HO mechanisms can be used without any specification impact.

For hard FLSO, there would be a break in NGAP connection between AMF and gNB. The gNB and AMF needs to stop transmission over NG (NG-C and NG-U) at time T and restart at time T1 after hard FLSO. This interruption of NG due to hard FLSO needs to be handled in NG application layer as this needs handshaking between peers at both the end of NG connection to stop and start transmission synchronously.

For hard feeder link switch over without AMF relocation, the NG connection will be interrupted and both gNB and AMF should stop and start transmission over NG-C and NG-U before and after hard FLSO. This needs to be handled over NGAP to stop and start transmission synchronously.

Existing NGAP interface management procedures (e.g RAN/AMF Config Update procedures) could be enhanced with an IE to indicate the interruption in the NG connection due to hard FLSO. When AMF receives the indication, it can inform SMF/UPF accordingly.

Existing NGAP interface management procedures like RAN/AMF Config Update procedures could be enhanced with an IE to indicate interruption on NG connection due to hard FLSO. Agree to Stage 2 TP to TS 38.300 in the Annex. Stage 3 TP in R3-251731

## RRC_Inactive UE Support

In the last RAN3 meeting it was agreed to reuse the existing mechanism and procedures to support RRC Inactive state in Regenerative payload. Due to the movement of the gNB on satellite, UE may have been moved to RRC inactive state in gNB1 and UE might resume on gNB4 in the same geographical location. As Xn connection may not be available between gNB1 and gNB4, it is not possible for gNB4 to retrieve UE context from gNB1.

Due to the satellite movement, UE may go inactive in gNB1 and resume in gNB4 in the same geographical location. Due to missing Xn between gNB1 and gNB4, it is not possible to retrieve the inactive UE context from gNB1 to gNB4 using the current mechanism.

To address the problem of inactive UE context retrieval, the following solutions are proposed.

Option 1 – RAN Paging to relocate Inactive UE context

This option is described in HW paper in R3-244548. In this solution, when the gNB moves away from a particular geographical area, the gNB pages all the RRC inactive UEs wakes them up and moves the UEs to the subsequent gNB which covers the same geographical area. This solution defeats the purpose of RRC Inactive state and there is no UE power savings.

Option 2 – Proactive Inactive UE Context movement

In this option the inactive UE context is proactively moved from the gNB leaving a particular geographical area to an incoming gNB covering the same geographical area. This way when the UE resumes, the UE context is available in the nearby gNB to retrieve the context.

Option 3 – Hop by hop Retrieve UE context

In this option, the gNB where the UE resumes sends Retrieve UE Context Request to its neighbouring gNBs and the neighbouring gNBs forwards the request to its neighbours until the gNB which has the inactive UE context get the Retrieve UE context request. This solution does not have much standards impact. But this solution causes latency and signaling storm.

Option 4 – Move the UE Context to ground

In this option, the inactive UE context is moved to AMF or NTN GW on the ground. When the UE resumes, the context is retrieved from the AMF or NTN GW by the gNB where the UE resumes.

These are the few options listed, however there could be other options as well which are not precluded. RAN3 can discuss further on the solutions to solve this issue.

RAN3 to discuss the above options to address the issue of retrieving RRC_Inactive UE context from anchor gNB in NTN Regenerative payload. Detailed analysis of the solution options and the preferred solution is provided in R3-251766

# Summary

Based on the discussion above, we have following observations:

Observation 1.	For soft and hard feeder link switch over without AMF relocation, the context in gNB and AMF remains intact. For soft FLSO, since AMF and gNB remains connected, current HO mechanisms can be used without any specification impact.

Observation 2.	For hard feeder link switch over without AMF relocation, the NG connection will be interrupted and both gNB and AMF should stop and start transmission over NG-C and NG-U before and after hard FLSO. This needs to be handled over NGAP to stop and start transmission synchronously.

Observation 3.	Due to the satellite movement, UE may go inactive in gNB1 and resume in gNB4 in the same geographical location. Due to missing Xn between gNB1 and gNB4, it is not possible to retrieve the inactive UE context from gNB1 to gNB4 using the current mechanism.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Existing NGAP interface management procedures like RAN/AMF Config Update procedures could be enhanced with an IE to indicate interruption on NG connection due to hard FLSO. Agree to Stage 2 TP to TS 38.300 in the Annex. Stage 3 TP in R3-251731

Proposal 2:	RAN3 to discuss the above options to address the issue of retrieving RRC_Inactive UE context from anchor gNB in NTN Regenerative payload. Detailed analysis of the solution options and the preferred solution is provided in R3-251766

# Annex (TP for TS 38.300)

16.14.4	Switchover

16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN.

16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption and/or NG link interruption may occur during the transition between the feeder links.

16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

For Regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface is sent to AMF from gNB in the NGAP Configuration Update message when the Hard Feeder Link switch occurs.