3GPP TSG-RAN3 Meeting #127bis	R3-251787

Wuhan, China, April 07-11, 2025

Agenda Item:	11.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on remaining issues in AI/ML enabled CCO

Document for:	Discussion/Decision

# Background

In RAN3#127 the below agreements were made, and the below open issues were captured.

RAN3#127 Each Future Coverage Modification Notification Item included in the Future Coverage Modification Notification List IE has a Future Coverage Modification Cause IE associated to it from DU to CU. The Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only. Value ‘0’ for the Future Cell Coverage State IE is needed and has the same meaning as cell inactive in legacy CCO. Exchange timer infor for predicted CCO issue over XnAP is not needed unless the usage can be identified? The maximum value of Time for predicted CCO issue and future CCO state? 60Sec, 120 Sec or 300 Sec? UE performance feedback for CCO? Legacy UE performance feedback is sufficient or other finer granularity, e.g., cell-/SSB-level UE performance feedback? Whether gNB-CU provides to the gNB-DU not only the predicted CCO issue but also the corresponding future CCO state as a recommendation?

In this paper we discuss our views on remaining stage 3 issues in AI/ML enabled CCO.

# Discussion

## Stage 3 Open Issues

The associated time information for predicted CCO issue and future CCO state was agreed to be a relative time of certain duration in seconds calculated from the time of receiving the Config Update message over Xn or F1 interface. However the max value of the time information could not be agreed upon in the last meeting. Few companies preferred a shorter duration and few companies wanted more relaxed duration. We think the max duration can be around 5 mins (300 seconds) to allow the node to react and change its coverage.

The associated time information for predicted CCO issue and future CCO state can have a maximum value of 300 seconds (5 mins) to allow the node to react to the issue and modify its coverage.

In the last meeting few companies were proposing to send Time for Predicted CCO issue to neighbouring CU over Xn and neighbouring CU can pass the same information to neighbouring DU. Currently the Predicted CCO issue is passed as Predicted Coverage Modification cause. Some companies in the previous meeting mentioned that if Predicted CCO issue time is provided to the neighbouring node, the neighbouring node can decide on a CCO state change time to align with the time associated with predicted CCO issue. It is ok to provide additional information to the neighbouring nodes to assist in deciding the future CCO state.

Exchange Time for Predicted CCO issue (“Time for Predicted CCO issue” IE) to the neighbouring CU over Xn along with future CCO state and associated time as an additional information to assist the neighbouring node to derive its own future CCO state at an appropriate future time instance.

## Future CCO State Feedback

During the SI phase Predicted CCO issue and future CCO state feedback was not discussed due to time constraints. We therefore propose to discuss the feedback for Predicted CCO issue and future CCO state in normative phase.  Once the predicted coverage pattern is applied, the neighboring gNBs can send feedback on Cell performance like throughput, packet loss, packet delay, RLF reports etc to the source gNB to inform the effects of applying predicted coverage pattern. With only UE performance it is unclear which cell the UE belong to (whether it is the cell where the future CCO state was applied or a normal cell). Hence the neighbouring node can aggregate the UE performance at a cell level and send it over Xn or the neighbouring node also reports the NR-CGI where the UE belongs for an UE reporting its performance.

RAN3 to discuss the following options as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

- Legacy UE performance metrics (throughput, packet delay, packet loss) including Cell ID where the UE belonged to.

- SON Reports like RLF, RACH report etc

## Future CCO State Assistance Information

A Note was added in the TR 38.743 to further discuss if gNB-CU-CP can generate the future CCO state as assistance information and send it to gNB-DU. On this Note, we are unclear on what are the input parameters which gNB-CU-CP will use to generate future CCO state. Moreover, OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the configuration model and SA5 impact. This is also not a legacy behaviour. There could also be a race condition of CU suggesting a different CCO state and OAM informing DU a different CCO State. Hence, we propose, that gNB-DU generates the future CCO state on its own and future CCO state need not be provided as assistance information from gNB-CU-CP.

The input parameters used by gNB-CU-CP to generate future CCO state is unclear. OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the OAM configuration model and SA5 impact. Race condition at gNB-DU is also a possibility if CU and OAM suggest different CCO state to gNB-DU. This will also result in duplication of functionality across gNB-CU-CP and gNB-DU which is not desired.

There is no need for gNB-CU to generate Future CCO state as assistance information and provide it to gNB-DU.

## Assistance Information for Prediction of CCO Issue

It was agreed the gNB-CU will predict the CCO issue and forward it to the gNB-DU. gNB-CU predicts the CCO issue based on L3 measurements. gNB-DU has L1 measurements and beam level load information. Hence gNB-DU has a better visibility of the beam level information for detecting CCO issue. We therefore propose that gNB-DU can send affected beams based on L1 measurements as assistance information to gNB-CU.

gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measurements for detecting CCO issue.

For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. RAN3 to discuss including “Assistance Information on Affected beams” over F1 as input data for CCO issue prediction.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	The input parameters used by gNB-CU-CP to generate future CCO state is unclear. OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the OAM configuration model and SA5 impact. Race condition at gNB-DU is also a possibility if CU and OAM suggest different CCO state to gNB-DU. This will also result in duplication of functionality across gNB-CU-CP and gNB-DU which is not desired.

Observation 2.	gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measurements for detecting CCO issue.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	The associated time information for predicted CCO issue and future CCO state can have a maximum value of 300 seconds (5 mins) to allow the node to react to the issue and modify its coverage.

Proposal 2:	Exchange Time for Predicted CCO issue (“Time for Predicted CCO issue” IE ) to the neighbouring CU over Xn along with future CCO state and associated time as an additional information to assist the neighbouring node to derive its own future CCO state at an appropriate future time instance.

Proposal 3:	RAN3 to discuss the following options as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

	Legacy UE performance metrics (throughput, packet delay, packet loss) including Cell ID where the UE belonged to.

	SON Reports like RLF, RACH report etc

Proposal 4:	There is no need for gNB-CU to generate Future CCO state as assistance information and provide it to gNB-DU.

Proposal 5:	For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. RAN3 to discuss including “Assistance Information on Affected beams” over F1 as input data for CCO issue prediction.