3GPP TSG-RAN WG3 Meeting #127bis	R3-251808

# Wuhan, China, 7– 11 April 2025

Agenda item:	9.2

Source: 	Samsung, ZTE, Huawei, Ericsson, Nokia, China Telecom, Qualcomm, CMCC, CATT

Title: 	Discussion on SRS information

Document for:	Discussion and decision

Introduction

Last meeting primarily discusses a correction to SRS information, and the outcome is to be continued, and companies will have more time to check,

| R3-250631 | Correction to SRS information (Samsung, ZTE, CATT, Qualcomm, Nokia, China Telecom) | CR0188r, TS 38.455 v18.4.0, Rel-18, Cat. F HW, E///: Further check needed To be continued... |
| --- | --- | --- |

In this contribution, we further discuss the possible corrections to this open issue.

Discussion

Last meeting one correction to SRS information is submitted in order to solve the issue identified on the incompleteness of Requested validity area specific SRS information, as given in the Reason for change in R3-250631,

In Rel-18, Validity Area Specific SRS Information IE was introduced in the Requested SRS Transmission Characteristics IE. The NG-RAN may take this information into account for configuring SRS transmissions for the UE in the indicated validity area during Positioning Information Exchange procedure, and/or may reserve the indicated SRS information during SRS Information Reservation Notification procedure.

However, the current information contained in the Validity Area Specific SRS Information IE corresponds to only one SRS resource, which leads to the following situations,

- During Positioning Information Exchange procedure, the LMF can recommend only one SRS resource for NG-RAN to configure SRS configuration containing multiple SRS resource sets and multiple SRS resources in each SRS resource set. Resulting in more chances for different NG-RAN node to allocate the same area-specific SRS resources for different UEs, which should be avoided.

- During SRS Information Reservation Notification procedure, the LMF can notify the NG-RAN to reserve only one SRS resource in each SRS INFORMATION RESERVATION NOTIFICATION message, which is inefficient if the LMF needs to notify NG-RAN to reserve multiple SRS resources.

It seems that for now all companies have acknowledged this issue, but the solution is still open.

Firstly, regarding the solution proposed in R3-250631, we adopted a loop over loop approach to provide more detailed requested information.

However, after further check, such solution may not work because it does not consider the maximum number of positioning SRS resources that can be configured per positioning SRS resource set.

More specifically, as defined in current spec,

1. a maximum of 64 positioning srs resources can be configured for one srs config

2. a maximum of 16 positioning srs resource sets can be configured for one srs config

3. a maximum of 16 positioning srs resources can be configured per positioning srs resource set

In fact, the loop over loop approach provided in R3-250631 only reflects bullet 1&2, and may reflect them in a wrong way without considering bullet3.

In order to correctly reflect the three bullets above, there are several options,

Option 1: Still use loop over loop approach, but revise the range of Validity Area Specific Positioning SRS Resource Item to maxnoSRS-PosResourcePerSet instead of maxnoSRS-PosResources. This is an NBC change, and does not reflect bullet 1.

Option 2: Revise 9.2.96 to have two separate list for Positioning SRS Resource and Positioning SRS Resource set (similar to what has specified in 9.2.28). This is an NBC change, but can reflect all three bullets.

Option 3: In 9.2.27, Add Validity area specific SRS information Extended IE which refers to 9.2.28. This is a BC change, and can reflect all three bullets.

Option 4: In 9.2.27, Add Validity area specific SRS information Extended IE which captures two separate list as defined in Option 2. THis is a BC change, and can reflect all three bullets.

And other options are not precluded.

Proposal 1: RAN3 is kindly asked to discuss potential options as follows,

Option 1: Still use loop over loop approach, but revise the range of Validity Area Specific Positioning SRS Resource Item to maxnoSRS-PosResourcePerSet instead of maxnoSRS-PosResources.

Option 2: Revise 9.2.96 to have two separate list for Positioning SRS Resource and Positioning SRS Resource set (similar to what has specified in 9.2.28).

Option 3: In 9.2.27, Add Validity area specific SRS information Extended IE which refers to 9.2.28.

Option 4: In 9.2.27, Add Validity area specific SRS information Extended IE which captures two separate list as defined in Option 2.

Other options are not precluded, if any.

We also provide tentative NRPPa corrections regarding Option 1-4 in the Annex. Note that F1AP will also be impacted.

Conclusion

In this paper, we provides further considerations on SRS information. The following observations are provided,

Proposal 1: RAN3 is kindly asked to discuss potential options as follows towards NRPPa CR,

Option 1: Still use loop over loop approach, but revise the range of Validity Area Specific Positioning SRS Resource Item to maxnoSRS-PosResourcePerSet instead of maxnoSRS-PosResources.

Option 2: Revise 9.2.96 to have two separate list for Positioning SRS Resource and Positioning SRS Resource set (similar to what has specified in 9.2.28).

Option 3: In 9.2.27, Add Validity area specific SRS information Extended IE which refers to 9.2.28.

Option 4: In 9.2.27, Add Validity area specific SRS information Extended IE which captures two separate list as defined in Option 2.

Other options are not precluded, if any.

Annex A – Option 1 for NRPPa

-------------------------------------------Start of changes-------------------------------------------

9.2.96	Validity Area Specific SRS Information

This IE contains the validity area specific SRS Information when area-specific SRS is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Validity Area Specific Positioning SRS Resource Set List |  | 1 |  |  | - |  |
| >Validity Area Specific Positioning SRS Resource Set Item |  | 1..<maxnoSRS-PosResourceSets> |  |  | - |  |
| >>Validity Area Specific Positioning SRS Resource List | M |  | 9.2.XX |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourceSets | Maximum no of positioning SRS resource sets per UL BWP. Value is 16. |

-------------------------------------------Next change-------------------------------------------

9.2.XX	Validity Area Specific Positioning SRS Resource List

This IE contains the validity area specific Positioning SRS resource list.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Validity Area Specific Positioning SRS Resource Item |  | 1..<maxnoSRS-PosResourcePerSet> |  |  | - |  |
| >CHOICE Transmission Comb | O |  |  |  | - |  |
| >>Comb Two |  |  |  |  |  |  |
| >>>Comb Offset | M |  | INTEGER(0..1) |  | - |  |
| >>>Cyclic Shift | M |  | INTEGER(0..7) |  | - |  |
| >>Comb Four |  |  |  |  |  |  |
| >>>Comb Offset | M |  | INTEGER(0..3) |  | - |  |
| >>>Cyclic Shift | M |  | INTEGER(0..11) |  | - |  |
| >>Comb Eight |  |  |  |  |  |  |
| >>>Comb Offset | M |  | INTEGER(0..7) |  | - |  |
| >>>Cyclic Shift | M |  | INTEGER(0..5) |  | - |  |
| >Resource Mapping |  | 0..1 |  |  | - |  |
| >>Start Position | M |  | INTEGER(0..13) |  | - |  |
| >>Number of Symbols | M |  | ENUMERATED(n1,n2,n4, n8, n12) |  | - |  |
| >Frequency Domain Shift | O |  | INTEGER(0..268) |  | - |  |
| >C-SRS | O |  | INTEGER(0..63) |  | - |  |
| >CHOICE Resource Type Positioning | O |  |  |  | - |  |
| >>periodic |  |  |  |  |  |  |
| >>>SRSPeriodicity | M |  | 9.2.99 |  | - |  |
| >>>Offset | M |  | INTEGER(0..81919,…) |  | - |  |
| >>>SRS PosPeriodicConfigHyperSFN Index | O |  | ENUMERATED(even0, odd1) | Indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480ms | YES | ignore |
| >>semi-persistent |  |  |  |  |  |  |
| >>>SRSPeriodicity | M |  | 9.2.99 |  | - |  |
| >>>Offset | M |  | INTEGER(0..81919,…) |  | - |  |
| >>>SRS PosPeriodicConfigHyperSFN Index | O |  | ENUMERATED(even0, odd1) | Indicates even or odd hyper SFN in which the SRS for positioning is transmitted for the periodicity value of 20480ms | YES | ignore |
| >>aperiodic |  |  |  | Not applicable if the Positioning Validity Area Cell List IE is included | - |  |
| >>>slot offset | M |  | INTEGER(0..32) |  | - |  |
| >Sequence ID | O |  | INTEGER(0..65535) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourcePerSet | Maximum no of positioning SRS resources per positioning SRS resource set. Value is 16. |

(ASN.1 to be added later)

Annex B – Option 2 for NRPPa

-------------------------------------------Start of changes-------------------------------------------

9.2.96	Validity Area Specific SRS Information

This IE contains the validity area specific SRS Information when area-specific SRS is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning SRS Resource List |  | 0..< maxnoSRS-PosResources> |  |  |  |  |
| >Positioning SRS Resource | M |  | 9.2.30 | Corresponds to information provided in SRS-PosResource contained in SRS-Config IE as defined in TS 38.331 [13] |  |  |
| Positioning SRS Resource Set List |  | 0..<maxnoSRS-PosResourceSets> |  |  |  |  |
| >Positioning SRS Resource Set | M |  | 9.2.32 | Corresponds to information provided in SRS-PosResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources per UL BWP. Value is 64. |
| maxnoSRS-PosResourceSets | Maximum no of positioning SRS resource sets per UL BWP. Value is 16. |

(ASN.1 to be added later)

Annex C – Option 3 for NRPPa

-------------------------------------------Start of changes-------------------------------------------

9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. | - |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  | - |  |
| CHOICE Bandwidth | M |  |  |  | - |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ... , 160mHz, 200mHz, 15mHz, 25mHz, 30mHz, 60mHz , 35mHz, 45mHz, 70mHz, 90mHz) |  |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,…, 600mHz, 800mHz, 1600mHz, 2000mHz) |  |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  | - |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  | - |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. | - |  |
| >>Periodicity List |  | 0.. 1 |  |  | - |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  | - |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds | - |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | - |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  | - |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  | - |  |
| SSB Information | O |  | 9.2.54 |  | - |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |
| Bandwidth Aggregation Request Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| Positioning Validity Area Cell List | O |  | 9.2.93 |  | YES | ignore |
| Validity Area Specific SRS Information | O |  | 9.2.96 | This IE is ignored if the Validity Area Specific SRS Information Extended IE is present. | YES | ignore |
| Validity Area Specific SRS Information Extended | O |  | SRS Configuration 9.2.28 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

(ASN.1 to be added later)

Annex D – Option 4 for NRPPa

-------------------------------------------Start of changes-------------------------------------------

9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. | - |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  | - |  |
| CHOICE Bandwidth | M |  |  |  | - |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ... , 160mHz, 200mHz, 15mHz, 25mHz, 30mHz, 60mHz , 35mHz, 45mHz, 70mHz, 90mHz) |  |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,…, 600mHz, 800mHz, 1600mHz, 2000mHz) |  |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  | - |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  | - |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. | - |  |
| >>Periodicity List |  | 0.. 1 |  |  | - |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  | - |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds | - |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | - |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  | - |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  | - |  |
| SSB Information | O |  | 9.2.54 |  | - |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |
| Bandwidth Aggregation Request Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| Positioning Validity Area Cell List | O |  | 9.2.93 |  | YES | ignore |
| Validity Area Specific SRS Information | O |  | 9.2.96 | This IE is ignored if the Validity Area Specific SRS Information Extended IE is present. | YES | ignore |
| Validity Area Specific SRS Information Extended | O |  | 9.2.x |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

-------------------------------------------Next change-------------------------------------------

9.2.x	Validity Area Specific SRS Information Extended

This IE contains the extended validity area specific SRS Information when area-specific SRS is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning SRS Resource List |  | 0..< maxnoSRS-PosResources> |  |  |  |  |
| >Positioning SRS Resource | M |  | 9.2.30 | Corresponds to information provided in SRS-PosResource contained in SRS-Config IE as defined in TS 38.331 [13] |  |  |
| Positioning SRS Resource Set List |  | 0..<maxnoSRS-PosResourceSets> |  |  |  |  |
| >Positioning SRS Resource Set | M |  | 9.2.32 | Corresponds to information provided in SRS-PosResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources per UL BWP. Value is 64. |
| maxnoSRS-PosResourceSets | Maximum no of positioning SRS resource sets per UL BWP. Value is 16. |

(ASN.1 to be added later)