3GPP TSG-RAN WG3 Meeting #127bis	R3-251813

# Wuhan, China, 7– 11 April 2025

Agenda item:	16.2

Source: 	Samsung

Title: 	Discussion on RAN architecture and procedures for AIoT

Document for:	Discussion and decision

Introduction

Last meeting primarily discussed the RAN architecture and procedures for AIoT, and the following open issues are captured,

FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support.

FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT?

FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure?

FFS: Command request procedure is a per device procedure or to be used for multiple devices?

FFS: The need for a Command Report procedure?

FFS: Device association between gNB and AIOTF is used in Command procedure.

In this contribution, we further discuss these open issues.

Discussion

Last meeting primarily discussed the RAN architecture and procedures for AIoT, and a bunch of open issues are captured.

From the aspect of RAN architecture, the following open issues are captured,

FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support.

FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT?

FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

In our understanding, there’s no issue to support signaling approach to transfer reader related information, just like what we did in TS 38.455 for TRP information exchange. About the details of the location if reader, since currently the reader is BS reader which is static in location, the current Geographical Coordinates IE specified in TS 38.455 can be referred to.

About whether to use existing Tracking Area or define a new area to describe AIoT related area, since the length of current TAC is 3 octets, there’s still abundant spaces to use TAC to deploy AIoT related area. So we prefer to use the existing Tracking Area to describe AIoT related area over NGAP.

Regarding the case when the gNB receives both the area and the reader list in Inventory Request, we see no issue to let the gNB to make the final decision on which readers are used.

From the aspect of Inventory procedures, the following open issue is captured,

FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

Firstly, the Correlation Identifier is surely to be included.

While for the AIOTF identifier, we need to identify whether one AIoT RAN node can connect to multiple AIOTF at this stage. In our understanding, currently we mainly focus on indoor scenario which is normally deployed in a comparatively small areas, we see no strong motivation to deploy multiple AIOTF for such scenarios. So we prefer not to include AIOTF identifier for Inventory related messages in this release.

From the aspect of Command procedures, the following open issues are captured,

FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure?

FFS: Command request procedure is a per device procedure or to be used for multiple devices?

FFS: The need for a Command Report procedure?

FFS: Device association between gNB and AIOTF is used in Command procedure.

For the inclusion of identifiers, same understanding as for Inventory related messages, include Correlation Identifier but do not include AIOTF Identifier.

On whether to include the same Correlation Identifier IE, we think it can be decided by SA2, RAN3 really does not care whether the Correlation ID is the same or not, since there’s no difference on operations for the BS reader no matter the Correlation ID is the same or not.

For the question on whether Command request procedure is a per device procedure or non-device associated. We think it is enough to use Correlation ID to identify the procedure, and there’s no need to define additional ‘NGAP device ID’ or something.

For the need of Command Report procedure, temporarily we see no need to introduce such procedure, although the Command request procedure can be used for multiple devices. The gNB is able to collect the replies from all the affected devices, and then send NGAP Command response once for all.

Proposal 1: Include the Correlation Identifier for Inventory and Command related messages.

From the aspect of Interface management procedures, the following open issues are captured,

FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF.

FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF.

FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

Now the question is about whether to reuse the existing interface management procedure, or define new procedure to transfer reader related info. The FFS captured here has clearly indicated one of the drawbacks to use the existing procedures, i.e. how to handle the not applicable mandatory IEs, which may take a lot of time to discuss if we follow this approach. Although it is feasible that the existing interface management procedures can be reused, we prefer to define a new procedure to transfer such reader related info.

In addition, since it has been agreed that the reader related info can be configured via OAM, if we further agree to use additional signaling approach, we think such procedure should be initiated by AIOT CN other than AIOT RAN; in other words, the AIOT RAN is not necessary to provide the reader related info proactively to CN.

Proposal 2: Prefer to add a new Class-1 procedure initiated by CN to exchange the reader related information.

For other potential parameters that can be included in the Inventory Request message, based on LS from SA2 to RAN2 (CC RAN3), SA2 has confirmed that the following information can be provided from CN to the reader if available,

- the service type of A-IoT (e.g. inventory, command)

- whether the service is targeted for a single or multiple devices;

- the approximate number of target devices (if available).

To align with the discussion in RAN2 and SA2, we believe the above three parameters are also needed to be included in the Inventory Request message. For the service type of AIoT, we think it can be distinguished by the procedure code so no need to additionally include such information in the Inventory Request message.

Proposal 3: Include the following parameters in the Inventory Request message,

- Whether the service is targeted for a single or multiple devices;

- The approximate number of target devices (if available).

The last open issue is actually identified by RAN2, as per the conclusion of RAN2 email discussion,

Wait for SA2/RAN3 progress before further discussing on how the reader calculates transaction ID.

Discuss whether to send an LS: To SA2 asking whether correlation ID is expected to be same or different when the same service is requested from different readers, and to RAN3 asking whether coordination between readers is expected.

The scenario is that multiple readers take part in a same service request (with the same Correlation ID), and there’s coverage overlap among multiple readers. Since RAN2 does not expect one specific AIOT device responds to multiple readers for the same service request, the network should have a way to ensure that the affected AIOT devices will not respond to multiple readers for the same service request. As a result, some kind of coordination between readers is taken into consideration.

Before we discuss the coordination, RAN3 also needs to decide whether it is possible that multiple AIoT RAN nodes receive a same service request from CN within a same time period, which is highly dependent on Operators’ requirement.

Proposal 4: RAN3 is kindly asked to discuss whether coordination between readers is expected.

Conclusion

In this paper, we provide further discussion on RAN-CN interface impact for Ambient IoT. The following proposals are provided,

Proposal 1: Include the Correlation Identifier for Inventory and Command related messages.

Proposal 2: Prefer to add a new Class-1 procedure initiated by CN to exchange the reader related information.

Proposal 3: Include the following parameters in the Inventory Request message,

- Whether the service is targeted for a single or multiple devices;

- The approximate number of target devices (if available).

Proposal 4: RAN3 is kindly asked to discuss whether coordination between readers is expected.

Reference