3GPP TSG-RAN WG3 Meeting #127bis	R3-251819

# Wuhan, China, 7th April – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1962 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | X2 TNL address exchange |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a possibility that retrieving the X2 TNL address of the candidate eNB (e.g., if the candidate eNB uses a Multi-Operator Core Network (MOCN) may fail in certain deployment scenarios. This can occur if the candidate eNB is not provided with unambiguous information regarding which X2 TNL address to return. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new Requested TNL Information IE within SON Configuration Transfer indicating the PLMN ID which has to be taken into account by the candidate eNB when providing X2 TNL Configuration Info. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release), as only the eNB/MME Configuration Transfer functions are impacted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, retrieving the TNL address of the candidate eNB (using Multi-Operator Core Network (MOCN) might result in the inability to setup a signalling TNL connection between the (source) eNB and the candidate eNB |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.16.2.1, 9.2.3.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.300 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Resubmission to R3#127bis |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.16.2	Successful Operation

#### 8.16.2.1	MME Configuration Transfer

Figure 8.16.2.1-1: MME Configuration Transfer procedure. Successful operation.

The procedure is initiated with an MME CONFIGURATION TRANSFER message sent from the MME to the eNB.

If the eNB receives, in the SON Configuration Transfer IE or the EN-DC SON Configuration Transfer IE, the SON Information IE containing the SON Information Request IE, it may transfer back the requested information either towards the eNB indicated in the Source eNB-ID IE of the SON Configuration Transfer IE or towards the eNB indicated in the Source eNB-ID IE of the EN-DC SON Configuration Transfer IE by initiating the eNB Configuration Transfer procedure. If the X2 TNL Configuration Info IE contains the eNB Indirect X2 Transport Layer Addresses IE, the eNB may use it for the X2 TNL establishment, and may transfer back the received eNB Indirect X2 Transport Layer Addresses towards the eNB indicated in the Source eNB-ID IE of the SON Configuration Transfer IE by initiating the eNB Configuration Transfer procedure or towards the eNB indicated in the Source eNB-ID IE of the EN-DC SON Configuration Transfer IE by initiating the eNB Configuration Transfer procedure.

If the eNB receives, in the SON Configuration Transfer IE, the X2 TNL Configuration Info IE containing the eNB X2 Extended Transport Layer Addresses IE, it may use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the eNB receives, in the SON Configuration Transfer IE or the EN-DC SON Configuration Transfer IE, the SON Information IE containing the SON Information Reply IE including the X2 TNL Configuration Info IE as an answer to a former request, it may use it to initiate the X2 TNL establishment. If the X2 TNL Configuration Info IE contains the eNB Indirect X2 Transport Layer Addresses IE, the eNB may use it for the X2 TNL establishment.

In case the IP-Sec Transport Layer Address IE is present and the GTP Transport Layer Addresses IE within the eNB X2 Extended Transport Layer Addresses IE is not empty, GTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel end point given in by the IP-Sec Transport Layer Address IE.

In case the IP-Sec Transport Layer Address IE is not present, GTP traffic is terminated at the end points given by the list of addresses in eNB GTP Transport Layer Addresses IE within the eNB X2 Extended Transport Layer Addresses IE.

In case the eNB GTP Transport Layer Addresses IE is empty and the IP-Sec Transport Layer Address IE is present, SCTP traffic is conveyed within an IP-Sec tunnel terminated at the IP-Sec tunnel end point given in the IP-Sec Transport Layer Address IE, within the eNB X2 Extended Transport Layer Addresses IE.

If the eNB is configured to use one IPsec tunnel for all S1 and X2 traffic (IPsec star topology) then the traffic to the peer eNB shall be routed through this IPsec tunnel and the IP-Sec Transport Layer Address IE shall be ignored.

If the eNB receives the SON Information IE containing the SON Information Reply IE including the Time Synchronisation Info IE as an answer to a former request, it may use it for over-the-air synchronisation by means of network listening and for triggering muting activation request.

If the eNB receives the SON Information IE containing the SON Information Report IE, it may use it as specified in TS 36.300 [14].

If the eNB receives the Inter-system SON Configuration Transfer IE containing the Inter-system SON Information Report IE, it may use it as specified in TS 38.300 [45] or in TS 36.300 [14].

If the eNB receives the Inter-system SON Information IE containing the Inter-system SON Information Reply IE, it may use it as specified in TS 38.300 [45] or in TS 36.300 [14].

If the eNB receives the SON Information IE containing the SON Information Request IE set to “Activate Muting”, the eNB should consider activating for over-the-air synchronisation by means of network listening, taking into account information on the selected source of synchronisation cell and the cells as indicated by the Aggressor E-CGI List IE. In case the Aggressor E-CGI List IE is not present, the eNB may consider the request applicable to all cells.

If the eNB receives the SON Information IE containing the SON Information Reply IE including the Muting Pattern Information IE as an answer to a former request, it may use it for over-the-air synchronisation by means of network listening. The Muting Pattern Information IE may apply to all cells that were requested to mute.

If the eNB receives the SON Information IE containing the SON Information Request IE set to “Deactivate Muting”, the eNB may consider deactivating muting for over-the-air synchronisation that was activated by a former muting request from the corresponding eNB.

Interaction with the eNB Configuration Transfer procedure:

If the eNB receives the SON Configuration Transfer IE containing the Requested TNL Information IE and the SON Information Request IE set to "X2 TNL Configuration Info" the eNB shall, if supported, take the PLMN ID contained in the Requested TNL Information IE into account and include X2 TNL configuration information within the eNB CONFIGURATION TRANSFER message associated with this PLMN.

<<<<<<<<<<<<<<<<<<<< for information only >>>>>>>>>>>>>>>>>>>>

### 9.1.16	eNB CONFIGURATION TRANSFER

This message is sent by the eNB in order to transfer RAN configuration information.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.2.3.26 |  | YES | ignore |
| EN-DC SON Configuration Transfer | O |  | 9.2.3.26a |  | YES | ignore |
| Inter-system SON Configuration Transfer | O |  | OCTET STRING | Contains the Inter-system SON Configuration Transfer IE as defined in TS 38.413 [44]. | YES | ignore |

### 9.1.17	MME CONFIGURATION TRANSFER

This message is sent by the MME in order to transfer RAN configuration information.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.2.3.26 |  | YES | ignore |
| EN-DC SON Configuration Transfer | O |  | 9.2.3.26a |  | YES | ignore |
| Inter-system SON Configuration Transfer | O |  | OCTET STRING | Contains the Inter-system SON Configuration Transfer IE as defined in TS 38.413 [44]. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.26	SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, and additionally includes the eNB identifier of the destination of this configuration information and the eNB identifier of the source of this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SON Configuration Transfer |  |  |  |  |  |  |
| >Target eNB-ID | M |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.1.37 |  |  |  |
| >>Selected TAI | M |  | TAI 9.2.3.16 |  |  |  |
| >Source eNB-ID | M |  |  |  |  |  |
| >>Global eNB ID | M |  | 9.2.1.37 |  |  |  |
| >>Selected TAI | M |  | TAI 9.2.3.16 |  |  |  |
| >SON Information | M |  | 9.2.3.27 |  |  |  |
| >X2 TNL Configuration Info | C-ifSONInformationRequest |  | 9.2.3.29 | Source eNB X2 TNL Configuration Info. | YES | ignore |
| >Synchronisation Information | C-if Activate Muting |  | 9.2.3.42 | Information on cell selected as source of synchronisation and aggressor cells. | YES | ignore |
| >Requested TNL Information | O |  | PLMN Identity 9.2.3.8 | Indicates to the candidate eNB the PLMN ID with which the X2 TNL Configuration Info IE is associated. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifSONInformationRequest | This IE shall be present if the SON Information IE contains the SON Information Request IE set to the value “X2TNL Configuration Info” |
| ifActivateMuting | This IE shall be present if the SON Information IE contains the SON Information Request IE set to the value “Activate Muting” |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.4	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RABInformationListItem,

id-E-RABItem,

id-GUMMEIType,

id-Bearers-SubjectToStatusTransfer-Item,

id-Time-Synchronisation-Info,

id-x2TNLConfigurationInfo,

id-eNBX2ExtendedTransportLayerAddresses,

id-MDTConfiguration,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-HO-Cause,

id-M3Configuration,

id-M4Configuration,

id-M5Configuration,

id-MDT-Location-Info,

id-SignallingBasedMDTPLMNList,

id-MobilityInformation,

id-ULCOUNTValueExtended,

id-DLCOUNTValueExtended,

id-ReceiveStatusOfULPDCPSDUsExtended,

id-eNBIndirectX2TransportLayerAddresses,

id-Muting-Availability-Indication,

id-Muting-Pattern-Information,

id-NRrestrictioninEPSasSecondaryRAT,

id-NRrestrictionin5GS,

id-Synchronisation-Information,

id-uE-HistoryInformationFromTheUE,

id-LoggedMBSFNMDT,

id-SON-Information-Report,

id-RecommendedCellItem,

id-RecommendedENBItem,

id-ProSeUEtoNetworkRelaying,

id-ULCOUNTValuePDCP-SNlength18,

id-DLCOUNTValuePDCP-SNlength18,

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

id-M6Configuration,

id-M7Configuration,

id-RAT-Type,

id-extended-e-RAB-MaximumBitrateDL,

id-extended-e-RAB-MaximumBitrateUL,

id-extended-e-RAB-GuaranteedBitrateDL,

id-extended-e-RAB-GuaranteedBitrateUL,

id-extended-uEaggregateMaximumBitRateDL,

id-extended-uEaggregateMaximumBitRateUL,

id-SecondaryRATDataUsageReportItem,

id-E-RABUsageReportItem,

id-UEAppLayerMeasConfig,

id-serviceType,

id-UnlicensedSpectrumRestriction,

id-CNTypeRestrictions,

id-DownlinkPacketLossRate,

id-UplinkPacketLossRate,

id-BluetoothMeasurementConfiguration,

id-WLANMeasurementConfiguration,

id-LastNG-RANPLMNIdentity,

id-PSCellInformation,

id-IMSvoiceEPSfallbackfrom5G,

id-RequestTypeAdditionalInfo,

id-AdditionalRRMPriorityIndex,

id-ContextatSource,

id-IntersystemMeasurementConfiguration,

id-SourceNodeID,

id-NB-IoT-RLF-Report-Container,

id-MDTConfigurationNR,

id-DAPSRequestInfo,

id-DAPSResponseInfoList,

id-DAPSResponseInfoItem,

id-Bearers-SubjectToEarlyStatusTransfer-Item,

id-TraceCollectionEntityURI,

id-EmergencyIndicator,

id-SourceTransportLayerAddress,

id-lastVisitedPSCellList,

id-RACSIndication,

id-SecurityIndication,

id-E-RABSecurityResultItem,

id-E-RABSecurityResultList,

id-RAT-Restrictions,

id-UEContextReferenceatSourceeNB,

id-LTE-NTN-TAI-Information,

id-SourceNodeTransportLayerAddress,

id-SourceSNID,

id-Direct-Forwarding-Path-Availability,

id-LoggedMDTTrigger,

id-SensorMeasurementConfiguration,

id-M4ReportAmount,

id-M5ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-TimeBasedHandoverInformation,

id-Bearers-SubjectToDLDiscarding-Item,

id-Bearers-SubjectToDLDiscardingList,

id-RequestedTNLInfo,

maxnoofCSGs,

maxnoofE-RABs,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

M1ReportingTrigger ::= ENUMERATED{

periodic,

a2eventtriggered,

...,

a2eventtriggered-periodic

}

RequestedTNLInfo ::= SEQUENCE {

pLMNidentity					PLMNidentity,

iE-Extensions					ProtocolExtensionContainer { { RequestedTNLInfo-ExtIEs} }	OPTIONAL,

...

}

RequestedTNLInfo-ExtIEs		S1AP-PROTOCOL-EXTENSION ::= {

...

}

RequestType	::= SEQUENCE {

eventType 				EventType,

reportArea 				ReportArea,

iE-Extensions			ProtocolExtensionContainer { { RequestType-ExtIEs} }	OPTIONAL,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SONConfigurationTransfer ::= SEQUENCE {

targeteNB-ID					TargeteNB-ID,

sourceeNB-ID					SourceeNB-ID,

sONInformation					SONInformation,

iE-Extensions			ProtocolExtensionContainer { { SONConfigurationTransfer-ExtIEs} }			OPTIONAL,

...

}

SONConfigurationTransfer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extension for Release 10 to transfer the IP addresses of the eNB initiating the ANR action --

{ID id-x2TNLConfigurationInfo		CRITICALITY ignore	EXTENSION X2TNLConfigurationInfo				PRESENCE conditional}|

-- The above IE shall be present if the SON Information IE contains the SON Information Request IE and the SON Information Request IE is set to the value "X2TNL Configuration Info"

-- Extension for Release 12 to transfer information concerning the source cell of synchronisation and the aggressor cell --

{ID id-Synchronisation-Information	CRITICALITY ignore	EXTENSION SynchronisationInformation			PRESENCE conditional}|

-- The above IE shall be present if the SON Information IE contains the SON Information Request IE set to the value " Activate Muting "

{ID id-RequestedTNLInfo				CRITICALITY ignore	EXTENSION RequestedTNLInfo						PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.6	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

S1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-Bearers-SubjectToDLDiscardingList				ProtocolIE-ID ::= 352

id-CoarseUELocationRequested						ProtocolIE-ID ::= 353

id-CoarseUELocation									ProtocolIE-ID ::= 354

id-RequestedTNLInfo									ProtocolIE-ID ::= 999 -- to be allocated

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>