3GPP RAN WG3 Meeting #127-bis	R3-251826

# Wuhan, China, 7th – 11th April 2025

Source:	CATT

Title:	Open issues on the CCO use case

Agenda Item:	11.3

Document for:	Discussion and decision

- Introduction

This document is mainly a revised one based on R3-250593. Some main differences include:

- Adding a section discussing whether to exchange time for predicted CCO issue over XnAP.

- Adding a section discussing the maximum value of time for predicted CCO issue and time for future CCO state

- Discussion

What a gNB-CU may do upon receiving Future Coverage Modification List

Figure 1: The step in question: what0 gNB2-CU shall do and send

There is some divergence on what a gNB-CU may do upon receiving a Future Coverage Modification List from its neighbour gNB. Two typical understanding includes:

- Alt 1: The gNB-CU forwards the cause of the received modification list toward the gNB-DU. Some other assistance information may be provided toward the gNB-DU simultaneously.

- Alt 2: The gNB-CU analyses (e.g. predicts) whether there will be any coverage or capacity problem after its neighbour implements the indicated modification, and if yes, where it will occur and what type it will be. The gNB-CU sends this analysis toward the corresponding gNB-DU(s).

There is a problem on Alt 1:

- In 38.401, for legacy CCO, it is stated “In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the CCO issue and the affected cells and beams” which implies that gNB-CU needs to detect the CCO issue based on current coverage status in the neighbourhood rather than just forwarding the “cause values” received from the neighbour CU.It is not aligned with the legacy CCO mechanism to include the Future Coverage Modification List from neighbour gNB into the “assistance information” provided from the gNB-CU toward the gNB-DU.

- But if the Future Coverage Modification List is not provided to the gNB-DU, it will be very hard for the gNB-DU to take any action—the neighbour node’s modification is yet to be performed, so the gNB-DU can only “guess” what the modification will be, in order to adjust its own cell correspondingly. Making the gNB-DU so intelligent contradicts our principle.

On the other hand, Alt 2 follows the principle that AI/ML function should be located at the gNB-CU, and reuses the same mechanism on F1AP as in the triggering gNB (i.e. low complexity on specification). Therefore, we prefer Alt 2.

Proposal 1: gNB2-CU predicts whether there will be any coverage or capacity problem after gNB1-CU implements the indicated modification, and if yes, sends the predicted CCO issue (which is predicted to occur after gNB1 adjusts its coverage) toward gNB2-DU.

Whether to support a gNB-CU provides to a served gNB-DU the suggested future coverage state of a served cell

Some companies proposed supporting that a gNB-CU provides to a served gNB-DU the suggested future coverage state of a served cell (i.e. not a neighbour cell). We are generally negative on this proposal, as we prefer the gNB-DU to determine what the CCO problem is or will be and what action to take, as in legacy CCO mechanism.

Proposal 2: Do not support the gNB-CU to provide to the gNB-DU the suggested future coverage state.

Node-level performance

Node-level performance information can be a type of feedback information, e.g. used as inputs in calculating the reward in the training phase or as monitor metrics. We consider it especially useful for the CCO use case as coverage adjustment may equally affects the UEs subject of handover and the UEs kept in the original cell. We propose discussing what Node-level performance report (e.g. what metric in TS 28.552) can be introduced for the CCO use case.

Proposal 3: To discuss what Node-level performance report (e.g. what metric in TS 28.552) can be introduced for the CCO use case.

A previously predicted issue is no longer predicted to occur

It is possible that a RAN node predicts that a coverage or capacity issue will occur somewhere, but later predicts that it will not occur. Every planned coverage modification should be cancelled then.

For indicating cancel of coverage modification itself, it is feasible to reuse the “Future Coverage Modification List”—which is already agreed to be introduced—to indicate no change by setting the “Future Cell/SSB Coverage State” of the concerned cell/SSB-area the same value as the current status.

And in gNB-CU/DU split scenario, there should be a step that the gNB-CU indicates the gNB-DU that the previously indicated issue is predicted not to occur. One simple solution is to clarify that the absence of “Predicted CCO issue” IE indicates so[2].

Proposal 4: To clarify that the absence of the “Predicted CCO issue” IE in a present “Predicted CCO Assistance Information” in F1AP indicates a previously predicted issue is no longer predicted to occur.

Whether to exchange time for predicted CCO issue over XnAP

The critical information for the receiving node is the time for the predicted future CCO state, as this enables the neighboring node to align its coverage configuration effectively. Once the gNB adjusts its coverage, the predicted CCO issue is resolved and will no longer occur. Given this dynamic, exchanging the time of the predicted CCO issue over the Xn interface becomes unnecessary. Therefore, the time for the predicted CCO issue does not need to be exchanged over the Xn interface.

Proposal 5: It is proposed that the time for the predicted CCO issue does not need to be exchanged over the Xn interface.

The maximum value of time for predicted CCO issue and time for future CCO state

Last RAN3 meeting discussed the maximum value of the time for predicted CCO issue and future coverage state, that is, 60s, 120s and 300s. Adopting a 60s or 90s value for CCO timing strikes a balance between accuracy and flexibility. In contrast, 300s is considered too long, as it risks reducing the accuracy of predictions and may not align with the practical needs of real-time network optimization. Therefore, we prefer 60s or 90s as the maximum value of time, but open to discuss on other options if justified.

Proposal 6: We prefer 60s or 90s as the maximum value of time, but open to discuss on other options if justified.

Miscellaneous

There is a few miscellaneous issues regarding the current version of F1AP BL CR:

The places of the future coverage modification list are different: It is directly within the XnAP NG-RAN NODE CONFIGURATION UPDATE message, but within the F1AP “Future Coverage Modification Notification” IE, which is a SEQUENCE ASN.1 type. It seems not needed to introduce such SEQUENCE ASN.1 type, so we prefer aligning F1AP with XnAP.

Proposal 7: To address the abovementioned miscellaneous issues. Discuss and agree the provided F1AP TP [2]

- Conclusion

Proposal 1: gNB2-CU predicts whether there will be any coverage or capacity problem after gNB1-CU implements the indicated modification, and if yes, sends the predicted CCO issue (which is predicted to occur after gNB1 adjusts its coverage) toward gNB2-DU.

Proposal 2: Do not support the gNB-CU to provide to the gNB-DU the suggested future coverage state.

Proposal 3: To discuss what Node-level performance report (e.g. what metric in TS 28.552) can be introduced for the CCO use case.

Proposal 4: To clarify that the absence of the “Predicted CCO issue” IE in a present “Predicted CCO Assistance Information” in F1AP indicates a previously predicted issue is no longer predicted to occur.

Proposal 5: It is proposed that the time for the predicted CCO issue does not need to be exchanged over the Xn interface.

Proposal 6: We prefer 60s or 90s as the maximum value of time, but open to discuss on other options if justified.

Proposal 7: To address the abovementioned miscellaneous issues. Discuss and agree the provided F1AP TP [2]

- Reference

- RAN3_127_agenda_20250221_1300_EOM

- R3-251825 (TP on 38.473) Open issues on the CCO