3GPP TSG-RAN WG3 #127bis	 										R3-251835

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	13.2

Source:	CATT

Title:	(TP for 38.473, 38.423) Discussion for security handling on non-DC case

Document for:	Decision

- Introduction

SA3 has given the feedback about LS [1][2]on the security handling for inter-CU LTM in non-DC cases, indicating the NCC shall be transmitted to the UE in plaintext via MAC CE message in REL-19. SA3 has requested RAN3 to discuss the details on key distribution and list the scenario where the serving gNB utilizes the unused NH to derive the KNG-RAN* for LTM handover. In this paper, we analyse the impact on RAN3 to support security handling for inter-CU LTM in non-DC cases.

SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

- Discussion

First we give a background about the security handling in legacy L3 handover.

Fig.1 illustrates the security interaction for a legacy L3 HO procedure. The S-gNB can derive the k*gNB according to the {NCC, NH}(vertical key derivation) pair or the kgNB (horizontal key derivation), which is depend on whether S-gNB have un-used {NCC, NH}. And then the S-gNB will send the derived k*gNB to the T-gNB, which will be used for UE access. The NCC value will also be contained in the RRCReconfiguration message to the UE, and the UE will derive k*gNB by the same way of network.

Fig.1 Security interaction for a legacy L3 HO (take vertical key derivation as example)

For the inter-CU LTM, we can divide it into two part, initial LTM and subsequent LTM.

- Initial LTM:

In the initial LTM, from the network perspective, S-CU can send the derived k*gNB via HO Request message when request C-gNB to provide candidate cells, which is aligned with legacy L3 HO and CHO. Form the UE perspective, UE can obtain the NCC value from the RRCReconfiguration(legacy way) or by the MAC CE, and this part should be decided by RAN2. If AN2 decided to use MAC CE, the S-CU shall transfer the NCC value to S-DU for generating MAC CE.

Proposal 1: In the initial LTM, the S-CU sends the derived k*gNB and NCC value via HO Request message. How the NCC value is transferred to UE is depends on RAN2.

Proposal 2: If AN2 decided to use MAC CE to transfer NCC value to UE in the initial LTM, the S-CU shall transfer the NCC value to S-DU by UE CONTEXT MODIFICATION REQUEST message.

- Subsequent LTM

Fig 2 shows the security interaction for inter-CU LTM. Similar, the K*gNB shall be derived by the S-gNB, either by vertical key derivation or horizontal key derivation, and then sends to candidate cells. There are two potential impacts for inter-CU LTM:

Fig.2 Security interaction for a legacy L3 HO (take vertical key derivation as example)

- Impact on XnAP:

For subsequent LTM, the new S-gNB needs to send the{K*gNB, NCC} to other candidate gNBs via LTM CONFIGURATION UPDATE message, and the candidate gNBs shall replace the previous {K*gNB, NCC} received from the last S-gNB.

- Impact on the F1AP:

The NCC is send to UE by DU in the MAC CE, thus, the S-CU shall transfer the NCC value to the S-DU for generate the MAC CE.

Proposal 3: For subsequent LTM, the new S-gNB needs to derived new K*gNB the sends the {K*gNB, NCC} to other candidate gNBs via LTM CONFIGURATION UPDATE message, the AS Security Information IE can be reused.

Proposal 4: For subsequent LTM, new S-CU shall transfer the NCC value to the new S-DU to generate the MAC CE.

The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

Another issue is listed by SA3 is about the key distribution between the source gNB and the target gNB, there may exist a confliction between PATH SWITCH from CN and CELL SWITCH COMMAND from DU.

- UE performs a first inter-gNB cell switch from gNB1 to gNB2

- gNB2 initiates a path switch procedure towards the CN

- While the path switch procedure is ongoing, DU of gNB2 trigger cell switch

In our understanding, this mismatch a corner case as CN will send the PATH SWITCH ACK in a very quickly manner. And when inter CU LTM is triggered, DU will trigger next inter CU cell switch after receive the new NCC from CU.

Proposal 5: When inter-CU LTM is triggered, DU will trigger next inter CU cell switch after receive the new NCC from CU.

Proposal 6: Draft reply LS provided in the R3-251836, discuss and agree the reply LS.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: In the initial LTM, the S-CU sends the derived k*gNB and NCC value via HO Request message. How the NCC value is transferred to UE is depends on RAN2.

Proposal 2: If AN2 decided to use MAC CE to transfer NCC value to UE in the initial LTM, the S-CU shall transfer the NCC value to S-DU by UE CONTEXT MODIFICATION REQUEST message.

Proposal 3: For subsequent LTM, the new S-gNB needs to derived new K*gNB the sends the {K*gNB, NCC} to other candidate gNBs via LTM CONFIGURATION UPDATE message, the AS Security Information IE can be reused.

Proposal 4: For subsequent LTM, new S-CU shall transfer the NCC value to the new S-DU to generate the MAC CE.

Proposal 5: When inter-CU LTM is triggered, DU will trigger next inter CU cell switch after receive the new NCC from CU.

Proposal 6: Draft reply LS provided in the R3-251836[3], discuss and agree the reply LS.

4. Reference

[1]	R2-2411136 Reply LS on security handling for inter-CU LTM in non-DC cases

[2]	S3-251124 Reply LS on security handling for inter-CU LTM in non-DC cases

[3]	R3-251836 Draft Reply LS to SA3 on the security handling for inter-CU LTM in non-DC case

TP for 38.473

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

/*********************Skipped***********************/

[48]	IEEE Std 1588: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems", Edition 2019.

[49]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[50]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[51]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

8.3.5	UE Context Modification Required (gNB-DU initiated)

8.3.5.1	General

The purpose of the UE Context Modification Required procedure is to modify the established UE Context, e.g., modifying and releasing radio bearer resources, or sidelink radio bearer resources or candidate cells in conditional handover, conditional PSCell addition, conditional PSCell change, or subsequent CPAC. The procedure uses UE-associated signalling.

8.3.5.2	Successful Operation

Figure 8.3.5.2-1: UE Context Modification Required procedure. Successful operation

The F1AP UE CONTEXT MODIFICATION REQUIRED message is initiated by the gNB-DU.

The gNB-CU reports the successful update of the UE context in the UE CONTEXT MODIFICATION CONFIRM message.

If the LTM Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the gNB-DU.

If the LTM Cells to be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the configured candidate cells in the list.

If the LTM Reset Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account for L2 reset (i.e., RLC re-establishment) during an intra-DU LTM cell switch as specified in TS38.331 [8].

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the Next Hop Chaining Count IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account for inter-CU LTM.

9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| Next Hop Chaining Count | O |  | INTEGER (0..7) | Next Hop Chaining Count (NCC) defined in TS 33.501 [51] | YES | reject |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |

TP for 38.423

8.x.1	LTM Configuration Update

Editor’s note: Details need to be continued.

8.x.1.1	General

The purpose of the LTM Configuration Update procedure is to update LTM configuration data, or to inform the change of serving NG-RAN node and setup the UE associated signalling between the new serving NG-RAN node and all other candidate NG-RAN nodes. The procedure uses UE associated signalling.

8.x.1.2	Successful Operation

Figure 8.x.1.2-1: LTM Configuration Update procedure. Successful operation

The NG-RAN node1 initiates the procedure by sending the LTM CONFIGURATION UPDATE message to NG-RAN node2.

The LTM CONFIGURATION UPDATE message may contain:

-	the LTM Information Request IE;

-	the Early Sync Information Request IE;

-	the Early Sync Information IE;

-	the Early Sync Information for SUL IE;

-	the LTM Information Request IE;

-    the Data Forwarding Info IE.

If the LTM Information Request IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider that the NG-RAN node1 has requested the LTM information update for the UE, include the LTM Information Response IE in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message and act as defined in TS 38.300 [9].

If the Reference Configuration IE is contained in the LTM Information Request IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, take it into account for generating the LTM configuration.

If the Request for CSI Resource Configuration IE is contained in the LTM Information Request IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, include the CSI Resource Configuration IE in the LTM Information Response IE in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message.

If the CSI Resource Configuration IE is contained in the LTM Information Request IE included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, use it to generate the LTM CSI reporting configuration included in the LTM-CandidateConfig IE for the serving cell.

If the LTM Configuration ID Mapping List IE is contained in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider that the NG-RAN node1 has requested the early synchronization information for the UE in the NG-RAN node2.

If the Early Sync Information Response IE is contained in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall, if supported, store it and use it as defined in TS 38.300 [9].

If the Early Sync Information IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider it as the consolidated early sync information to use for subsequent LTM.

If the Early Sync Information for SUL IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider it as the consolidated early sync information for SUL to use for subsequent LTM.

If the LTM Information Request IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider that the NG-RAN node1 has requested the LTM information update for the UE, include the LTM Information Response IE in the LTM CONFIGURATION UPDATE ACKNOWLEDGE message and act as defined in TS 38.300 [9].

(FFS if List is needed) If the Data Forwarding Info IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, use the information for forwarding data traffic to the NG-RAN node2.

If the AS Security Information IE is included in the LTM CONFIGURATION UPDATE message, the NG-RAN node2 shall, store and replace the previously provided AS Security Information.

9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |
| Early Sync Information Request | O |  | 9.2.1.xx3 |  | YES | ignore |
| Early Sync Information | O |  | 9.2.1.xx6 |  | YES | ignore |
| Early Sync Information for SUL | O |  | 9.2.1.xx6 |  | YES | ignore |
| Data Forwarding Info | FFS if List is needed |  | 9.2.1.16 |  | YES | ignore |
| AS Security Information | O |  | 9.2.3.50 |  | – |  |