3GPP TSG-RAN WG3 Meeting #127bis	R3-251856

Wuhan, China, 7-11 April, 2025

Agenda item:	21.3

Source: 	Huawei

Title: 	Discussion on multi-modality awareness and UL rate control

Document for:	Discussion and Decision

# 1. Introduction

The Rel-19 XR Phase 3 objectives are as follows:

-	Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]:

….

-	Specify uplink congestion signalling [RAN2, RAN3]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

According to SA#106 meeting, SA has reached consensus that the multi-modality awareness at NG-RAN can be achieved by the CN providing the MMSID [1]. In RAN #107 meeting, the WID of R19 XR was updated in [2] to include the following object for the multi-modality awareness and the UL rate control:

In this contribution, we will analyse the impacts on RAN3 specifications to transfer MMSID from CN to NG-RAN to support the multi-modality awareness.

Besides, RAN2 has agreed to support a rate indication from the gNB to the UE on a per QoS flow level, and RAN3 was added for this task. We will also provide our views on the RAN3 impacts to support UL rate control.

# 2. Discussion

## 2.1 Multi-modality awareness

To enable the multi-modality awareness, RAN3 should consider the specific impacts on NG, Xn, F1 and E1 interfaces.

- Impacts on NGAP

First of all, to obtain the multi-modality information from CN, N2 signaling should be enhanced to support the CN providing MMSID during the PDU Session Resource Setup and PDU Session Resource Modify procedures. Besides, in case of NG-based handover, the MMSID may be provided by the CN to the target network, thus the Handover Resource Allocation procedure should also be considered.

Since the MMSID is associated with each data flow, it is suitable to include it in the QoS Flow Level QoS Parameters IE, which is sent from the CN to the RAN during the above mentioned procedures. A new IE for MMSID should be introduced, its definition and format can refer to the R18 MMSID defined by SA, as specified in TS 29.514 [3].

- Impacts on XnAP

Considering the Xn interface, the cases of Xn-based handover and DC should be taken into account. During handover, the source gNB may provide the MMSID to the target gNB so that the target gNB can perform joint admission control, which means the MMSID should be included in the HANDOVER REQUEST message. For DC case, the MN may need to provide the MMSID to the SN during the SN addition procedure and SN modification procedure to allow the SN to take the information into account.

Similar to NGAP, the MMSID can be added into the QoS Flow Level QoS Parameters IE so that it can be transferred associated with the QoS flow within the handover and DC related massages.

- Impacts on F1AP

For the scenario of CU-DU split, the MMSID can be further transferred to the DU so that the DU can utilize the information when performing scheduling or resource allocation. For example, the DU may reserve sufficient resource for the QoS flows with the same MMSID so that the service can be satisfied.

The MMSID can be carried in the QoS Flow Level QoS Parameters IE within the UE context setup and modification messages.

- Impacts on E1AP

If the NG-RAN node using disaggregated architecture with CU-CP and CU-UP split, it would be needed to transfer the MMSID over the E1 interface so that the CU-UP can take the information into account. Similarly, the CU-CP can provide the MMSID during bearer context setup and bearer context modification procedures by including MMSID in the QoS Flow Level QoS Parameters IE.

Proposal 1: Add a new IE for MMSID in the QoS Flow Level QoS Parameters IE in NGAP, XnAP, E1AP and F1AP specifications.

## 2.2 UL rate control

2.2.1  Feasibility of QoS flow level UL rate control

During RAN2#128 meeting, RAN2 agreed to support per QoS flow level UL rate control. To provide the indication to the UE, the gNB should estimate the recommended bit rate for the QoS flow. In case of CU-DU split, it should be the gNB-DU to be responsible to estimate and provide the recommended bit rate.

From RAN3 perspective, it is feasible and up to implementation for the gNB-DU to decide the recommended bit rate for a QoS flow. It is noted that F1AP is already able to transfer the QoS flow level QoS parameters to the gNB-DU. So, as one possible implementation, the gNB-DU can derive the available rate for each QoS flow based on the available resource and the QoS requirements. For example, assuming there are two QoS flows with their GFBR equal to 10Mbps (QoS flow 1) and 20Mbps (QoS flow 2) respectively, if the gNB-DU can provide up to 45Mbps for the two QoS flows in total, then it can determine the recommended bit rate as 15Mbps for QoS flow 1 and 30Mbps for QoS flow 2.

Another potential implementation is that the gNB-DU may monitor the actual bit rate of each QoS flow by taking statistics of the received DL or UL packets. Specifically, if there is only one QoS flow mapping to the DRB, the gNB-DU can measure the data rate for the DRB, which equals to the rate of the QoS flow. If there are multiple QoS flows mapping to the DRB, the gNB-DU can distinguish the packets belong to different QoS flows via the SDAP header (for DL/UL) so that the rate of each QoS flow can be measured. It is worth noting that the CU can configure the DL/UL SDAP header to be included if N to 1 mapping will be performed for the QoS flow which supports bit rate control, and the SDAP header will not be ciphered in PDCP layer as specified in TS 38.323 [4], as excerpted below. In fact, for UL data transmission, if one DRB is used to carry data from multiple QoS flows, the SDAP header must be added in the UL packets by the UE, considering that the gNB should add the QFI in the NG-U header when forwarding the packets to UPF. With monitoring the actual rate for the QoS flows, the gNB-DU can determine the recommended rate for each QoS flow by taking the available resource, measured date rate per QoS flow and the QoS requirement of each Qos flow into account.

## 5.8 Ciphering and deciphering The ciphering function includes both ciphering and deciphering and is performed in PDCP, if configured. The data unit that is ciphered is the MAC-I (see clause 6.3.4) and the data part of the PDCP Data PDU (see clause 6.3.3) except the SDAP header and the SDAP Control PDU if included in the PDCP SDU. The ciphering is not applicable to PDCP Control PDUs.

Observation 1: It is feasible for the gNB-DU to provide the QoS flow level recommended bit rate to UE.

Both the above implementations have no specification impact. RAN3 can confirm that even for the disaggregated RAN architecture, it is feasible for the gNB-DU to provide the QoS flow level recommended bit rate to UE, regardless of how the QoS flows and the DRBs are mapped. But how will the gNB-DU determine the value for each QoS flow level recommended bit rate should be up to implementation, no specification effort is needed.

The following two options considered by RAN2 to implement the QoS flow level rate control

Option 1: Use MAC CE to indicate the QoS flow and the corresponding recommended rate directly.

Option 2: Use RRC signalling to indicate the QoS flows for rate control, and use MAC CE to indicate per DRB recommended rate. How to divide the rate among multiple QoS flows is up to UE implementation.

Since the gNB-DU is capable to determine per QoS flow recommended rate, it is efficient and intuitive to indicate QoS flow level recommended rate in the MAC CE directly. Moreover, regarding the potential specification impacts, both options have similar impacts from RAN3 perspective. Specifically, the gNB-DU has to be aware of which QoS flows are subject to perform rate control from gNB-CU, so that it can decide for which QoS flows (for option 1) or DRBs (for option 2) the recommended bit rate should be evaluated. So, the F1AP impacts for the two options are exactly similar.

Observation 2: Both options have the similar F1AP impacts to implement UL rate control.

With the similar specification impacts, option 1 can provide more accurate QoS flow level UL rate control, while in option 2, the NW is actually unable to control the adjusted bit rate as expected for a specific QoS flow when there are multiple QoS flows configured with UL rate control being mapped to the DRB. Based on the above analysis and observations, option 1 should be preferred to provide the UE with more accurate recommended bit rate.

Proposal 2: In case of CU-DU split architecture, the gNB-DU provides QoS flow level recommended bit rate to UE directly. How to derive the recommended bit rate is up to implementation.

2.2.2  Discussion on RAN2’s LS R2-2501347/ R3-251513

In RAN2 #129 meeting, RAN2 also agreed that the gNB can obtain the information about “which QoS flows can be throttled” from the CN. An LS was sent to RAN3 in R2-2501347/R3-251513 [5], as excerpted below.

1. Overall Description: For XR rate control, RAN2 discussed how the gNB gets the information of QoS flows which are subject to uplink rate control, i.e., whether the information is from CN or UE, and agreed that: - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. 2. Actions: To SA2, RAN3. ACTION: RAN2 respectfully asks SA2 and RAN3 to take the above decision into consideration for further work on XR uplink rate control and provide feedback.

We understand that such information is used to inform the gNB about which QoS flows can be applicable for UL rate control, considering that some XR services may lack the capability to adjust the codecs to mitigate the network congestion [6]. So it is essential to allow the gNB to identify which QoS flows can dynamically adjust their bit rate.

From RAN3 point of view, such information can be transferred though NGAP. For example, an indicator indicating whether the QoS flow is subject to bit rate control can be carried along with the QoS flow level QoS parameters. RAN3 can reply to RAN2 to confirm the feasibility on transferring the above information from CN to RAN. How to obtain or determine the information would be up to SA2. We provide a draft reply LS in the annex part.

Proposal 3: RAN3 confirms the feasibility of NGAP update to support that the CN provides information to RAN on which QoS flow are subject to bit rate control.

After receiving the information from CN, the gNB can decide to apply UL rate control to which QoS flows. The gNB may indicate which QoS flows are considered for UL rate control to the UE via the RRC configuration procedure. The configuration is probably generated and sent by the gNB-CU in case of CU-DU split. While the subsequent bit rate recommendation is evaluated and performed by the gNB-DU, the gNB-DU should first be aware of which QoS flows are considered for UL rate control.

To facilitate the mechanism, the gNB-CU should inform the gNB-DU about which QoS flows require UL bit rate control. This information can be provided via F1AP, e.g. within the UE context setup and modification messages.

Proposal 4: In case of CU-DU split architecture, the gNB-CU informs the gNB-DU on which QoS flows require UL bit rate control.

# 3. Conclusion

In this contribution, we analyzed the specification impacts to support multi-modality awareness and UL rate control. The following observations and proposals were made:

Observation 1: It is feasible for the gNB-DU to provide the QoS flow level recommended bit rate to UE.

Observation 2: Both options have the similar RAN3 impacts to implement UL rate control.

Proposal 1: Add a new IE for MMSID in the QoS Flow Level QoS Parameters IE in NGAP, XnAP, E1AP and F1AP specifications.

Proposal 2: In case of CU-DU split architecture, the gNB-DU provides QoS flow level recommended bit rate to UE directly. How to derive the recommended bit rate is up to implementation.

Proposal 3: RAN3 confirms the feasibility of NGAP update to support that the CN provides information to RAN on which QoS flow are subject to bit rate control.

Proposal 4: In case of CU-DU split architecture, the gNB-CU informs the gNB-DU on which QoS flows require UL bit rate control.

# 4. Reference

[1]. SP-241921, TSG SA, LS on multi-modality awareness.

[2]. RP-250107, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur). 3GPP TSG RAN Meeting #107

[3]. 3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[4]. 3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification ".

[5]. R2-2501347, 3GPP TSG RAN WG2, LS on uplink rate control.

[6]. R2-2501202, Nokia, Nokia Shanghai Bell, XR rate control.

# Annex A. Draft reply LS on UL rate control

Title:	[Draft] Reply LS on XR UL bit rate control

Response to:	R2-2411218/ R3-251513

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	Huawei [To be RAN3]

To:	RAN2

Cc: 	SA2

# Contact Person:

# Name:

# Email:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

RAN3 thanks RAN2 for the information on UL bit rate control for XR. RAN3 has the following feedback considering RAN2 agreements:

From RAN3 perspective, it is feasible to enhance NGAP to support that CN providing the information on which QoS flows are subject to uplink rate control to NG-RAN. In addition, in case of CU-DU split at NG-RAN, the gNB-CU will inform the gNB-DU on which QoS flows require UL bit rate control .

# 2. Actions:

# To RAN2:

ACTION: RAN3 kindly asks RAN2 to take the above feedback into consideration.

# 3. Date of Next RAN3 Meetings:

RAN3#128	19th May – 23rd May 2025					Malta, MT

RAN3#129	25th August – 29th August 2025				Bangalore, India