3GPP TSG RAN WG3#127bis	R3-251861

Wuhan, China, 07 – 11 April 2025

Agenda item:	12.3

Source: 	China Telecom

Title: 	On remaining issues for NR Femto

Document for:	Discussion and decision

- Introduction

In RAN3#127, some agreements were made as following:

Considering that NAT is an IP router functionality, and that IP routers are part of the transport network, NAT does not need to be mentioned in the stage 2 description of the NR Femto GW; the FFS below is thus resolved. “In case of user plane transport concentration at the Femto GW, the Femto GW takes the role described in Option 3 (routing at the IP). FFS whether NAT support.” To avoid routing ambiguities, a TAI used in a NR Femto GW shall not be reused in another NR Femto GW. Reuse the Global gNB ID to identify the NR femto node.

Some TPs i.e. R3-250817, R3-250821 and R3-250822 were agreed.

In this paper, we discuss some remaining issues and present our proposals.

- Discussion

2.1 Selection of an IP version for NG-U

In last meeting, how to select an IP version for NG-U was discussed, the remaining issue was noted as following:

The Femto GW hosts the following function:

-	Selection of an IP version to be used for NG-U, if a NG-U UP transport layer information configuration contains two transport layer addresses of different versions.

To be continued…

In LTE, MME selects IP version for S1-U according to the IP version of S1-C, meaning the same IP version for both S1-U and S1-C.

If the HeNB GW is deployed, as the S1-C of HeNB is terminated at HeNB GW, so MME was supposed to never know the IP version supported by the HeNB. As a result, if a HeNB is connected with MME directly, the MME cannot determine the IP version supported by the HeNB and allocate appropriate IP version for UP of it. For example, if the MME assigns an IPv6 address while the HeNB only supports IPv4, the IP tunnel between the HeNB and MME will fail to establish successfully.

In this case, the solution is “The MME may send two transport layer addresses of different versions only in case of HeNB GW which does not terminate user plane.” When the HeNB GW received the two different version addresses via S1-C, HeNB GW selects an IP version address which is suitable for the HeNB to help it establish the S1-U connection with the MME according to the IP version of the S1-C. Then HeNB uses the IP version selected by HeNB GW to connect with MME directly.

Now for NR Femto, we think the same issue exists and the similar mechanism can be applied for Femto GW and AMF.

Proposal 1: The similar mechanisms of IP selection for LTE HeNB can be reused for NR Femto, i.e. the AMF sends both IPv6 and IPv4 addresses in case of Femto GW which does not terminate user plane, and the Femto GW is responsible for selection of an IP address to be used for UP connectivity.

2.2 NR Femto cell shared by both PLMN and PNI-NPN

In the recent meeting, SA2 sent an LS [1] to RAN3 seeking clarification regarding the following statement in the RAN3 BL CR for NR Femto:

"The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in SIB1, but without the cellReservedForOtherUse indication."

SA2 would like to understand whether this implementation approach contradicts any existing SA2 specifications regarding cell sharing between PLMN and PNI-NPN networks.

From a RAN perspective, in the case of RAN sharing, a cell shared by both a PLMN and a PNI-NPN refers to a single physical cell serving both normal UEs and CAG UEs -- not a logical cell. However, this shared physical cell will have distinct logical cell IDs (i.e., NR Cell Identities) corresponding to the PLMN and PNI-NPN, respectively. This aligns with the statement in TS 38.300 [2]: "A Cell Identity can only belong to one network type among PLMN, PNI-NPN, or SNPN."

When a UE accesses an NR Femto shared cell, it includes the selected PLMN ID or PNI-NPN ID in the RRCSetupComplete message sent to the NR Femto node. The NR Femto then includes the NPN Access Information IE and User Location Information IE in the INITIAL UE MESSAGE forwarded to the AMF. Based on the logical NR CGI (within User Location Information) and NPN Access Information, the AMF can determine whether the UE is a normal UE or a CAG UE and apply the appropriate access control.

Observation 1: As per 3GPP RAN specifications, a Cell Identity can only belong to one network type (PLMN, PNI-NPN, or SNPN). Therefore, PLMN and PNI-NPN must be associated with different Cell Identities.

Proposal 2: Send an LS to SA2 confirming that the second option in the RAN3 BL CR does not conflict with existing SA2 specifications.

2.3 Mobility procedures related to NR Femto GW

When examining the LTE Femto-related content in TS 36.300[1], we find detailed descriptions of mobility procedures to/from CSG (Closed Subscriber Group) and Hybrid cells, including the role of the HeNB Gateway (HeNB GW). However, TS 38.300 currently lacks equivalent content for NR Femto, particularly regarding the NR Femto GW's role in mobility procedures.

While the PNI-NPN (Public Network Integrated Non-Public Network) section describes CAG (Closed Access Group) UE mobility in connected mode [5], it lacks the critical aspect of NR Femto Gateway involvement. This gap results in an incomplete specification of NR Femto Gateway functionality. This content should be supplemented.

Observation 2: The current specification does not address the NR Femto Gateway’s role in mobility procedures.

Proposal 3: The role of NR Femto GW in mobility procedures should be clarified and updated in TS38.300.

- Conclusion

Based on the above discussion on the remaining issues for NR Femto, we provide the following observations and proposals:

Proposal 1: The similar mechanisms of IP selection for LTE HeNB can be reused for NR Femto, i.e. the AMF sends both IPv6 and IPv4 addresses in case of Femto GW which does not terminate user plane, and the Femto GW is responsible for selection of an IP address to be used for UP connectivity.

Observation 1: As per 3GPP RAN specifications, a Cell Identity can only belong to one network type (PLMN, PNI-NPN, or SNPN). Therefore, PLMN and PNI-NPN must be associated with different Cell Identities.

Proposal 2: Send an LS to SA2 confirming that the second option in the RAN3 BL CR does not conflict with existing SA2 specifications.

Observation 2: The current specification does not address the NR Femto Gateway’s role in mobility procedures.

Proposal 3: The role of NR Femto GW in mobility procedures should be clarified and updated in TS38.300.

- Reference

[1] R3-251524 LS on NR Femto node shared by PLMN and PNI-NPN, SA2.

[2] TS 38.331, NR; Radio Resource Control (RRC) protocol specification Rel-18, 3GPP.

[3] RP-242395, New WID on additional topological enhancements for NR, NTT DOCOMO, AT&T.

[4] RP-241925, TR 38.799 v2.1.0 NR; Study on additional topological enhancements for NR, NTT DOCOMO INC., AT&T.

[5] TS 38.300, NR; NR and NG-RAN Overall description; Stage-2 Rel-18, 3GPP.

[6] TS 38.413, NG-RAN; NG Application Protocol (NGAP) Rel-18, 3GPP.

[7] TS 36.300, E-UTRAN; Overall description; Stage 2 Rel-18, 3GPP.

[8] TS 36.413, E-UTRAN; S1 Application Protocol (S1AP) Rel-18, 3GPP.