3GPP TSG-RAN WG3 Meeting #127-bis	R3-251869

Wuhan, China, 7th - 11th April 2025

Title:	Discussion on AIML based Coverage and Capacity Optimization

Source:				China Telecom

Agenda item:		11.3

Document for:		Discussion and Decision

1. Introduction

In the last meetings, the following agreements were made with several FFSs [3].

Each Future Coverage Modification Notification Item included in the Future Coverage Modification Notification List IE has a Future Coverage Modification Cause IE associated to it from DU to CU. The Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only. Value ‘0’ for the Future Cell Coverage State IE is needed and has the same meaning as cell inactive in legacy CCO. UE performance feedback for CCO? Legacy UE performance feedback is sufficient or other finer granularity, e.g., cell-/SSB-level UE performance feedback? Whether gNB-CU provides to the gNB-DU not only the predicted CCO issue but also the corresponding future CCO state as a recommendation? Whether to exchange time for predicted CCO issue over XnAP needs further discussion. The maximum value of time for predicted CCO issue and time for future CCO state needs further discussion.

In this document, we continue discussing the following FFS aspects:

Whether to exchange time for predicted CCO issue over XnAP needs further discussion.

The maximum value of time for predicted CCO issue and time for future CCO state needs further discussion.

2. Discussion

2.1 Timing information for AI/ML-based CCO

In RAN3#125bis meeting, it was agreed to introduce separate time information for predicted CCO issue and future CCO state as follows [1]:

Introducing separate time information for future coverage state and predicted CCO issue. Time for future coverage state: The point in time when the future coverage state will be applied. Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

In RAN3#126 and RAN3#127 meeting, RAN3 reached the following agreements with several FFSs [2][3]:

RAN3#126: WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE. Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1. Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node Configuration Update message over Xn. Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds. FFS on whether to send time information related to coverage modification cause over Xn. RAN3#127: Whether to exchange time for predicted CCO issue over XnAP needs further discussion. The maximum value of time for predicted CCO issue and time for future CCO state needs further discussion.

RAN3 agreed to transfer the Time for Future Coverage Modification IE (i.e. time information for future CCO state) within the NG-RAN CONFIGURATION UPDATE message [4]. However, whether to send time information for the coverage modification cause which reflect the predicted CCO issue over Xn needs further discussion.

From our side, providing time information for predicted CCO issues over Xn is essential for proactive network management. For example, consider a scenario where gNB1 predicts a capacity overload issue at 5 PM, with offloading to gNB2 starting at 4:30 PM. Knowing 5 PM, gNB2 can ensure it has sufficient resources ready at 5 PM to handle the offloaded users, especially if the offloading process takes time to complete and some strain might still occur at 5 PM if not fully mitigated. If gNB2 only knows the future CCO state will be applied at 4:30 PM, it might assume that the issue is fully handled by then, but if the offloading isn't complete and the issue still occurs at 5 PM, gNB2 might be unprepared for residual effects, risking degraded service.

Proposal 1: RAN3 to agree that timing information for the predicted CCO issue can be transferred over Xn.

Proposal 2: Change the agreement “Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1” to “Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1 and NG-RAN node Configuration Updata message over Xn.”

Proposal 3: Introduce the Time for coverage modification cause IE within the NG-RAN CONFIGURATION UPDATE message.

In RAN3#126 meeting [2], it was agreed that the time for predicted CCO issue and time for future coverage state can be in the granularity of seconds. In RAN3#127 meeting [3], there were several proposals (i.e. 60s, 120s, 300s) provided for the range of the time for predicted CCO issue and time for future coverage state. From our side, these two pieces of information can be encoded as follows:

- Time for predicted CCO issue: INTEGER (1..60, …), with granularity of seconds

- Time for future coverage state: INTEGER (1..60, …), with granularity of seconds

These two IEs above can be extended in the future if needed.

Proposal 4: RAN3 to agree that the Time for Predicted CCO Issue and the Time for Future Coverage Modification are encoded as INTERGER (1..60, …) with granularity of seconds. It can be extended in the future if needed.

3. Conclusion

In this contribution, we further discuss AI/ML based Coverage and Capacity Optimization (CCO). We have the following proposal:

Proposal 1: RAN3 to agree that timing information for the predicted CCO issue can be transferred over Xn.

Proposal 2: Change the agreement “Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1” to “Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1 and NG-RAN node Configuration Updata message over Xn.”

Proposal 3: Introduce the Time for coverage modification cause IE within the NG-RAN CONFIGURATION UPDATE message.

Proposal 4: RAN3 to agree that the Time for Predicted CCO Issue and the Time for Future Coverage Modification are encoded as INTERGER (1..60, …) with granularity of seconds. It can be extended in the future if needed.

4. Reference

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- R3-247928, (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE