3GPP TSG-RAN WG3 Meeting #127bis	R3-251887

Wuhan, China, 7th - 11th Apr 2025

Agenda Item:	8.3

Source:	ZTE Corporation

Title:	Discussion on energy saving indicator from CN to RAN

Document for:	Discussion

- Introduction

In last meeting, RAN3 has discussed the LS from SA2 on the energy saving indicator[1], i.e., whether to introduce the energy saving indicator from CN to RAN in this release. While, some companies had concern on the usage of this indication, which is not clear. And there is no agreement reached. The conclusion in last meeting on this issue is given as below[2].

There is no consensus in RAN3 on the usage of this energy saving indication in this meeting, making the decision in next meeting and provide reply LS to SA2. To be discussed in AI8.3 in RAN3#127bis.

In this contribution, we provide our further view on the energy saving indicator from CN.

2	Discussion

In NR, RAN has been developing the NES and UE power-saving functions since Rel-15. The following table lists the main features and our perspective on whether a UE-associated ES indication (as mentioned in the SA2 LS) is needed.

| Release | ES feature | Strategy Level | Is UE-Associated ES Indication Needed? |
| --- | --- | --- | --- |
| Rel-15 | NES: Intra-system Cell Switch on/off | Cell level | Not needed, as it is a cell-level ES strategy. |
| Rel-17 | NES: Inter-system inter-RAT Cell Switch on/off | Cell level | Not needed, as it is a cell-level ES strategy. |
|  | UE power saving: UE paging subgroup | UE associated | Not needed, but indicated by UE capability and does not impact UE performance. |
| Rel-18 | NES: cell DTX/DRX | Cell level | Not needed, as it is a cell-level ES strategy. |
|  | NES: Inter-node beam activation | Cell level | Not needed, as it is a cell-level ES strategy. |
|  | NES: Paging UE restricting in a list visited SSB. | UE associated | Not needed, as it only applies to stationary UEs and does not impact UE performance.. |
| Rel-19 | NES: OD-SSB Scell operation | UE associated | Not needed, as Scell activation/deactivation is based on the UE's traffic characteristics and does not impact UE performance |
|  | NES: OD-SIB1 | Cell level | Not needed, as it is a cell-level ES strategy. |
|  | NES; others, e.g, paging adaption | Cell level or UE associated | Not needed, as it is a cell-level ES strategy or does not impact UE performance. |

From our perspective, a UE-associated ES indication is only necessary when an ES function on the network side may impact UE performance, requiring the UE’s explicit consent before the network can proceed with the ES function.For example, to optimize energy efficiency, operators may choose to limit the network's energy consumption when serving a particular user. This could involve agreements between operators and users or vertical industries on various factors, such as different levels of energy savings, service quality, energy sources (e.g., whether green energy is permitted), and pricing standards. While network delivering services to UE, it must obtain explicit user consent regarding the acceptable level of energy reduction and service quality (SLA). This ensures a balance between network energy efficiency and user experience.

Proposal 1: A UE-associated ES indication is only necessary when an ES function on the network side may degrade UE performance, requiring the UE's explicit consent before the network can proceed with the ES function.

Based on the RAN-developed ES features from Rel-15, some are cell-level ES strategies, where explicit UE consent for energy saving is not required. Others are UE-associated ES strategies, which have already been evaluated without degrading UE performance. The use of these developed ES features depends on the RAN implementation, and there is no need to require UE consent for energy saving. In addition, based on the attached SA2’s CR[3], whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN is still FFS. Therefore, from the prospective of RAN, it is useless to introduce this indicator in current version unless SA2 or RAN2 provides more clarification on the usage.

Proposal 2: Based on the RAN-developed ES features from Rel-15, some are cell-level ES strategies, while others are UE-associated ES strategies that have been evaluated without degrading UE performance. The use of these developed ES features depends on the RAN implementation, and introducing a UE-associated ES indication would be unnecessary.

Back to what we mentioned in Proposal 1, a UE-associated ES indication may be necessary when an ES function on the network side could impact UE performance. For example, operators may choose to limit the network's energy consumption when serving a particular user with an alternative SLA degradation. We assume this indicator may be needed if it aims to support per-UE energy consumption in the RAN. However, the approved reply LS from RAN plenary in RAN#103 meeting [4] has already clarified that RAN does not expect to support UE-level energy consumption measurement in R19 and no further action is required from RAN WGs.

Proposal 3: RAN does not expect to support UE-level energy consumption measurement in Rel-19, therefore, an indicator aimed at supporting per-UE energy consumption is still not needed in Rel-19.

Based on the above analysis, there is no need to introduce the energy saving indication from CN to RAN in Rel-19.

Proposal 4: There is no need to introduce the energy saving indication from CN to RAN in Rel-19, and RAN3 sends the reply LS to SA2 on this issue.

3	Summary

Proposal 1: A UE-associated ES indication is only necessary when an ES function on the network side may degrade UE performance, requiring the UE's explicit consent before the network can proceed with the ES function.

Proposal 2: Based on the RAN-developed ES features from Rel-15, some are cell-level ES strategies, while others are UE-associated ES strategies that have been evaluated without degrading UE performance. The use of these developed ES features depends on the RAN implementation, and introducing a UE-associated ES indication would be unnecessary.

Proposal 3: RAN does not expect to support UE-level energy consumption measurement in Rel-19, therefore, an indicator aimed at supporting per-UE energy consumption is still not needed in Rel-19.

Proposal 4: There is no need to introduce the energy saving indication from CN to RAN in Rel-19, and RAN3 sends the reply LS to SA2 on this issue.

The draft reply LS is given in [5].

4	Reference

# [1] R3-250024 (R2-2413034), LS on energy saving indication from CN to RAN, SA2

# [2] RAN3#127 Chairman Notes

# [3] S2-2413019, KI#2: UE subscription and policy control for energy efficiency and energy saving, Lenovo

# [4] RP-240825, Reply LS RP-240029 on per UE energy consumption in RAN, RAN

[5] R3-251888,  [Draft] Reply LS on energy saving indication from CN to RAN, ZTE