3GPP TSG-RAN WG3 Meeting #127	R3-251890

Athens, Greece, 17th-21st Feb 2025

Agenda Item:	14.2

Source:	ZTE Corporation

Title:	Further Discussion on NTN broadcast service supporting

Document for:	Discussion & Approval

- Introduction

In previous meetings, the support of NTN MBS broadcast service was discussed, and the following agreements were reached.

RAN3 starts work on MBS broadcast service. WA: The mapped cell ID can be used to define MBS broadcast service in NTN. Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN. RAN3 takes the assumption that geographical service area information can be transmitted from CN with the same format as defined in Uu. The current MBS broadcast procedures can be reused for NR NTN without any enhancement. Encode the Intended MBS service area as OCT STRING which refers to RAN2 spec.

In this contribution, we provide our further views on the support of broadcast service for NTN in Rel-19.

- Discussion

In the previous meetings, it was agreed that the current MBS broadcast procedures can be reused for NR NTN without any enhancement, and the encoding of the MBS intended service area info should refers to RAN2 specification. Based on the progress so far, the MBS broadcast service has been supported for NTN, and there is no explicit work to be done for this feature.

However, based on the objective of R19 NR NTN WI, the signaling of intended service area is for a broadcast service, not only for the MBS broadcast. Therefore, the other broadcast services could be also considered, such as the PWS service.

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

As mentioned above, since the MBS broadcast service has already made some progress, and not much work on this feature is foreseen, there is still some time for our RAN3 to study whether to introduce intended service area for PWS service.

Since Rel-17, it has been already agreed that cell ID used for PWS in NTN field corresponds to a mapped cell ID. Currently, in TS 38.413, the Warning Area List IE is contained in the PWS related messages(e.g., WRITE-REPLACE WARNING REQUEST, PWS CANCEL REQUEST), as given below.

- ---------------------------------------------------------TS 38.413------------------------------------------------------------------------

- 9.3.1.37	Warning Area List

This IE indicates the areas where the warning message needs to be broadcast or cancelled.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Warning Area | M |  |  |  |
| >E-UTRA Cell IDs |  |  |  |  |
| >>EUTRA CGI List for Warning |  | 1..<maxnoofCellIDforWarning> |  |  |
| >>>E-UTRA CGI | M |  | 9.3.1.9 |  |
| >NR Cell IDs |  |  |  |  |
| >>NR CGI List for Warning |  | 1..<maxnoofCellIDforWarning> |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  |
| >TAIs for Warning |  |  |  |  |
| >>TAI List for Warning |  | 1..<maxnoofTAIforWarning> |  |  |
| >>>TAI | M |  | 9.3.3.11 |  |
| >Emergency Area IDs |  |  |  |  |
| >>Emergency Area ID List |  | 1..<maxnoofEmergencyAreaID> |  |  |
| >>>Emergency Area ID | M |  | 9.3.1.48 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforWarning | Maximum no. of Cell ID subject for warning message broadcast. Value is 65535. |
| maxnoofTAIforWarning | Maximum no. of TAI subject for warning message broadcast. Value is 65535. |
| maxnoofEmergencyAreaID | Maximum no. of Emergency Area ID subject for warning message broadcast. Value is 65535. |

9.3.1.48	Emergency Area ID

This IE is used to indicate the area which has the emergency impact.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Emergency Area ID | M |  | OCTET STRING (SIZE(3)) | Emergency Area ID may consist of several cells. Emergency Area ID is defined by the operator. |

- ---------------------------------------------------------TS 38.413------------------------------------------------------------------------

Based on the current specification, in addition to the (Mapped) Cell ID and TAI, the Emergency Area ID can be also used for indicating the warning area. However, as given in the corresponding semantics description, the Emergency Area ID may consist of several cells, which cannot represent specific geographical area as agreed in the MBS broadcast service.

Generally, there could be some use cases that the PWS service only needs to be provided in the specific area, such as the fire alarm in the building, or the heavy traffic accidents at the crossroad. And the existing granularity of Warning Area may be not specified. Therefore, whether to introduce the signaling of intended service area for PWS service could be further discussed by our group.

Proposal: RAN3 shall discuss whether to introduce the signaling of intended service area for PWS service.

- Conclusion

Proposal: RAN3 shall discuss whether to introduce the signaling of intended service area for PWS service.

- 4 Reference

- RAN3#127 Chairman Notes

[2]   TS38.413 v18.5.0