3GPP TSG-RAN WG3 Meeting #127-bis	R3-251898

Wuhan, China, 7-11 April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | emergency services and eDRX for RRC inactive UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As indicated in the RAN2 LS in R2-2411041, Idle eDRX should not be used by the UE when it has a PDU session associated with emergency services. However, it is unclear whether the RRC inactive UEs should be configured with a RAN eDRX in this case. The SA2 reply is provided in S2-2502427. In case that the RAN eDRX should not be used for RRC inactive UEs in case of PDU sessions associated with emergency services, two options below are provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two options are proposed for further down-selection. - Option 1: the AMF updates the RRC Inactive Assistance Information to exclude the extended idle mode eDRX information. Then the NG-RAN would not configure the RAN eDRX for the inactive UE. - Option 2: when the NG-RAN itself identifies the emergency PDU session, it would not configure the RAN eDRX for the inactive UE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it impacts the RAN eDRX for RRC inactive UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear whether the NG-RAN could configure the RAN eDRX for RRC inactive UE in case of emergency services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

### 9.2.2	Mobility in RRC_INACTIVE

#### 9.2.2.1	Overview

RRC_INACTIVE is a state where a UE remains in CM-CONNECTED and can move within an area configured by NG-RAN (the RNA) without notifying NG-RAN. In RRC_INACTIVE, the last serving gNB node keeps the UE context and the UE-associated NG connection with the serving AMF and UPF.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

The AMF provides to the NG-RAN node the Core Network Assistance Information to assist the NG-RAN node's decision whether the UE can be sent to RRC_INACTIVE, and to assist UE configuration and paging in RRC_INACTIVE. The Core Network Assistance Information includes the registration area configured for the UE, the Periodic Registration Update timer, and the UE Identity Index value, and may include the UE specific DRX, an indication if the UE is configured with Mobile Initiated Connection Only (MICO) mode by the AMF, the Expected UE Behaviour, the UE Radio Capability for Paging, the PEI with Paging Subgrouping assistance information, the NR Paging eDRX Information, the Paging Cause Indication for Voice Service, the Hashed UE Identity Index Value and the CN support indication for MT Communication Handling. The UE registration area is taken into account by the NG-RAN node when configuring the RNA. The UE specific DRX and UE Identity Index value are used by the NG-RAN node for RAN paging. The Periodic Registration Update timer is taken into account by the NG-RAN node to configure Periodic RNA Update timer. The NG-RAN node takes into account the Expected UE Behaviour to assist the UE RRC state transition decision. The NG-RAN node may use the UE Radio Capability for Paging during RAN Paging. The NG-RAN node takes into account the PEI with Paging Subgrouping assistance information for subgroup paging in RRC_INACTIVE except when the UE context contains an emergency PDU session in which case the PEI with Paging Subgrouping assistance information shall not be used according to TS 24.501 [28]. When sending the XnAP RAN Paging to neighbour NG-RAN node(s), the PEI with Paging Subgrouping assistance information may be included. The NG-RAN node takes into account the NR Paging eDRX Information to configure the RAN Paging when the NR UE is in RRC_INACTIVE. When sending XnAP RAN Paging to neighbour NG-RAN node(s), the NR Paging eDRX Information for RRC_IDLE and for RRC_INACTIVE may be included. The NG-RAN node takes into consideration the Paging Cause Indication for Voice Service to include the Paging Cause in RAN Paging for a UE in RRC_INACTIVE state. When sending XnAP RAN Paging to neighbour NG-RAN node(s), the Paging Cause may be included. When sending XnAP RAN Paging to neighbour NG-RAN node(s), the Hashed UE Identity Index Value may be included to determine the start point of PTW. The NG-RAN takes into account the CN support indication for MT Communication Handling when deciding to request the AMF for MT Communication Handling for a UE in RRC_INACTIVE state with long eDRX beyond 10.24 seconds as described in TS 23.501 [3].

<<<<<<<<<<<<<<<<<<<< Option 1 begins >>>>>>>>>>>>>>>>>>>>

When the UE has an emergency PDU Session, the AMF updates the Core Network Assistance Information to exclude the NR Paging eDRX Information to avoid the NG-RAN node configuring the UE in RRC_INACTIVE with RAN eDRX. When the PDU Session associated with emergency services is released, the AMF updates the Core Network Assistance Information to include the NR Paging eDRX Information, as described in TS 23.501 [3].

<<<<<<<<<<<<<<<<<<<< Option 1 ends >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Option 2 begins >>>>>>>>>>>>>>>>>>>>

The NG-RAN node takes into account the NR Paging eDRX Information for configuring the RAN eDRX in RRC_INACTIVE except when the UE context contains an emergency PDU session in which case the RAN eDRX shall not be configured according to TS 23.501 [3].

<<<<<<<<<<<<<<<<<<<< Option 2 ends >>>>>>>>>>>>>>>>>>>>

Change Ends