3GPP TSG-RAN WG3 Meeting #127-bis	R3-251916

Wuhan, China, 7-11 April 2025

Title: 	AIML enabled CCO - Multiple CCO issues

Source: 	Ericsson, InterDigital, Jio Platforms, Deutsche Telekom

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution we present our view on how to handle multiple CCO issues (predicted and/or detected).

# 2. Discussion

## Overview

In this paper, we present our view on how to handle multiple CCO issues (predicted and/or detected), for example in the following scenario:

- a CCO issue is predicted by a first gNB1, the corresponding future CCO state is generated and signalled to a neighbour gNB2. The gNB2 generates a matching future CCO state to adapt – in the future – to the future CCO state that will be used at gNB1. Before the future CCO states are applied, another CCO issue is detected (or predicted) by gNB2 (or by a third gNB3), and the new CCO issue impacts the future CCO states generated to address the first predicted CCO issue (from gNB1).

Our approach to this problem is in line with the legacy design of the CCO feature, where each CCO issue is addressed individually and independently from others. We think this is a good design principle because it allows to detect and resolve problems on a “one by one” basis. It is true that different CCO issues may influence each other. However, a solution that tries to deduce the effect of many issues happening at the same time, and that tries to solve all such issues at once, is a very complex and potentially not reliable solution.

For this reason, our approach is to classify all the procedures aimed at resolving a single CCO issue under the same CCO transaction. This helps NG-RAN nodes to understand which CCO issue is resolved by specific CCO state changes as well as to identify processes to resolve each CCO issue for the purpose of prioritising which CCO issue should be resolved first.

With the AI/ML based CCO use case we have predicted CCO issues and detected CCO issues. The steps through which a detected CCO issue or a predicted CCO issue are resolved are logically the same. Processes to resolve both of these CCO issue types can be labelled with a CCO transaction.

Observation 1: All the steps through which the network handles one detected CCO issue or one predicted CCO issue can be regarded as one process, or CCO transaction.

## Handling of multiple CCO issues

Let’s consider an example scenario, shown in the figure below, where at different times, multiple CCO issues appear in the system. In the example we consider gNBs, gNB A (CU-A + DU-A), gNB B (CU-B + DU+B) and gNB C (CU-C + DU-C), and each CCO issue (predicted or detected) is handled individually. All the gNBs are neighbor to each other. To avoid complexity, some steps are not shown in the picture, as described later.

In step 1, CU-A sends to DU-A the predicted CCO issue A#1.

In step 2, DU-A sends to CU-A the generated future CCO state A#1 and the time for future CCO state A#1. To clearly identify the process handling this predicted CCO issue, CU-A sends to DU-A also a CCO-transaction #1, which DU-A returns to CU-A.

In step 3, CU-A sends to its neighbour gNB B (CU-B) the predicted CCO issue A#1, the future CCO state A#1 and the time for future CCO state A#1. To clearly identify that this is the same process handling the predicted CCO issue A#1, CU-A sends to CU-B the CCO transaction #1.

In step 4, CU-B informs DU-B and in step 5 DU-B sends the generated matching future CCO state B#1. To make it clear that the matching future CCO state B#1 is associated with the predicted CCO issue A#1, CU-B sends to DU-B the CCO transaction #1 received from CU-A, and DU-A returns the same CCO transaction #1 in step 5.

We assumed that gNB-A and gNB-C are neighbours, so the step 3 from CU-A to CU-B can be repeated from CU-A to CU-C, and steps 4-5 can be repeated for gNB-C. To avoid making the picture too complex, these are omitted.

In step 6, before the future CCO state A#1 is applied, CU-C sends to DU-C a new detected CCO issue C#1 . Now, we have two CCO issues in the system, one predicted, and one detected. In step 7, DU-C sends the generated CCO state C#1 to CU-C.

In step 8, CU-C (gNB C) sends to its neighbour gNB A the detected CCO issue C#1 and the CCO state C#1. At this point the gNB A needs to handle two CCO issues. In particular, gNB A needs to immediately adjust its coverage to adapt to the detected CCO issue C#1, and it also needs to evaluate if, due to this adjustment, the future CCO state A#1 is still appropriate, or if it needs to select a new (updated) future CCO state to address the predicted CCO issue A#1.

In step 9, CU-A sends to DU-A the detected CCO issue C#1 and the CCO state C#1. Since this is a detected CCO issue, in step 10 DU-A immediately adapts to CCO state C#1 and sends to CU-A a matching CCO state A#2 generated for this purpose.

As said above, in light of the matching CCO state A#2, DU-A may need to adjust the future CCO state A#1 previously derived to prevent the predicted CCO issue A#1. For this reason, in step 14, DU-A may also send to CU-A a future CCO state A#3. CCO state A#3 is a new Future CCO state to solve an already predicted CCO issue, namely CCO issue A#1. To keep the processes for handling the different CCO issues independent from each other, the DU-A indicate to CU-A that the future CCO state A#3 is for CCO transaction #1.

If a new future CCO state A#3 is derived to prevent CCO transaction #1, the gNB-A needs to inform the neighbour gNB-B. This is shown in step 15, which carries the same information as step 3. Note that without stating the CCO Transaction #1, it would not be possible to understand that the new CCO state A#3 is for an already predicted issue. This would create a problem at gNB-B because gNB-B would not understand what is the cause of the newly generated future CCO State A#3. For that, gNB-B will not be able to prioritise the resolution of such issue, which as stated above, should be resolved before moving to the resolution of other issues.

The gNB-C needs to inform also gNB-B of the detected CCO issue C#1. This is done in step 11, where CU-C (gNB C) sends to its neighbour gNB B the detected CCO issue C#1 and the CCO state C#1. In step 12, CU-B informs DU-B, and in step 13, DU-B sends the matching CCO state #2 addressing the detected CCO issue C#1.

If the gNB-A derived a future CCO state A#3 as an update of the future CCO state A#1, gNB-B receives this information in step 15. In this case, similarly to what described for step 14, also DU-B needs to evaluate if the future CCO state B#1, previously generated to prevent the predicted CCO issue A#1, is still appropriate, or if it needs to select a new (updated) future CCO state for that. To clearly identify that the future CCO state A#3 is an update of the predicted CCO issue A#1, in step 16, CU-B sends to DU-B the CCO-transaction #1, and DU-B returns to CU-B the new future CCO state B#3 to CU-B in step 17.

Finally, if no more CCO issues are detected or predicted, DU-A applies the future CCO state A#3 in step 18 and DU-B applies the future CCO state B#3 in step 19.

The proposed example does not intend to represent all possible scenarios where multiple CCO issues are detected or predicted at a given point in time. On the other hand, it allows to analyse and discuss some general principles.

In the figure, for example, actions and signalling related to a certain CCO issue are highlighted in different colours (green for the predicted CCO issue, red for the detected CCO issue), to remark that each CCO issue is handled independently. Each colour represents the process (CCO transaction) used to handle the individual CCO issue. An observation that can be made is therefore that when multiple CCO issues (detected or predicted) are present, each CCO issue can be handled separately.

Observation 2: When multiple CCO issues (detected or predicted) are present, each CCO issue can be handled independently.

If multiple CCO issues are handled independently, we think that sending the time information related to a predicted CCO issue (e.g., related to a coverage modification cause) over Xn is an important part of the solution, as it indicates to the receiver what is the point in time in the future when the issue will occur. On the basis of the Predicted CCO Issue timing, the future CCO state time can be calculated. In case multiple predicted CCO issues are present, the timing information related to each one of them can be used to understand which CCO issue to prioritise in case such issues are predicted for the same time of for very close points in time. Therefore, we propose to resolve the FFS “Exchange time infor for predicted CCO issue over XnAP is not needed unless the usage can be identified?” and send this information via Xn.

Proposal 1: Sending time information related to coverage modification cause over Xn is beneficial to handle multiple CCO issues.

In the figure, the predicted CCO issue #1 is associated with CCO transaction #1 and it is associated with the predicted CCO issue A#1.

If we increase the number of involved gNBs and the number of CCO issues, multiple processes (or transactions) will be active at the same time, with some gNBs handling multiple CCO issues at the same time. A modular approach is still possible, where each CCO issue is treated individually using the same type of process. Namely, if we consider that a future CCO state can be generated at gNB A (DU-A) and that one future CCO state is tied to one predicted CCO issue, then that CCO state can be adjusted quickly when gNB A receives a new “input” (another predicted CCO issue, or another detected CCO issue). This approach makes AI/ML enabled CCO fully aligned with legacy CCO.

Observation 4: Handling of each multiple CCO issue as an independent process (or transaction) for which current/future CCO states can be updated ensures alignment between the legacy CCO and the AI/ML supported CCO solution.

If multiple CCO issues are present, it becomes quite complex for a gNB to keep track of which CCO state (or future CCO state) is associated to which CCO issue (predicted or detected), and which nodes should be contacted to match the generated (future) CCO state.

To support a gNB that needs to handle multiple CCO issues at the same time, and that needs to update the CCO state or the affected cells and beams for each CCO issue, we see a benefit in explicitly identifying in F1 and Xn “CCO transactions”, where a CCO transaction is associated with the CCO state change/update corresponding to a CCO issue (predicted or detected). The use of a CCO transaction identifier enables a gNB to understand which CCO issue is at the root of the problem. Only with this knowledge it is possible for the gNB to appropriately select/update the appropriate CCO state.

With a “CCO transaction identity” it becomes easy to identify which future CCO state (e.g., which future change in cell/beam coverage) refers to which CCO issue. Therefore, to enable a gNB to follow in a good way how a future CCO state (associated with a predicted CCO issue) and a matching future CCO states (generated by neighbour gNBs) may evolve over time due to multiple predicted/detected CCO issues, we propose to introduce a CCO transaction identity in the IE defined for AI/ML enabled CCO in F1 (Predicted CCO Assistance Information, Future Coverage Modification Notification) and in Xn (Future Coverage Modification List).

Proposal 2: To handle multiple CCO issues, introduce a CCO transaction identity in F1 and Xn.

Proposal 3: Agree the TPs to F1AP and XnAP provided in R3-251917 and R3-251918.

# 3. Conclusions

Based on the discussion in this paper, the observations and proposals listed below are derived.

Observation 1: All the steps through which the network handles one detected CCO issue can be regarded as one process, or CCO transaction.

Observation 2: All the steps through which the network handles one predicted CCO issue can be regarded as one process, or CCO transaction.

Observation 3: When multiple CCO issues (detected or predicted) are present, each CCO issue can be handled independently.

Proposal 1: Sending time information related to coverage modification cause over Xn is beneficial to handle multiple CCO issues.

Observation 4: Handling of each multiple CCO issue as an independent process (or transaction) which can be updated (i.e. the current/future CCO states and/or the affected cells and beams can be updated) ensures alignment between the legacy CCO and the AI/ML supported CCO solution.

Proposal 2: To handle multiple CCO issues, introduce a CCO transaction identity in F1 and Xn.

Proposal 3: Agree the TPs to F1AP and XnAP provided in R3-251917 and R3-251918.