3GPP TSG-RAN WG3 Meeting #127-bis	R3-251923

Wuhan, CN, 7-11 Apr, 2025

Title:	Ping pong detection and mitigation

Agenda Item:	10.2

Source:	Huawei, China Unicom, CMCC

Document for:	discussion

# 1	Introduction

This is a general paper discussing some important principles regarding propagation of UE history information.

# 2	Discussion

Solving ping pong was one of the main reasons the UE history information was introduced early in LTE. With the new architecture without a central coordinator (RNC) there was a need to carry information from the source to target cell regarding the UE’s past behaviour. This information could also be used to understand the mobility properties of the UE, i.e., how the UE moved, in order to better understand how to handle it in the future. All this was previously possible in the RNC.

Ping pong was also part of the discussions from SON from the beginning. There has however been no motivation to describe the general detection and mitigation for intra-system mobility in stage 2 specs, since it was assumed that the node receiving the UE history would be the node to detect and mitigate the problem.

Only one scenario was described, namely inter-system ping pong. This because it was felt that performing an inter system mobility action was quite costly and should also span cases where the UE does not necessarily return to the source cell but also to another cell in the same system. Therefore, a mechanism for detecting this when the UE returned to the system and informing the source node for the inter-system ping pong was introduced. This is done in the Handover Report.

Even if the general ping-pong case was not described, the mobility settings change was introduced to manage the hysteresis between nodes. With this function, node1 can ask node2 to adjust the mobility decisions when handing back to node1. Node1 can also (optionally) inform node2 about how node1 has changes his own mobility decisions.

The latest addition to UE history information was when SCG mobility was added. This enables the MN to include decisions on SCG mobility in the information sent to the target MN. In this discussion (which is similar to the discussion on LTM) it was considered valuable to send the full UE history information to the SN. Two benefits were seen for SN triggered mobility decisions: the ability to avoid ping-pong and the possibility to understand the mobility pattern of the UE. The MN was considered in charge for collecting information to be passed over to the next MN, with some assistance from the SN. Since the MN was not always aware of the intra-SN mobility, SN was assigned the task to collect this information and pass it to MN which correlated the information in to the full history which is sent to the next target MN.

We think the following principles are important to re-iterate:

- UE history information is useful for ping pong detection and to determine the mobility profile of the UE

- Ping pong detection is performed in the node that makes the adjustment of the mobility parameters

- UE history is sent in the handover over preparation (or similar)

- For successive mobility (CPAC or LTM) there is still no solution for the mobility events after the initial mobility. For S-CPAC it is currently only possible to send the UE history information to the SN during initial configuration.

One new proposal for mitigating ping-pong, is to modify the candidate list. The claimed benefit is that the analysis can be made in a node different from the one determining the exit conditions. The drawback is however that the candidate is fully excluded from any mobility decision. One problem with this is that the candidate would be removed even if the only problem would be a too low hysteresis. Another problem is that this is not useful in subsequent scenarios, where removing a candidate cell would means that handovers from all other candidate cells this cell is forbidden.

Another argument against was that it would make sense for the node managing the candidate set to directly remove candidates. It should be noted that this is still possible if the ping-pong detection is in the cell responsible for the exit condition. In addition to this, the adjustment to the exiting connections could eventually lead to one candidate cell being used less often, which is a much better criteria for removing the cell from the candidate cell list.

We propose that the same principles as for legacy should apply for LTM cell switch:

The principles for UE history should apply also for LTM cell switch, i.e. the node adjusting the execution condition should get the UE history info.

One new functionality that was added recently are the subsequent mobility cases. There is subsequent C-PAC and subsequent LTM. For these cases, there is an initial configuration which is kept. This means that it may not be enough to send the UE history information at the initial configuration. One possible approach is to mandate that the UE history information is updated to all participating node whenever a mobility event occurs.

Some companies express that this may not be desirable. The subsequent mobility scheme reduces the need for reconfiguring nodes and forcing an update of the UE history information would somehow partially negate this benefit. It should also be noted that in subsequent cases, the node itself can store information related to a specific UE which can be later retrieved once the UE returns. Hence, the node could at least detect the time the UE was outside the node which may be used as a simple for of ping pong detection.

Hence, another possible approach is that the UE history information is updated more seldomly, i.e., not at ever mobility event.  A potential way forward could be to specify that the node who has the information (CU and MN respectively) is allowed reduce (filter) the update of UE history information e.g. if the sending node determines that there is no ping-pong or if a recent update of the UE history information is sent.

It should be possible to update the UE history information for subsequent mobility cases. RAN3 to discuss whether to reduce the update of UE history information in subsequent mobility cases.

It should be noted that the node responsible for keeping the UE history information (MN or CU) may also use this for adjusting the candidate cell list.

# 3	Conclusion

This document proposes:

Proposal 1:	The principles for UE history should apply also for LTM cell switch, i.e. the node adjusting the execution condition should get the UE history info.

Proposal 2:	It should be possible to update the UE history information for subsequent mobility cases. RAN3 to discuss whether to reduce the update of UE history information in subsequent mobility cases.