3GPP TSG-RAN WG3 Meeting #127-bis	R3-251930

Wuhan, CN, 7-11 Apr, 2025

Title:	(TP for SON BLCR for 38.401) SLrelay control plane

Agenda Item:	22.2

Source:	Huawei

Document for:	discussion

# 1	Introduction

In RAN3#127 meeting, we had the first time to discuss the RAN3 aspects to support multi-hop relay, including the control plan procedure, service continuty and the authorization. Based on the LS from SA2 [S2-2501296], we agreed the CRs for the authorization for multi-hop relay [1][2][3]. For the control plan procedures, we have the following agreements and agreed the BL CR to TS 38.401[4].

RAN3 can work on the new signaling flow for multi-hop relay for the purpose of Initial Access procedure based on the existing procedures in TS 38.401. RAN3 agrees to support the Multi-hop relay in CU-DU split architecture. The existing F1AP UE Context Setup and UE Context Modification procedure can be used as a baseline to establish the PC5/Uu Relay RLC channel(s) for relaying of Remote UE’s SRB/DRB. For the initial UL RRC transfer message, the gNB-DU informs the gNB-CU of the multi-hop relay related information including at least gNB-DU UE F1AP ID of the Last Relay UE and Remote UE Local ID.

The BL CR to 38.401 provided the overall procedure for Remote UE’s initial access in multi-hop relay, and left some FFSs as highlighted below.

## 8.xx Overall procedures for Multi-hop Layer-2 UE-to-Network Relay ### 8.xx.1 Remote UE initial access Editor’s Note: FFS where to capture the following procedure. Editor’s Note: Current signaling flows are based on the agreements that the intermediate Relay UE is in RRC_CONNECTED state. Whether the intermediate Relay UE in RRC_IDLE/RRC_INACTIVE state is supported or not is pending to RAN2. The signalling flow for Remote UE Initial access is shown in Figure 8.xx.1-1. Figure 8.xx.1-1: Overall procedure for Remote UE’s initial access via multi-hop relay 1. The U2N Remote UE, the U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure. 2. The U2N Remote UE sends an RRCSetupRequest message to the U2N First Relay UE via PC5 Relay RLC channel. 3. The U2N First Relay UE withholds the received RRC message. If the U2N First Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should send its own RRCSetupRequest message to the U2N Intermediate Relay UE via PC5 Relay RLC channel in order to trigger the RRC establishment/resume procedure to enter RRC_CONNECTED state upon reception of the RRC message from the U2N Remote UE. If the U2N Intermediate Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should trigger the RRC establishment/resume procedure in clause 8.19.1 to enter RRC_CONNECTED state upon reception of the RRC message from the U2N First Relay UE. If all Relay UEs are in RRC_CONNECTED state, this step could be skipped. 4. The U2N First Relay UE in RRC_CONNECTED state sends the SidelinkUEInformationNR message to the gNB-DU via the Intermediate Relay UE and U2N Last Relay UE. 5. The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N First Relay UE by encapsulating the SidelinkUEInformationNR message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE. 6. The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the U2N Last Relay UE to gNB-DU. Such message may request the establishment of Uu Relay RLC channel(s) and PC5 Relay RLC channel(s) for the transmission of U2N Remote UE’s SRB0. 7. The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the U2N Last Relay UE to gNB-CU. 8. The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the U2N Last Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE. The RRCReconfiguration message shall also contain the Uu Relay RLC channel(s) configuration and PC5 Relay RLC channel(s) configuration if not configured and bearer mapping for relaying of U2N Remote UE’s SRB0. 9. The gNB-DU sends the RRCReconfiguration message to the U2N Last Relay UE to configure the local ID of the U2N Remote UE, the Uu Relay RLC channel(s) configuration, PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0. 10. The U2N Last Relay UE sends the RRCReconfigurationComplete message to gNB-DU. 11. The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Last Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU. 12. The gNB-CU configures the U2N Intermediate Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the U2N Intermediate Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11. 13. The gNB-CU configures the U2N First Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the U2N First Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11. Editor’s Note: FFS whether each Relay UE needs to establish new PC5/Uu RLC channel for a Remote UE. Editor’s Note: FFS how/what information is used to indicate the counterpart information of a configured PC5 Relay RLC channel to the gNB-DU. Editor’s Note: FFS how to configure relay UEs for relaying of remote UE’s SRB/data over F1, e.g. via separate procedures of each relay UE or via an single procedure for all relay UEs’ configuration. 14. After receiving the local ID of the U2N Remote UE and the PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0, the U2N First Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU via the U2N Intermediate Relay UE and the U2N Last Relay UE. The local ID of the U2N Remote UE and RB ID for SRB0 are conveyed in the SRAP header. 15. The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE, the gNB-DU UE F1AP ID of the U2N Last Relay UE and the sidelink configuration container for the PC5 Relay RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message. Editor’s Note: FFS whether the gNB-DU UE F1AP IDs of the U2N First Relay UE and the U2N Intermediate Relay UE are included in the INITIAL UL RRC MESSAGE TRANSFER message. Editor’s Note: FFS whether and how the gNB-DU becomes aware that the U2N Remote UE is connected to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE. 16. The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 Relay RLC channel and bearer mapping at least for the transmission of U2N Remote UE’s SRB1. 17. The gNB-DU sends the RRCSetup message to the U2N Remote UE via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE. 18. The gNB-CU configures the U2N Last Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the U2N Last Relay UE establishes a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB1 over PC5 and establishes a Uu Relay RLC channel for relaying of U2N Remote UE’s SRB1 towards gNB-DU if not configured yet. The gNB-CU configures the First Relay UE and the U2N Intermediate Relay UE with PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the First Relay UE and the U2N Intermediate Relay UE establish the PC5 Relay RLC channels for relaying of U2N Remote UE’s SRB1 over PC5 if not configured yet. Editor’s Note: FFS whether Step 18 can be performed earlier, e.g., via Steps 6-13. 19. The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE. 20. The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU. 21. Upon receiving the RRCSetupComplete message of U2N Remote UE, the gNB-CU sends the INITIAL UE MESSAGE message to the AMF. 22. The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU. 23. The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU. Such message may request the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs, and may also encapsulate the SecurityModeCommand message. 24. The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE. 25. The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs. 26. The U2N Remote UE responds with the SecurityModeComplete message. 27. The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU. 28. The gNB-CU generates the RRCReconfiguration message for U2N Remote UE and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration message contains the configuration of PC5 Relay RLC channels and bearer mapping for the transmission of U2N Remote UE’s SRB2 and DRBs. 29. The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE. 30. The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE. 31. The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU. 32. The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF. 33. The gNB-CU configures additional Uu Relay RLC channels between the gNB-DU and the U2N Last Relay UE, and additional PC5 Relay RLC channels for the U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE for relaying of U2N Remote UE’s DRBs and SRBs. Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB and PC5/Uu Relay RLC channel at the U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE. Editor’s Note: FFS whether this step can be performed earlier.

In this paper, we will discuss the above FFSs and provided the corresponding TPs in the Annex.

# 2	Discussion

## 2.1	SRB message forwarding

In above step 12 and 13, the gNB-CU and gNB-DU initiate the F1 procedures to configure the relaying configuration for remote UE’s SRB0. For the Following three FFS, they are related to the relaying configuration of SRB0 on the F1 interface for the relay UE.

- Editor’s Note: FFS whether each Relay UE needs to establish new PC5/Uu RLC channel for a Remote UE.

- Editor’s Note: FFS how/what information is used to indicate the counterpart information of a configured PC5 Relay RLC channel to the gNB-DU.

- Editor’s Note: FFS how to configure relay UEs for relaying of remote UE’s SRB/data over F1, e.g. via separate procedures of each relay UE or via an single procedure for all relay UEs’ configuration.

For the first FFS, whether new PC5/Uu RLC channel need to be established for remote UE SRB message depends on the gNB-CU implementation, which is similar to that in Rel-17 SL relay. If the existing PC5/Uu relay RLC channel is reused to multiplexing this remote UE’s SRB message, no new relay RLC channel establishment is needed and the modification of the existing PC5/Uu relay RLC channel maybe triggered by using Uu/PC5 RLC Channel to Be Modified List. Otherwise, the gNB-CU can configure to establish the new PC5/Uu RLC channel over the F1 interface, and can reuse the current signalling and IEs, i.e., Uu/PC5 RLC Channel to Be Setup List, to indicate gNB-DU.

Whether each Relay UE needs to establish new PC5/Uu RLC channel for a Remote UE depends on the NW implantation, i.e., depends on whether the existing PC5/Uu relay RLC channel or new ones are used.

The existing Uu/PC5 RLC Channel to Be Setup List IE and Uu/PC5 RLC Channel to Be Modified List IE can be reused to configure the PC5 Relay RLC channel to the gNB-DU.

Following the legacy mechanism, we should use separate F1 signalling procedures to configure each relay UE, which contains the UE F1AP ID for the corresponding relay UE.

Separate procedures of each relay UE should be used to configure the relaying configuration at each relay UE.

## 2.2 Remote UE identification on gNB-DU

In step 15, the gNB-DU forwards the remote UE’s SRB0 message to the gNB-CU. In release-17, the gNB-DU UE F1AP ID of the relay UE and the remote UE local ID will be contained in the INITIAL UL RRC MESSAGE TRANSFER message to the gNB-CU, so that gNB-CU can related the SRB0 message to a specific remote UE which was reported in the SUI message by the relay UE.

For the multi-hop relay, the first relay UE report the remote UE L2 ID in the SUI message in step 4 to request the local ID. Similarly, the gNB-DU can contain the gNB-DU UE F1AP ID of the U2N First Relay UE  and the remote UE local ID is contained in the INITIAL UL RRC MESSAGE TRANSFER message, then the gNB-CU can know it is a remote UE with the indicated local ID in the scope of the first relay UE.

In release-17, the relay UE sending SUI connects with the gNB-DU directly, thus it is easy for the gNB-DU to identify the relay UE F1AP ID. However, in multi-hop relay, the gNB-DU cannot know which first relay UE is related to the remote UE. Therefore, how for the gNB-DU to identify the gNB-DU UE F1AP ID of the first relay UE is a problem.

- Observation 1: How for the gNB-DU to identify the gNB-DU UE F1AP ID of the first relay UE is a problem.

As an alternative solution, the gNB-DU can carry the Last relay UE’s gNB-DU UE F1AP ID and the remote UE local ID. The problem of this solution is the gNB-CU may not be able to uniquely identify the remote UE if the local ID of remote UEs that connects to different first relay UE are the same (as shown in Figure 1). Restricting the uniqueness of local ID in the scope of Last relay UE can solve this problem, but it will limit the number of connected remote UEs to the Last relay UE.

Figure 1. Local ID allocation for the remote UEs

- Observation 2: If the gNB-DU carries the Last relay UE’s gNB-DU UE F1AP ID and the remote UE local ID in the INITIAL UL RRC MESSAGE TRANSFER message. It will restrict the number of connected remote UE with the last relay UE.

For the local ID issue, RAN3 can wait for RAN2 for more progress before determining the content in the INITIAL UL RRC MESSAGE TRANSFER message.

RAN3 can wait for RAN2 for more progress on the local ID allocation before determining the content in the INITIAL UL RRC MESSAGE TRANSFER message.

2.3 other FFSs

In step 18, gNB-CU and gNB-DU configures the relay RLC channels for remote UE’s SRB1 message. Technically, after receiving the SUI message from the first relay UE in Step 5, the gNB-CU can know there is one remote UE identified with the indicated L2 ID in the SUI is trying to access and prepare the necessary configurations for this remote UE, including the relaying configuration for SRB1. However, there can be some cases that the RRC Setup of the remote UE fails, e.g., the remote UE SRB0 message cannot be successfully forwarded or the access control failure. In above cases, the gNB-CU and gNB-DU has to de-configure the relaying configuration for the remote UE, which causes signalling and resource overhead.

It is gNB implementation when to configure the relay RLC channels for remote UE’s SRB1 message, including before or after the successfully forwarding of remote UE’s SRB0 message.

Similarly, when to configure the relaying configuration for the SRB2 and DRBs can be gNB implementation. For the relaying configuration for DRB, it should be after the after the PDU Session resource setup, since the gNB-CU and gNB-DU need the QoS information of the DRBs to determine the mapping configuration and the relay RLC channel configurations.

It is gNB implementation when to configure the relay RLC channels for remote UE’s SRB2 and DRB message. The relaying configuration determination for remote UE’s DRB needs the QoS information of the DRBs, thus should be after the PDU Session resource setup procedure.

According to the above discussion and proposals, we update the BL CR in Annex 1.

# 3	Conclusion

This document proposes:

- Observation 1: How for the gNB-DU to identify the gNB-DU UE F1AP ID of the first relay UE is a problem.

- Observation 2: If the gNB-DU carries the Last relay UE’s gNB-DU UE F1AP ID and the remote UE local ID in the INITIAL UL RRC MESSAGE TRANSFER message. It will restrict the number of connected remote UE with the last relay UE.

- Whether each Relay UE needs to establish new PC5/Uu RLC channel for a Remote UE depends on the NW implantation, i.e., depends on whether the existing PC5/Uu relay RLC channel or new ones are used.

The existing Uu/PC5 RLC Channel to Be Setup List IE and Uu/PC5 RLC Channel to Be Modified List IE can be reused to configure the PC5 Relay RLC channel to the gNB-DU.

Separate procedures of each relay UE should be used to configure the relaying configuration at each relay UE.

RAN3 can wait for RAN2 for more progress on the local ID allocation before determining the content in the INITIAL UL RRC MESSAGE TRANSFER message.

It is gNB implementation when to configure the relay RLC channels for remote UE’s SRB1 message, including before or after the successfully forwarding of remote UE’s SRB0 message.

It is gNB implementation when to configure the relay RLC channels for remote UE’s SRB2 and DRB message. The relaying configuration determination for remote UE’s DRB needs the QoS information of the DRBs, thus should be after the PDU Session resource setup procedure.

# 4	Reference

[1] R3-250940, (BL CR to 38.413) Support of Multi-hop relay, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, LG Electronics, NEC, CATT, InterDigital, Samsung, ZTE, FirstNet, NIST, 3GPP TSG-RAN WG3 Meeting #127

[2] R3-250941, (BL CR to 38.423) Ericsson, LG Electronics, Nokia, Nokia Shanghai Bell, Huawei, NEC, CATT, InterDigital, Samsung, ZTE, FirstNet, NIST, 3GPP TSG-RAN WG3 Meeting #127

[3] R3-250942, (BL CR to 38.473) Support of Multi-hop relay, Huawei, LG Electronics, Nokia, Nokia Shanghai Bell, NEC, CATT, Ericsson, Samsung, ZTE, Interdigital, FirstNet, 3GPP TSG-RAN WG3 Meeting #127

[4] R3-250939, (BL CR to 38.401) Support of Multi-hop relay, LG Electronics Inc., ZTE, Nokia, Nokia Shanghai Bell, Ericsson, FirstNet, Huawei, 3GPP TSG-RAN WG3 Meeting #127

# Annex1	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 8.xx	Overall procedures for Multi-hop Layer-2 UE-to-Network Relay

### 8.xx.1	Remote UE initial access

Editor’s Note: Current signaling flows are based on the agreements that the intermediate Relay UE is in RRC_CONNECTED state. Whether the intermediate Relay UE in RRC_IDLE/RRC_INACTIVE state is supported or not is pending to RAN2.

The signalling flow for Remote UE Initial access is shown in Figure 8.xx.1-1.

Figure 8.xx.1-1: Overall procedure for Remote UE’s initial access via multi-hop relay

1.	The U2N Remote UE, the U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

2.	The U2N Remote UE sends an RRCSetupRequest message to the U2N First Relay UE via PC5 Relay RLC channel.

3.	The U2N First Relay UE withholds the received RRC message. If the U2N First Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should send its own RRCSetupRequest message to the U2N Intermediate Relay UE via PC5 Relay RLC channel in order to trigger the RRC establishment/resume procedure to enter RRC_CONNECTED state upon reception of the RRC message from the U2N Remote UE. If the U2N Intermediate Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should trigger the RRC establishment/resume procedure in clause 8.19.1 to enter RRC_CONNECTED state upon reception of the RRC message from the U2N First Relay UE. If all Relay UEs are in RRC_CONNECTED state, this step could be skipped.

4.	The U2N First Relay UE in RRC_CONNECTED state sends the SidelinkUEInformationNR message to the gNB-DU via the Intermediate Relay UE and U2N Last Relay UE.

5.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N First Relay UE by encapsulating the SidelinkUEInformationNR message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE.

6.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the U2N Last Relay UE to gNB-DU. Such message may request the establishment of Uu Relay RLC channel(s) and PC5 Relay RLC channel(s) for the transmission of U2N Remote UE’s SRB0.

7.	The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the U2N Last Relay UE to gNB-CU.

8.	The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the U2N Last Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE. The RRCReconfiguration message shall also contain the Uu Relay RLC channel(s) configuration and PC5 Relay RLC channel(s) configuration if not configured and bearer mapping for relaying of U2N Remote UE’s SRB0.

9.	The gNB-DU sends the RRCReconfiguration message to the U2N Last Relay UE to configure the local ID of the U2N Remote UE, the Uu Relay RLC channel(s) configuration, PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0.

10.	The U2N Last Relay UE sends the RRCReconfigurationComplete message to gNB-DU.

11.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Last Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

12.	The gNB-CU configures the U2N Intermediate Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the U2N Intermediate Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

13.	The gNB-CU configures the U2N First Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the U2N First Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

14.	After receiving the local ID of the U2N Remote UE and the PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0, the U2N First Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU via the U2N Intermediate Relay UE and the U2N Last Relay UE. The local ID of the U2N Remote UE and RB ID for SRB0 are conveyed in the SRAP header.

15.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE, the gNB-DU UE F1AP ID of the U2N Last Relay UE and the sidelink configuration container for the PC5 Relay RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message.

Editor’s Note: FFS whether the gNB-DU UE F1AP IDs of the U2N First Relay UE and the U2N Intermediate Relay UE are included in the INITIAL UL RRC MESSAGE TRANSFER message.  Wait for RAN2 progress on the remote UE local ID allocation.

Editor’s Note: FFS whether and how the gNB-DU becomes aware that the U2N Remote UE is connected to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE. Wait for RAN2 progress on the remote UE local ID allocation.

16.	The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 Relay RLC channel and bearer mapping at least for the transmission of U2N Remote UE’s SRB1.

17.	The gNB-DU sends the RRCSetup message to the U2N Remote UE via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

18.	The gNB-CU configures the U2N Last Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the U2N Last Relay UE establishes a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB1 over PC5 and establishes a Uu Relay RLC channel for relaying of U2N Remote UE’s SRB1 towards gNB-DU if not configured yet.

The gNB-CU configures the First Relay UE and the U2N Intermediate Relay UE with PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the First Relay UE and the U2N Intermediate Relay UE establish the PC5 Relay RLC channels for relaying of U2N Remote UE’s SRB1 over PC5 if not configured yet.

NOTE 1: Step 18 can be performed earlier, e.g., via Steps 6-13.

19.	The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

20.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

21.	Upon receiving the RRCSetupComplete message of U2N Remote UE, the gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

22.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

23.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU. Such message may request the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs, and may also encapsulate the SecurityModeCommand message.

24.	The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

25.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs.

26.	The U2N Remote UE responds with the SecurityModeComplete message.

27.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

28.	The gNB-CU generates the RRCReconfiguration message for U2N Remote UE and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration message contains the configuration of PC5 Relay RLC channels and bearer mapping for the transmission of U2N Remote UE’s SRB2 and DRBs.

29.	The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

30.	The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

31.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

32.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

33.	The gNB-CU configures additional Uu Relay RLC channels between the gNB-DU and the U2N Last Relay UE, and additional PC5 Relay RLC channels for the U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE for relaying of U2N Remote UE’s DRBs and SRBs. Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB and PC5/Uu Relay RLC channel at the U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE.

NOTE 2: this step can be performed earlier.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>