3GPP TSG-RAN WG3 Meeting #127bis	R3-251933

Wuhan, CN, April 7–11, 2025

Agenda item:	 10.2

Source:		Lenovo

Title:		(TP for SON BLCR for 38.423) Discussion on MRO for subsequent CPAC and CHO with candidate SCG(s)

Document for:		Discussion and Decision

1	Introduction

In RAN3#125, we agreed that:

MRO for CHO with Candidate SCG(s)

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed.

MRO for case 4/5/6 is in the scope.

The RLF report includes: The type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met.

MRO for S-CPAC:

Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.

In RAN3#125bis, we further agreed that:

MRO for CHO with Candidate SCG(s)

RLF report is enhanced to including

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

In RAN3#126, we agreed that:

MRO for CHO with Candidate SCG(s)

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

The failure types defined for CHO in TS38.300 and for CPAC defined in TS37.340 are used as baseline. Clarification or amendment could be made on top of that if needed.

In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization.

SCGFailureInfomation handling for case 1b)/2b)/3b)/9b) and case 7b)/8b):

The serving MN which receives SCGFailureInformation from the UE performs the initial analysis.

The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g for case 3b).

Whether the above mechanism works depends on RAN2 progress on SCGFailureInfomation.

In RAN3#127, we further agreed that:

MRO for CHO with Candidate SCG(s):

In case of too late CHO execution, reusing Handover Report message from the last serving MN to the relevant candidate MN.

FFS on whether Handover Report message is needed for case 7a.

FFS on the additional information included in handover report.

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

When to send the message? Immediately or not?

Check whether respective MN is only a candidate?

In addition to SCGFailureInformation, information needed in the new message?

- CPC failure type

New stage-2 definition referring to CHO with candidate SCG is needed.

MRO for S-CPAC：

The forwarding mechanism in BLCR for TS37.340 need to be refined.

In this paper, we would further discuss the details of MRO for subsequent CPAC and CHO with candidate SCG(s).

2	Discussion

  - 2.1 MRO for subsequent CPAC

For MN initiated subsequent CPAC, the MN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC (e.g. CPA or CPC). For MN initiated subsequent CPAC, the candidate SN generates the execution conditions for the following execution of subsequent CPAC when the candidate SN prepares the candidate SCG configuration(s) for candidate PSCell(s).

For SN initiated subsequent CPAC, the source SN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC. For SN initiated subsequent CPAC, the candidate SN generates the execution conditions for the following execution of subsequent CPAC when the candidate SN prepares the candidate SCG configuration(s) for candidate PSCell(s). For SN initiated intra-SN subsequent CPAC, the source SN generates the execution conditions for the following execution of subsequent CPAC when the source SN prepares the candidate SCG configuration(s) for candidate PSCell(s).

For MRO for MN initiated subsequent CPAC,

- if the suitable PSCell is not one of the candidate PSCells provided by MN at S-CPAC preparation, the MN performs the final MRO related optimisation e.g. optimize the candidate PSCell list, in case of initial execution of subsequent CPAC (e.g. CPA or CPC) or following execution of subsequent CPAC;

- if the suitable PSCell is one of the candidate PSCells provided by MN at S-CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, the MN sends the SCG FAILURE INFORMATION REPORT message to candidate or target SN for candidate PSCell selection optimization, in case of initial execution of subsequent CPAC or following execution of subsequent CPAC;

- otherwise, MN performs the final MRO related optimisation e.g. optimize the execution conditions for the initial execution of subsequent CPAC, in case of initial execution of subsequent CPAC, or, the MN may forward the SCG FAILURE INFORMATION REPORT message to source SN of following execution of subsequent CPAC for optimising the execution conditions for the following execution of subsequent CPAC, in case of following execution of subsequent CPAC.

For MRO for SN initiated subsequent CPAC, the MN sends the SCG Failure Information Report message to SN initiating the subsequent CPAC procedure,

- if the suitable PSCell is not one of the candidate PSCells provided by SN initiating the subsequent CPAC procedure, the SN initiating the subsequent CPAC procedure performs the final MRO related optimisation e.g. optimize the candidate PSCell list, in case of initial execution of subsequent CPAC (e.g. CPA or CPC) or following execution of subsequent CPAC;

- if the suitable PSCell is one of the candidate PSCells provided by SN initiating the subsequent CPAC procedure, but not one of the candidate PSCells selected by the candidate or target SN, SN initiating the subsequent CPAC procedure indicates the MN, then, MN sends the SCG FAILURE INFORMATION REPORT message to candidate or target SN for candidate PSCell selection optimization, in case of initial execution of subsequent CPAC or following execution of subsequent CPAC;

- otherwise, SN initiating the subsequent CPAC procedure performs the final MRO related optimisation e.g. optimize the execution conditions for the initial execution of subsequent CPAC, in case of initial execution of subsequent CPAC, or, SN initiating the subsequent CPAC procedure indicates the MN to send the SCG FAILURE INFORMATION REPORT message to source SN of following execution of subsequent CPAC for optimising the execution conditions for the following execution of subsequent CPAC, in case of following execution of subsequent CPAC.

The corresponding TP for TS37.340 was agreed in [1].

When a SCG failure happens in subsequent CPAC procedure, we can find that, if execution conditions are not proper, different node performs optimisation in case of different execution phase, i.e. in case of initial execution of subsequent CPAC, the node initiating S-CPAC procedure optimizes the execution conditions for the initial execution of subsequent CPAC, in case of following execution of subsequent CPAC, the MN forwards the SCG FAILURE INFORMATION REPORT message to source SN of following execution of subsequent CPAC for optimising the execution conditions for the following execution of subsequent CPAC. To enable MN to forward SCG FAILURE INFORMATION REPORT message correctly, it is benecifical to enable MN know whether the failure happens in initial execution of subsequent CPAC or following execution of subsequent CPAC. Based on existing SCGfailureInformation message from the UE, MN can’t know this. Therefore, an indication concerning the SCG Failure Information message is for initial execution of subsequent CPAC or following execution of subsequent CPAC may be included in the SCG Failure Information message‎.

Proposal 1: An indication concerning the SCG Failure Information message is for initial execution of subsequent CPAC or following execution of subsequent CPAC may be included in the SCG Failure Information message.

Similar as legacy CPAC, it may be possible that source SN has released UE context when receiving the SCG Failure Information Report message, to enable source SN to identify the received SCG failure information, subsequent CPAC configuration (e.g. candidate PSCell list for S-CPAC, S-CPAC execution condition(s)) may be included in the SCG Failure Information Report message.

Proposal 2: Include subsequent CPAC configuration (e.g. candidate PSCell list for S-CPAC, S-CPAC execution condition(s)) in the SCG Failure Information Report message.

  - 2.2 MRO for CHO with candidate SCG(s)

    - 2.2.1 MRO for connection failure case

In previous meetings, Case 1a), 2a), 3a) and 9a) are defined as Too late handover, and, in case of too late CHO execution, Handover Report message is agreed from the last serving MN to the relevant candidate MN, i.e. a new Handover Report Type "Too Late CHO with candidate SCG" is introduced. Based on the existing IEs in Handover Report message and the new Handover Report Type, the relevant candidate MN can deduce a too late CHO with candidate SCG happens, and perform corresponding MRO optimisation. Thus, no additional information is needed in Handover Report message.

Observation 1: In case of too late CHO with candidate SCG(s), no additional information is needed in Handover Report message.

For case 1b)/2b)/3b)/9b) and case 7b)/8b), the serving MN which receives SCGFailureInformation from the UE performs the initial analysis, and a new message (i.e. SCG Failure Indication) is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

For the issue about when to send the message e.g. immediately or not, since SCGFailureInformation in Uu interface is an immediate message, similar as SCG FAILURE INFORMATION REPORT and SCG FAILURE TRANSFER procedure for MRO for legacy SCG failure, the new message (i.e. SCG Failure Indication) should be an immediate message.

Proposal 3: The SCG Failure Indication message should be an immediate message.

For the issue about whether SCG Failure Indication message will contain only one SCGFailureInformation message or a list of SCGFailureInformation messages, take case 3b) as an example, when a candidate PCell1 in which CHO execution condition is met is configured with multiple candidate PSCells (e.g. PSCell 1, PSCell 2 and PSCell 3), but none CPAC execution conditions of these multiple candidate PSCells is met, when SCG failure happens, the serving MN which receives SCGFailureInformation from the UE would forward the SCGFailureInformation to the MN (e.g. MN1) in which candidate PCell1 belongs to, there are two options:

- Option 1: the serving MN only sends one SCG Failure Indication message to the MN1 to indicate that CPAC execution conditions of candidate PSCells (e.g. PSCell 1, PSCell 2 and PSCell 3) are not met;

- Option 2: for each candidate PSCell (e.g. PSCell 1/2/3), the serving MN sends one SCG Failure Indication message to the MN1 separately to indicate that CPAC execution condition of one candidate PSCell (e.g. PSCell 1/2/3) is not met.

Compared the above two options, Option 1 has more spec impacts but less signalling overhead in Xn interface, e.g. the SCG Failure Indication message needs to be enhanced to contain a list of SCGFailureInformation messages, however, in Option 2, the SCG Failure Indication message needs to be sent in Xn interface for several times. We are fine with any of the two options, and Option 2 is slightly preferred.

Proposal 4: The SCG Failure Indication message can contain only one SCGFailureInformation from the UE.

For the issue about which information (e.g. CPC failure type) is needed in the new message (i.e. SCG Failure Indication) in addition to SCGFailureInformation, since SCGFailureInformation from the UE includes necessary information, after the respective MN receives the SCG Failure Indication message, it can perform failure type analysis and do corresponding optimisation, it is not needed to include CPC failure type in the SCG Failure Indication message.

Proposal 5: No need to include CPC failure type in the SCG Failure Indication message.

    - 2.2.2 MRO for near failure successful case

When CHO with candidate SCG(s) is executed, a near-failure successful PCell change and/or a near-failure successful PSCell addition or PSCell change may happen. For sub-optimal successful CHO execution, to help network know how to modify the configuration of the CHO with candidate SCG(s) procedure, legacy SHR trigger condition(s) can be reused, and SHR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SHR.

Proposal 6: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Similarly, for any sub-optimal successful CPAC, legacy SPR trigger condition(s) can be reused, and SPR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SPR.

Proposal 7: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

In case the SPR is retrieved in a node different from source MN that sent the SPR configuration to the UE or target MN that to which the UE successfully access, since SPR trigger condition(s) may be generated by target MN or target SN, for SPR forwarding, there are two potential options:

- Option 1: the receiving node that fetches the SPR from the UE first forwards the SPR to source MN, then source MN forwards the SPR to target MN, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

- Option 2: the receiving node that fetches the SPR from the UE forwards the SPR to target MN, then, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

In Option 1, to enable the receiving node to forward the SPR to source MN, the UE should store and report cell ID of source PCell in the SPR, furthermore, the UE should store and report cell ID of target PCell in the SPR for forwarding the SPR from source MN to target MN. Compared with Option 1, in Option 2, the UE should only store and report cell ID of target PCell in the SPR, which has fewer spec impacts. RAN3 should discuss the SPR forwarding mechanism for CHO with candidate SCG(s).

Proposal 8: RAN3 to discuss SPR forwarding for CHO with candidate SCG(s):

- Option 1: the receiving node that fetches the SPR from the UE first forwards the SPR to source MN, then source MN forwards the SPR to target MN, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

- Option 2: the receiving node that fetches the SPR from the UE forwards the SPR to target MN, then, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

3	Conclusion

In this paper, the details of MRO for subsequent CPAC and CHO with candidate SCG(s) are discussed. We have the following proposals:

MRO for subsequent CPAC

Proposal 1: An indication concerning the SCG Failure Information message is for initial execution of subsequent CPAC or following execution of subsequent CPAC may be included in the SCG Failure Information message.

Proposal 2: Include subsequent CPAC configuration (e.g. candidate PSCell list for S-CPAC, S-CPAC execution condition(s)) in the SCG Failure Information Report message.

MRO for CHO with candidate SCG(s)

Observation 1: In case of too late CHO with candidate SCG(s), no additional information is needed in Handover Report message.

Proposal 3: The SCG Failure Indication message should be an immediate message.

Proposal 4: The SCG Failure Indication message can contain only one SCGFailureInformation from the UE.

Proposal 5: No need to include CPC failure type in the SCG Failure Indication message.

Proposal 6: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Proposal 7: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

Proposal 8: RAN3 to discuss SPR forwarding for CHO with candidate SCG(s):

- Option 1: the receiving node that fetches the SPR from the UE first forwards the SPR to source MN, then source MN forwards the SPR to target MN, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

- Option 2: the receiving node that fetches the SPR from the UE forwards the SPR to target MN, then, if needed target MN forwards the SPR to the respective SN(s) which should perform the SPR optimization (e.g. corresponding target SN that generates the SPR trigger condition that triggers the SPR).

Reference

[1] R3-250900, (TP for SON BLCR TS37.340) MRO for S-CPAC, Samsung, Lenovo, CATT

Annex for TS38.423

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

### 8.4.8	Handover Report

#### 8.4.8.1	General

The purpose of the Handover Report procedure is to transfer mobility related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

#### 8.4.8.2	Successful Operation

Figure 8.4.8.2-1: Handover Report, successful operation

NG-RAN node1 initiates the procedure by sending the HANDOVER REPORT message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a mobility-related problem was detected.

If the Handover Report Type IE is set to "HO too early" or "HO to wrong cell", then NG-RAN node1 indicates to NG-RAN node2 that, following a successful handover from a cell of NG-RAN node2 to a cell of NG-RAN node1, a radio link failure occurred and the UE attempted RRC Re-establishment or re-connected either at the original cell of NG-RAN node2 (Handover Too Early), or at another cell (Handover to Wrong Cell). The detection of Handover Too Early and Handover to Wrong Cell events is made according to TS 38.300 [9].

If the Handover Report Type IE is set to "Too Late CHO with candidate SCG", then NG-RAN node2 shall deduce that a too late CHO with candidate SCG from a cell of NG-RAN node1 to a cell of NG-RAN node2 occurred. The detection of Too Late CHO with candidate SCG event is made according to TS 38.300 [9].

The HANDOVER REPORT message may include:

-	the Mobility Information IE, if the Mobility Information IE was sent for this handover from NG-RAN node2 (in case the NG-RAN node2 provided it more than once, the most recent Mobility Information IE is included in the HANDOVER REPORT message);

-	the Source cell C-RNTI IE.

-	the CHO Configuration IE, if the CHO Configuration IE was sent for this handover from NG-RAN node2.

If received, NG-RAN node2 uses the above information according to TS 38.300 [9].

If the Handover Report Type IE is set to "Inter-system ping-pong", then NG-RAN node2 shall deduce that a completed handover from a cell of NG-RAN node2 to a cell in another system might have resulted in an inter-system ping-pong and the UE was successfully handed over to a cell of NG-RAN node1 (indicated with Target cell CGI IE).

If the Target cell C-RNTI IE and the Time Since Failure IE are received, NG-RAN node2 uses the information as specified in TS 38.300 [9].

Interaction with the Failure Indication procedure:

If NG-RAN node1 receives a UE RLF Report from an NG-RAN node via the FAILURE INDICATION message, as described in TS 38.300 [9], NG-RAN node1 may also include it in the UE RLF Report Container IE included in the HANDOVER REPORT message.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.17	HANDOVER REPORT

This message is sent by NG-RAN node1 to NG-RAN node2 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong. …, Too Late CHO with candidate SCG) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| Source cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| Target cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of target cell for handover procedure (in NG-RAN node 1). If the Handover Report Type is set to “Inter-system ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node 1 cell. If the Handover Report Type is set to “Too Late CHO with candidate SCG”, it is the candidate cell in the NG-RAN node 2. | YES | ignore |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCell |  | Global Cell Identity 9.2.2.73 | CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the failure | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message or in the SN STATUS TRANSFER message from NG-RAN node 2. | YES | ignore |
| UE RLF Report Container | O |  | UE RLF Report 9.2.2.59 | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |
| CHO Configuration | O |  | 9.2.2.76 |  | YES | ignore |
| Target cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the target NG-RAN node. | YES | ignore |
| Time Since Failure | O |  | INTEGER (0.. 172800, ...) | Corresponds to the TimeSinceFailure IE received from the UE in RLF Report as defined in TS 36.331 [14]. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.xx	SCG Failure Indication

This message is sent by NG-RAN node1 to NG-RAN node2 to report a SCG failure event that occurred while CHO with candidate SCG was configured.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformation message as defined in TS 38.331 [10] | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>