3GPP TSG-RAN WG3 Meeting #127bis	R3-251936

Wuhan, CN, April 7–11, 2025

Agenda item:	 11.3

Source:		Lenovo

Title:		Discussion on AI/ML based CCO

Document for:		Discussion and Decision

1	Introduction

RAN3#125bis meeting agreed that:

Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message.

The Coverage Modification Cause reflects the predicted CCO issue.

WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

Introducing separate time information for future coverage state and predicted CCO issue.

Time for future coverage state: The point in time when the future coverage state will be applied.

Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

RAN3#126 meeting further agreed that:

Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1.

Introducing new IE for Future Coverage Modification List over Xn.

Introducing new IE for Future Coverage Modification Notification over F1.

Introducing new IE for predicted CCO assistance information over F1.

The direction of the information over F1 can be further discussed.

WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE.

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn.

Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds.

RAN3#127 meeting further agreed that:

Each Future Coverage Modification Notification Item included in the Future Coverage Modification Notification List IE has a Future Coverage Modification Cause IE associated to it from DU to CU.

The Future Coverage Modification Cause IE follows the same design of the existing Coverage Modification Cause IE, i.e., it is an ENUMERATED type, but with codepoints “coverage” and “cell edge capacity” only.

Value ‘0’ for the Future Cell Coverage State IE is needed and has the same meaning as cell inactive in legacy CCO.

Whether to exchange time for predicted CCO issue over XnAP needs further discussion.

The maximum value of time for predicted CCO issue and time for future CCO state needs further discussion.

In this contribution, we would further discuss issues related to AI/ML based CCO.

2	Discussion

  - 2.1 AI/ML based CCO over Xn interface

In non-split architecture, it has agreed that:

- Step 0: gNB1 predicts the CCO issue/Coverage Modification Cause.

- Step 1: gNB1 generates the future coverage status based on the predicted CCO issue/Coverage Modification Cause and other information.

- Step 2: gNB1 sends the predicted CCO issue/Coverage Modification Cause and future coverage status to neighbour gNB2.

As captured in [1], gNB1 sends the Time for Future Coverage State to neighbour gNBs, which indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB1 relative to the time of receiving the NG-RAN NODE CONFIGURATION UPDATE message. It is FFS on whether to send time information related to the predicted coverage modification cause over Xn. Regarding CU-DU split architecture, time information for Predicted CCO Issue/Coverage Modification Cause is a relative time from the time of receiving the gNB-CU Configuration Update message over F1, and time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1, the two time information are totally different. To make neighbour gNB2 clearly understand and use the information for deducing the CCO issue predicted in gNB1 and for configuring coverage state of its served cell(s), as well as to enable gNB2 to predict its own CCO issue2 based on the predicted CCO issue1 and future coverage status received from gNB1, it is beneficial to introduce time information for predicted coverage modification cause over Xn.

Proposal 1: Introduce time information for predicted coverage modification cause over Xn.

Since the future CCO state is a prediction or based on a predicted CCO issue, the prediction might be updated or might not be correct. For example, NG-RAN node may predict a future CCO state or CCO issue for 10 seconds later and transfer the related information to a neighbour NG-RAN node, such that the neighbour NG-RAN node would expect a CCO state change after 10 seconds. However, the NG-RAN node may later detects/predicts the CCO issue to happen at 5 seconds sooner than previously predicted. In such case, it is important for the neighbour NG-RAN node to understand the newly detected or predicted CCO issue with corresponding CCO state and the neighbour NG-RAN node should consider the previously received future CCO state and/or predicted CCO issue to be no longer valid.

An example is depicted below to illustrate the point, wherein the NG-RAN node 1 first informs NG-RAN node 2 about a predicted CCO state/issue after time T, but then trigger a CCO state modification before time T due to detected CCO issue. The NG-RAN node 2 should consider the previously received CCO state/issue prediction no longer valid.

Proposal 2: NG-RAN node can update a previously transferred future CCO state and/or CCO issue (e.g., due to newly detected/predicted CCO issue).

Regarding whether/what feedback is needed after CCO issue prediction, the serving NG-RAN node itself can collect UE performance or measurement results from UEs that are under its coverage, so it can monitor whether the predicted CCO issue or future CCO state is valid or not, feedback information per UE (e.g. UE performance or measurement results) from neighbour node(s) is not needed after CCO issue prediction.

Proposal 3: The monitoring of CCO issue prediction relies on the local NG-RAN node itself, e.g., collecting UE performance or measurement result from UEs that are under its coverage.

  - 2.2 AI/ML based CCO over F1 interface

In split-architecture, it has agreed that:

- Step 0: gNB-CU predicts the CCO issue.

- Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

- Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information.

- Step 3: gNB-DU sends the future coverage status to gNB-CU.

- Step 4: gNB-CU sends the predicted CCO issue and future coverage status to neighbour gNB-CU.

Regarding the predicted CCO issue from gNB-CU to gNB-DU, the following two scenarios are agreed:

- gNB-CU1 predicts the CCO issue1 for gNB-DU1 and sends to its gNB-DU1;

- Based on the received predicted CCO issue1 and future CCO state from gNB-CU1, gNB-CU2 will deduce for gNB-DU2 the predicted CCO issue2 as well as affected cells and beams, and send it to its gNB-DU2.

For the issue on whether gNB-CU2 would send the received CCO issue1 predicted by gNB-CU1 to gNB-DU2, regarding gNB-CU2 would predict CCO issue2 for gNB-DU2 based on the predicted CCO issue1 and future coverage status received from gNB-CU1, it makes no sense of gNB-CU2 to redundantly send the CCO issue1 predicted by gNB-CU1 for gNB-DU1 to gNB-DU2.

Proposal 4: gNB-CU2 may not send the CCO issue predicted by gNB-CU1 to its gNB-DU2.

RAN3#126 meeting discussed whether gNB-CU2 sends the received future coverage status from gNB-CU1 to gNB-DU2 but got no consensus. Since gNB-CU2 would predict CCO issue2 for gNB-DU2 based on the predicted CCO issue1 and future coverage status received from gNB-CU1, gNB-DU2 would generate future coverage status to solve the predicted CCO issue2 for gNB-DU2 after receiving the predicted CCO issue2 from gNB-CU2, it is not essential for gNB-CU2 to send the future coverage status for solving the predicted CCO issue1 for gNB-DU1 from gNB-CU1 to gNB-DU2.

Proposal 5: gNB-CU2 may not send the received future coverage status for solving the predicted CCO issue for gNB-DU1 from gNB-CU1 to its gNB-DU2.

RAN3#126 meeting agreed gNB-CU sends predicted CCO issue to gNB-DU, then gNB-DU sends future coverage state generated based on the received predicted CCO issue to gNB-CU, and the direction of the information over F1 can be further discussed.

Concerning whether gNB-DU predicts CCO issue and sends it to gNB-CU, since gNB-CU has more comprehensive perspective and gNB-DU may not always receive L1 measurement report from UE, similar as legacy CCO mechanism, it is better for gNB-CU rather than gNB-DU to predict CCO issue, i.e. gNB-CU sends its Predicted CCO Assistance Information to gNB-DU over F1, but gNB-DU does not predict or send the Predicted CCO Assistance Information to gNB-CU.

Proposal 6: gNB-DU does not predict or send the Predicted CCO Assistance Information to gNB-CU.

With regard to whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information, similar as legacy CCO mechanism, the gNB-DU may resolve the predicted CCO issue concerning own served cell by local action within the OAM configured limits, and the gNB-DU may also take into account information received for other cells when generating future CCO state. Therefore, it is unnecessary for the gNB-CU to generate suggested future CCO state and send it to gNB-DU as assistance information.

Proposal 7: gNB-CU does not generate suggested future CCO state and send it to gNB-DU as assistance information.

3	Conclusion

In this contribution, AI/ML based CCO are discussed. We have the following proposals:

Proposal 1: Introduce time information for predicted coverage modification cause over Xn.

Proposal 2: NG-RAN node can update a previously transferred future CCO state and/or CCO issue (e.g., due to newly detected/predicted CCO issue).

Proposal 3: The monitoring of CCO issue prediction relies on the local NG-RAN node itself, e.g., collecting UE performance or measurement result from UEs that are under its coverage.

Proposal 4: gNB-CU2 may not send the CCO issue predicted by gNB-CU1 to its gNB-DU2.

Proposal 5: gNB-CU2 may not send the received future coverage status for solving the predicted CCO issue for gNB-DU1 from gNB-CU1 to its gNB-DU2.

Proposal 6: gNB-DU does not predict or send the Predicted CCO Assistance Information to gNB-CU.

Proposal 7: gNB-CU does not generate suggested future CCO state and send it to gNB-DU as assistance information.

Reference

[1] R3-247915, (TP for BLCR of TS 38.423) AI/ML enabled CCO