3GPP TSG RAN WG3 #127bis	R3-251951

Wuhan, China, April 7 – April 11, 2025

Agenda item:	17.2

Source: 	Lenovo

Title: 	Discussion on On-demand SSB for SCell

Document for:	Discussion and Decision

Introduction

In RAN#102, a new work item “Enhancements of network energy savings for NR” was agreed [1]. The one objective of the work item is the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we analyse RAN3 impact and high-level solution based on NW-triggered on-demand SSB operation.

Discussion

RAN3#126 has discussed and reached agreement on RRC based signaling for on-demand SSB operation at following:

- For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact.

According to the agreement, if DU makes decision to activate/deactivate OD-SSB, DU may start/stop transmitting OD-SSB to UE first and convey the activation state to CU, then CU transmits it in RRC message to UE via DU. There may be gap between transmission of RRC message and OD-SSB transmission. No critical issue is foreseen.

RAN1 has agreed to support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Scenario #2A refers to “When UE receives SCell activation command”, and Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command. In both Scenario #2A and Scenario #2, if DU makes decision to activate/deactivate OD-SSB, it transmits MAC-CE for on-demand SSB transmission indication to UE and start/stop transmitting OD-SSB to UE, and when and how long to transmit OD-SSB is up to RAN1 decision. There is no RAN3 impact.

For the MAC CE based signaling, RAN3 discusses whether CU node needs to decide to trigger on-demand SSB. If CU determines to trigger on-demand SSB, e.g., on/off the SSB transmission or switch to long or short SSB transmission period, then CU transmits a command to DU, accordingly DU transmits MAC CE message to UE and transmits SSB based on the command.

Proposal 1: RAN3 discusses whether CU node could decide to trigger on-demand SSB for MAC CE based signaling. E.g., CU determines to trigger on-demand SSB, e.g., on/off the SSB transmission, or switch to long or short SSB transmission period.

An NG-RAN node may determine to trigger on-demand SSB transmission based on request/indication by the neighbor NG-RAN node. For example, if an NG-RAN node enters energy saving mode by switch off SSB transmission or switch to long SSB transmission period for a NES cell and it transmits an indication to indicate that the related cell is in this energy saving mode to the neighbour NG-RAN node. The neighbour NG-RAN can trigger On-demand SSB transmission for the cell if the cell needs to be used for layer 3 measurement.

A straightforward solution from RAN3 point of view is to introduce an on-demand SSB trigger IE, which is separate from the Cell and SSB activation IE, to implement the on-demand SSB operation between NG-RAN nodes. In case of CU-DU split, an on-demand SSB trigger IE is also needed on F1 interface from gNB-CU to gNB-DU.

Proposal 2: RAN3 to introduce an on-demand SSB trigger IE, which is separate from the Cell and SSB activation IE, to implement the on-demand SSB operation between NG-RAN nodes and from gNB-CU to gNB-DU.

Conclusion

In summary, we have following observations and proposals for on-demand SSB for SCell in carrier aggregation:

Proposal 1: RAN3 discusses whether CU node could decide to trigger on-demand SSB for MAC CE based signaling. E.g., CU determines to trigger on-demand SSB, e.g., on/off the SSB transmission, or switch to long or short SSB transmission period.

Proposal 2: RAN3 to introduce an on-demand SSB trigger IE, which is separate from the Cell and SSB activation IE, to implement the on-demand SSB operation between NG-RAN nodes and from gNB-CU to gNB-DU.

References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, RAN#102.

- R3-247440, Discussion on On-demand SSB for SCell, Lenovo