3GPP TSG RAN WG3 #127bis	R3-251952

Wuhan, China, April 7 – April 11, 2025

Agenda item:	17.3

Source: 	Lenovo

Title: 	(TP to BL CR 38.423 and 38.473) Discussion on On-demand SIB1 for Idle/Inactive UE

Document for:	Discussion and Decision

Introduction

RAN3 has discussed the on demand SIB1 for Case 2 and reached the agreements：

Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell. - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed - UL WUS configuration is decided by NES Cell DU. - UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. - Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). - The UL WUS configuration will be transferred over Xn via a new class1 defined procedure. - Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. RAN3#126： - The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. - Cell A stores the UL WUS configuration information after it has received it. - A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The procedure uses non UE-associated signalling. - Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures. RAN3#127： It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity. It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx. It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

In this contribution, we further analyse potential RAN3 impacts based on RAN1 and RAN2 progress.

Discussion

UL WUS Configuration Provision over Xn：

If the CU of Cell A receives UL WUS config of a list of NES cells from the CU of NES cells and transmits accept response to the CU of NES cells, Cell A can start to provide UL WUS config for the NES cells, i.e., no need further instruction from NES cell. In the response message, the CU of Cell A needs to notify the CU of NES cell that some NES cells are accepted and other NES cells are rejected for providing the WUS config.

Proposal 1:  Cell A gNB includes a list of NES cells accepted for providing with the UL WUS configuration in UL WUS CONFIGURATION PROVISION RESPONSE message.

Considering the OD-SIB1 operation for an NES Cell is semi-static, it is not necessary to notify the status of OD-SIB1 operation to the Cell A gNB. If the OD-SIB1 for an NES cell is not needed, the NES cell can request Cell A to release the UL WUS configuration for the NES cell by UL WUS CONFIGURATION PROVISION REQUEST message.

Proposal 2: NES cell gNB includes UL WUS configuration release indication for a list of cells in UL WUS CONFIGURATION PROVISION REQUEST message.

UL WUS Configuration Transmission Status Update over Xn：

In addition, Cell A should be able to stop providing UL WUS config for an NES cell. In this case, the CU of Cell A can directly notify the CU of NES cell with the cell ID of the NES cell, so that the NES cell can decide to stop the OD-SIB1 operation.

Proposal 3: Cell A gNB includes a list of NES cells stopped for providing with the UL WUS configuration in UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message.

In addition, RAN3 has agreed that a Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. RAN3 needs to discuss whether to maintain the stored WUS configuration.

- Option 1 is that Cell A gNB releases the UL WUS configuration after signalling the stopping the UL WUS configuration broadcast. It can avoid the additional release signalling from NES gNB. However, NES gNB needs to transmit the complete UL WUS configuration to Cell A gNB again when the UL WUS configuration request is triggered.

- Option 2 is Cell A gNB maintains the UL WUS configuration after signalling the stopping the UL WUS configuration broadcast. It can avoid the complete UL WUS configuration signalling from NES gNB. However, the NES gNB needs to transmit an explicit release request to Cell A if NES cell determines to release the UL WUS configuration.

We observe each option has different benefits, and slightly prefer option 2. In option 2, the UL WUS configuration in cell A is managed in a unified way by the NES gNB.

Observation 1: Considering the OD-SIB1 operation for an NES Cell is semi-static, it is not necessary to notify the status of OD-SIB operation to the Cell A gNB.

Proposal 4: After transmitting stop indication to NES cell gNB, Cell A gNB maintains the UL WUS configuration till receiving UL WUS configuration release indication from the NES cell gNB.

On-demand SIB1 allowed and disable indication over F1：

After the NES cell gNB-CU transmits UL WUS configuration of a list of NES cells to Cell A gNB and receives accept response with a list of accepted NES cells, the NES cell gNB-CU transmits on-demand SIB1 allowed indication for the NES cells to NES cell gNB-DU, then the NES cell gNB-DU considers that the indicated NES cells are allowed to enter the on-demand SIB1 operation. For example, the NES cell can stop periodical SIB1 broadcast and enter OD-SIB1 operation. If NES cell gNB-DU determines not to perform On-demand SIB1 operation anymore, it transmits the OD-SIB1 disable indication to NES cell gNB-CU. Based on the indication, NES cell gNB can further request the cell A to release UL WUS configuration.

Proposal 5:  NES cell gNB-CU includes On-demand SIB1 allowed indication for a list of NES cell to DU in GNB-CU CONFIGURATION UPDATE message.

Proposal 6:  If NES cell gNB-DU determines not to perform On-demand SIB1 operation, it transmits On-demand SIB1 disable indication for a list of cells in the GNB-DU CONFIGURATION UPDATE message to NES cell gNB-CU.

Transfer UL WUS configuration in new SIB over F1：

Based on RAN2 agreement, a NES cell can include neighbouring NES cell’s UL WUS configuration, and the NES cell’s UL WUS configuration is included in a new SIB (including its own UL WUS configuration). Therefore, the UL WUS configuration needs to be exchanged between NES cells as that between Cell A and NES cell. The agreed class 1 procedure about UL WUS configuration transmission should be extended to the Xn between the NES cells.

In case of CU-DU split case, after the CU of Cell A or NES cell accepting the UL WUS configuration, it includes the received UL WUS configuration in a new SIB and transfers it to the DU of Cell A or NES cell. As for the IE design, RAN3 can reuse gNB-CU System Information IE.

Proposal 7:  gNB-CU includes the UL WUS configuration in a new SIB and transfers it to gNB-DU in gNB-CU System Information IE.

on-demand SIB1 procedure：

The OD-SIB1 mechanism is similar as on-demand other SI. RAN2 agreed to use the existing Msg 1 based on-demand procedure for on-demand SIB1 acquisition. If receiving request via MSG1, DU can response via MSG2 and broadcast SIB1. This procedure has no impact on RAN3.

Observation 2: DU broadcasts SIB1 upon receiving MSG1 based on-demand SIB1 request, there’s no RAN3 impact foreseen.

Conclusion

In summary, we have following observations and proposals for on-demand SIB1 for Idle/Inactive UE:

Observation 1: DU broadcast SIB1 upon receiving MSG1 based on-demand SIB1 request, there’s no RAN3 impact foreseen.

Observation 2: Considering the OD-SIB1 operation for an NES Cell is semi-static, it is not necessary to notify the status of OD-SIB operation to the Cell A gNB.

Proposal 1:  Cell A gNB includes a list of NES cells accepted for providing with the UL WUS configuration in UL WUS CONFIGURATION PROVISION RESPONSE message.

Proposal 2: NES cell gNB includes UL WUS configuration release indication for a list of cells in UL WUS CONFIGURATION PROVISION REQUEST message.

Proposal 3: Cell A gNB includes a list of NES cells stopped for providing with the UL WUS configuration in UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message.

Proposal 4: After transmitting stop indication to NES cell gNB, Cell A gNB maintains the UL WUS configuration till receiving UL WUS configuration release indication from the NES cell gNB

Proposal 5:  NES cell gNB-CU includes On-demand SIB1 allowed indication for a list of NES cell to DU in GNB-CU CONFIGURATION UPDATE message.

Proposal 6:  If NES cell gNB-DU determines not to perform On-demand SIB1 operation, it transmits On-demand SIB1 disable indication for a list of cells in the GNB-DU CONFIGURATION UPDATE message to NES cell gNB-CU.

Proposal 7:  gNB-CU includes the UL WUS configuration in a new SIB and transfers it to gNB-DU in gNB-CU System Information IE.

Text Proposal for BL CR for TS 38.423 – on top of R3-250912

*************** Start of TP ***************

Editor’s Note: The procedure, message and the IE names are FFS

Editors Note: How to address the role of the NG-RAN node1 and NG-RAN node2 to be further discussed.

- 8.x.x	UL WUS Configuration Provision (FFS)

- 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2 “and request NG-RAN node2 to transmit UL WUS configuration information (FFS)”.

“The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to discontinue transmission of UL WUS configuration.  (FFS)”

The procedure uses non UE-associated signaling.

- 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

If the “start with UL WUS Configuration” (details on this codepoint/choice of “start with UL WUS Configuration” are FFS) is included in the UL WUS CONFIGURATION PROVISION REQUEST (Naming FFS) message, the Cell A gNB (Naming FFS) shall, if supported, broadcast UL WUS Configuration (Naming FFS) in SIBx (SIB naming FFS) and reply with the UL WUS CONFIGURATION PROVISION RESPONSE (Naming FFS) message including UL WUS Config Accepted Cell list  IE. The Cell A gNB (Naming FFS) stores the UL WUS configuration information (Naming FFS).

“Cell A stores the UL WUS configuration information after it has been requested to be discontinued (FFS)”

If the “UL WUS Config release cell list” is included in the UL WUS CONFIGURATION PROVISION REQUEST message,  Cell A gNB removes the UL WUS configuration information of the indicated cells and stop providing UL WUS Configuration in SIBx for the indicated cells.”

- 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the Cell A gNB (Naming FFS) cannot broadcast UL WUS Configuration in SIBx (Naming FFS), or a failure occurs during UL WUS Configuration Provision (Naming FFS), the Cell A gNB (Naming FFS) shall send UL WUS CONFIGURATION PROVISION FAILURE (Naming FFS) message. The message shall contain the Cause IE with an appropriate value.

- 8.x.x.4	Abnormal Conditions

Void.

- 8.x.y	UL WUS Configuration Transmission Status Update (FFS)

- 8.x.y.1	General

The purpose of the UL WUS Configuration Transmission Status Update procedure is to enable an NG-RAN node2 to transmit the status of stop providing UL WUS configuration information to NG-RAN node1 .

The procedure uses non UE-associated signalling.

- 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update, successful operation

If Cell A gNB decides to stop providing UL WUS configuration information for an NES cell, Cell A gNB transmits with the UL WUS CONFIGURATION TRANSMITIION UPDATE message including UL WUS config Stopped cell List IE.

If UL WUS config Stopped cell List IE is included in the UL WUS CONFIGURATION TRANSMITIION UPDATE message, the NES cell gNB shall, consider the Cell A gNB has stopped for providing UL WUS Configuration for the indicated cell.

- 8.x.y.3	Unsuccessful Operation

Not applicable.

- 8.x.y.4	Abnormal Conditions

Void.

*************** End of TP ***************

5. TP for TS 38.473 – on top of R3-247898

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Editor’s Note: The procedure, message and the IE names are FFS.

- 8.2.3	F1 Setup

- 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the F1 SETUP REQUEST message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the UL WUS Configuration Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

- 8.2.4	gNB-DU Configuration Update

- 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

- 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the UL WUS Configuration Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

If the On-demand SIB1 disabled cell list IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider whether the on-demand SIB1 operation for the indicated cells is disabled.

- 8.2.5	gNB-CU Configuration Update

- 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

- 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the Cells Allowed to be On-Demand SIB1 List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to enter the on-demand SIB1 operation within the indicated cells.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>