3GPP TSG-RAN WG3 Meeting #127bis									R3-252002

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	20.2

Source:	Huawei

Title:	(TP for AI/ML BLCR to TS 38.455) Discussion on RAN3 impacts for NG-RAN node-assisted AI/ML positioning (Case 3a)

Document for:	Discussion and decision

# Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” – latest WID in [1]. To be specific, the discussion was on the specification support for AI/ML-based positioning accuracy enhancements.

This contribution focuses on AI/ML assisted positioning Case 3a – being the only case of AI/ML positioning that have RAN3 impacts, together with Direct AI/ML positioning Case 3b, discussed in a separate contribution [2] – and provides proposals based on the latest progress in RAN3.

# Discussion

In the following we further discuss aspects related to model training, model inference and model monitoring for AI/ML positioning Case 3a (i.e., NG-RAN node-assisted positioning with a gNB-side model).

In the last meeting RAN3 discussed technical aspects for Case 3a only, and the following agreements and open issues were captured in the meeting minutes [3]:

Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement. For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner. For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs. WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB. To be continued: Stage 2 (TS 38.305) text proposals for case 3a data collection. Companies are encouraged to consolidate solutions, if possible.

For AI/ML positioning, the preliminary Rel-18 study phase – whose outcome is summarized in TR 38.843 [6] – identified that, for Case 3a, the output of the AI/ML model hosted in the gNB is an intermediate measurement to be used for successive determination of the UE location at the Location Management Function (LMF).

In order to assess the RAN3 impacts for supporting AI/ML positioning we will discuss model training, model inference and model monitoring aspects.

## 2.1 Model training considerations

As per RAN1 agreements below, a training data sample is composed of two parts, a “measurement” (Part A in RAN1 discussions) and a “ground-truth label” (Part B in RAN1 discussions). For Part A RAN1 agreed that it is generated by the TRP/gNB (for both Case 3a and Case 3b), that is, NG-RAN; while Part B can be generated at least by the LMF for Case 3a. RAN1 also agreed on the components that could be included in Part A and Part B of the training data sample (valid for both Case 3a and Case 3b).

******** RAN1#116bis ******** Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. ******** RAN1#118bis ******** Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; FFS: details on how to associate quality indicator to timing information

For Case 3a, for both Part A and Part B, RAN3 also reached the following agreements in RAN3#125bis meeting [5]:

Case 3a agreements: - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1.

Based on the above, RAN3 had a discussion on how the gNB can request the LMF to provide Part B information, and whether this request will also trigger a UE positioning session for the UE for which Part B information is provided, but no conclusion was reached. Therefore RAN3 needs to discuss further, in light of the agreements from last meeting [3]:

Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement. For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner. For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs. WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB.

The reason for using both data collected at the gNB side – note that the “gNB” here is the one hosting the AI/ML model to be trained – from both served and non-served UEs (where a served UE is a UE which is served by gNB hosting the model to be trained), as well as to consider the “proactive” approach for data collection in addition to the “opportunistic” one, is to ensure not only a sufficient amount of data for training the AI/ML model, but also to avoid bias effects that may negatively impact the whole training process due to the use of data from served UEs only, hence ensuring some kind of “data diversity”. Moreover, the “proactive” approach may be useful for determining the “right” UEs for training, that is, UEs located in different coverage areas of the gNB, having specific UE capabilities or transmitting UL SRS with specific characteristics. We will discuss “opportunistic” and “proactive” data collection approaches in more details in the following subsections.

## 2.1.1 “Opportunistic” data collection for training

The “opportunistic” data collection approach exploits ongoing positioning sessions for the UE, and can be summarized as in Figure 1. Note that the call flow uses generic messages over NRPPa to simply explain the mechanism, that is, it does not discuss whether the messages are new or existing ones. The “opportunistic” approach works for both served and non-served UEs, where the distinction between served and non-served UEs is made from the perspective of the gNB hosting the model to be trained, meaning that training data samples can be from UEs which have different serving gNBs (and hence they are non-served UEs from the perspective of the gNB that has a model to train) provided that the LMF has configured TRP(s) from the gNB hosting the model to obtain measurements.

Figure 1 - "Opportunistic" data collection for model training in gNB.

- There is an ongoing positioning session for a UE, by using existing positioning methods, for which the LMF may obtain UE’s location information (Part B) that could be useful for training a model at the gNB. The UE can be either a served UE or a non-served UE;

- The LMF sends a NRPPa message to the gNB to inform the gNB that the LMF can provide data for training the model in gNB (if the gNB is interested in receiving such data);

- The gNB replies with a NRPPa message to express its interest in receiving training data information (Part B);

- The LMF selects a UE for which it can provide UE’s location information (Part B) in step 0.;

- The LMF sends a NRPPa message to the gNB to trigger Part A generation corresponding to the UE selected by the LMF in step 3.;

- The gNB replies with a NRPPa message to acknowledge the request message sent by the LMF in step 4. and stores the configuration provided in the measurement request to then generate Part A for the UE selected by the LMF;

NOTE: it is up to gNB implementation to fail the procedure if the UE selected by the LMF is not a proper UE, meaning that its corresponding Part A and Part B information may not be useful for model training.

- The LMF sends a NRPPa message to the gNB to provide Part B information for the selected UE;

- Part A is generated for the selected UE;

- Paired Part A and Part B are used for model training.

Proposal 1: RAN3 to agree to take the call flow in Figure 1 for the “opportunistic” data collection in gNB as the baseline for normative work.

## 2.1.1 “Proactive” data collection for training

Based on some pre-meeting coordination with some companies, it seems to us that further discussion in RAN3 is needed concerning the actual meaning of the “proactive” approach for data collection at the gNB. From such discussions, the following options can be considered for discussions:

- Option 1: the “proactive” approach assumes that there is not any ongoing positioning sessions (as per RAN3 agreement from last meeting) but it is the gNB that indicates to the LMF that it is interested in receiving training data for a specific UE, meaning that the gNB makes an initial selection of the UE. Then, after the user consent has been checked in 5GC, if there is user consent the LMF initiates a positioning session for UE indicated by the gNB by potentially involving multiple gNBs (including the gNB hosting the model).

- Option 2: the “proactive” approach still assumes that there is not any ongoing positioning sessions but, following an indication from the gNB that the gNB itself needs training data, it is anyway the LMF that selects a UE (which could be a served or a non-served UE, and for which there is user consent) and then the LMF initiates a positioning session for the selected UE by potentially involving multiple gNBs (including the gNB hosting the model). This also means that the gNB never indicates that is wants data from the LMF associated to a specific UE, but it simply informs the LMF that it is interested in receiving training data.

Figure 2 shows the “proactive” data collection Option 1 and, as for the “opportunistic” approach, the call flow uses generic messages over NGAP and NRPPa to simply explain the mechanism, that is, it does not discuss whether the messages are new or existing ones.

Figure 2 - "Proactive" data collection at the gNB - Option 1.

- The gNB hosting the model to be trained selects a UE for which it is interested in receiving Part B information from the LMF; since it is the gNB which selects the UE, this means that only served UEs can be considered;

- The gNB sends a NGAP message to the AMF to express its interest in data collection for the UE selected by the gNB;

- The user consent is checked in 5GC (either in AMF or LMF) for the UE selected by the gNB;

- If UE’s user consent is available from step 2., the AMF triggers a location service request to the LMF to trigger a positioning session for the selected UE;

- A positioning session is initiated by the LMF for the selected UE, by using existing positioning methods, during which the LMF may obtain UE’s location information that could be useful for training a model (Part B);

- The LMF sends a NRPPa message to the gNB to trigger Part A generation corresponding to the selected UE;

- The gNB replies with a NRPPa message to acknowledge the request message sent by the LMF in step 5. and stores the configuration provided in the measurement request to then generate Part A for the UE selected by the gNB;

- The LMF sends a NRPPa message to the gNB to provide Part B information for the selected UE;

- Part A is generated for the selected UE;

- Paired Part A and Part B are used for model training.

Figure 3 shows the “proactive” data collection Option 2 and, as for the other approaches, the call flow uses generic messages over NRPPa to simply explain the mechanism, that is, it does not discuss whether the messages are new or existing ones.

Figure 3 - "Proactive" data collection in gNB - Option 2.

- The LMF sends a NRPPa message to the gNB to inform the gNB that the LMF can provide data for training the model in gNB (if the gNB is interested in receiving such data);

- The gNB replies with a NRPPa message to express its interest in receiving training data information (Part B);

- The LMF selects a UE (with user consent) for which it needs to obtain UE’s location information (Part B); the UE can be either a served UE or a non-served UE;

- A positioning session is initiated by the LMF for the selected UE, by using existing positioning methods, during which the LMF may obtain UE’s location information that could be useful for training a model (Part B);

- The LMF sends a NRPPa message to the gNB to trigger Part A generation corresponding to the UE selected by the LMF in step 2.;

- The gNB replies with a NRPPa message to acknowledge the request message sent by the LMF in step 4. and stores the configuration provided in the measurement request to then generate Part A for the UE selected by the LMF;

NOTE: it is up to gNB implementation to fail the procedure if the UE selected by the LMF is not a proper UE, meaning that its corresponding Part A and Part B information may not be useful for model training.

- The LMF sends a NRPPa message to the gNB to provide Part B information for the selected UE;

- Part A is generated for the selected UE;

- Paired Part A and Part B are used for model training.

In our view, Option 1 has the benefit that it allows the gNB to indicate UEs which can be considered as the “right UEs” to train a model: since the model is hosted in the gNB – and hence the gNB knows the details of the model to be trained – a proper initial UE selection by the gNB should be done in such a way to ensure sufficient data diversity of the training information. The main drawback, however, is that the initial UE selection by the gNB is restricted to served UEs only, due to the fact that the gNB has no means to indicate to the LMF that it wishes to receive data associated to a UE served by another gNB.

With Option 2, the main benefit is that the data collection procedure is still kept under the control of the LMF without any gNB involvement, that is, similarly to the “opportunistic” data collection, the gNB only informs the LMF that it is interested in receiving training data. One may argue that the data diversity requirement could not be fulfilled, as the UE selection from the LMF lacks information on the details of the model to be trained. In our view this is not a big issue because, to ensure data diversity, the LMF may select the “right UEs” for training by considering that the LMF knows the UE location (within the coverage area of the gNB, a well as in neighbor gNBs) and it also knows the gNB which is interested in receiving the training data. Moreover, it is still possible for the gNB to fail the procedure if the UE selected by the LMF is not a “right UE” for the model hosted in the gNB, hence its corresponding Part A and Part B do not need to be made available at the gNB – this consideration also applies to the “opportunistic” data collection.

In addition to that, the LMF may select both served and non-served UEs: for instance, if a non-served UE is selected, the LMF can configure SRS measurements also to the TRPs of the gNB not serving the UE, and these measurements could serve the purpose of training a model hosted in this gNB, therefore ensuring an higher degree of data diversity compared to Option 1.

Another aspect that should be considered is the protocol impact: for Option 1, we think that some changes in the NGAP protocol are needed so that the gNB can indicate to the AMF which is the UE for which it is interested in receiving training data. While for Option 2 it seems to us that the protocol impact could be kept limited to NRPPa only.

The Table 1 below shows a comparison of the options for “proactive” data collection:

Table 1 - Comparison of the options for the "proactive" data collection in the gNB.

| Option for “proactive” data collection | Entity in charge of UE selection | UEs that can be selected | Protocol impact |
| --- | --- | --- | --- |
| Option 1 | gNB (UE selection, to be confirmed by 5GC after user consent check) | Only served UEs | NGAP and NRPPa |
| Option 2 | LMF (if there is user consent for the UE) | Served UEs and non-served UEs | NRPPa |

Proposal 2: RAN3 to discuss the following options to achieve a common understanding of what “proactive” data collection is:

- Option 1: gNB selects the UE;

- Option 2: LMF selects the UE.

Based on Table 1, we suggest taking Option 2 as the baseline for the normative work of the “proactive” data collection in gNB, which ensures higher “data diversity” and limited specification impact compared to Option 1. It also maximizes the common aspects with the “opportunistic” data collection mechanism.

Proposal 3: RAN3 to select Option 2 (UE selection by LMF) as the baseline for the normative work related to the “proactive” data collection in the gNB.

Concerning the WA above, our understanding is that making Part A available outside the gNB (i.e., in the LMF) may be useful for correlating (i.e., pairing) Part A and Part B of the training data sample, so that the LMF can provide the gNB with correlated (paired) Part A and Part B. In our view, this seems to go against the following RAN1 agreement from RAN1#120:

******** RAN1#120 ******** Agreement - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above.

Let’s “isolate” Case 3a from the above agreement: Part A is generated in gNB, while Part B is generated in LMF. From RAN3#125bis meeting, training for Case 3a is in the gNB [5]:

The AI/ML model (inference and training functions) is at the gNB.

Therefore it is Part B only that needs to be transmitted to the gNB. Based on RAN1 agreement above, the time stamp of Part B needs to be provided as well to the gNB for pairing with Part A (which is already available in the gNB, hence not transmitted). As a consequence we think that there is no need to send Part A to the LMF, and propose to basically confirm the WA from the last meeting with some rewording:

Proposal 4: RAN to agree that, for case 3a data collection, Part A is known internally by the gNB and is not signalled outside the gNB.

To complete the picture, we may recall the RAN1 agreement from RAN1#120 for the definition of time stamp for the label (i.e., Part B), refer to the highlighted text in the RAN1 agreement:

******** RAN1#120 ******** Agreement For AI/ML based positioning, - When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed. - When label is to be reported/sent, time stamp of label is the time instance for which the label is valid.

For Part A, instead, since it is generated by the TRP/gNB, there is no RAN3 impact in making this information available in the entity performing AI/ML model training, i.e., the NG-RAN (if the AI/ML model training is located in the NG-RAN). Part A’s content for Case 3a is still under discussion in RAN1 and our understanding is that it reflects the content of the measurement report provided by the gNB to the LMF which includes the inference results (discussed in Section 2.2).

Even though RAN1 didn’t agree on a specific indication concerning the user data privacy requirements for the provisioning of Part B from the LMF to the NG-RAN for Case 3a, we think that a similar note from RAN1 about the user data privacy as included in the description of Option A for the AI/ML model monitoring options for Case 3a should also be considered for the model training as well – note that the model monitoring considerations for Case 3a can be found in Section 2.3 of this contribution.

******** RAN1#118 ******** Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

In addition, RAN1 agreed that the purpose for which a certain information is provided does not need to be indicated in the specifications. In our view, this is in line with the principle which drove the specification of “AI/ML for NG-RAN” since Rel-18 (refer to TS 38.300 clause 16.20.2). Therefore, it is suggested that the same principle is also considered during the discussions in RAN3 about the impacts for supporting Case 3a. Based on the above, we have the following proposal:

Proposal 5: For Case 3a, the signalling procedure from LMF to gNB for transferring Part B information of the training data sample shall fulfil the following:

- Provisioning of Part B ensures and preserves the corresponding user data privacy requirements.

- Purpose, such as “training data collection”, will not necessarily be specified in RAN3 specification.

Proposal 6: For Case 3a, there is no RAN3 impact for Part A of the training data sample to be made available in the NG-RAN (if AI/ML model training is located in the NG-RAN).

In our understanding, the AI/ML model training function can also be located in the OAM. This is based on what has been reported in TR 38.843 clause 7.2.4, where the training data generation and termination points are identified for the gNB-side model as follows [4]:

### 7.2.4 Positioning accuracy enhancements **** Skip non-relevant text **** For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: **** Skip non-relevant text **** - For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.  Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress. **** Skip non-relevant text ****

Both possibilities concerning the termination point for training data – gNB or OAM – are aligned with the deployment scenarios also specified by RAN3 as part of the “AI/ML for NG-RAN” WI in Rel-18 (refer to TS 38.300 clause 16.20.2). Therefore, we have the following proposal:

Proposal 7: For AI/ML assisted positioning Case 3a the AI/ML model training can also be located in the OAM other than in the gNB.

## 2.2 Model inference considerations

For Case 3a, to enable the AI/ML model in the gNB to generate measurements for successive determination at LMF side of the UE location, some input data needs to be provided to the AI/ML model itself which will then generate a proper output. In previous RAN1 meetings the following agreements were achieved concerning the AI/ML model input information:

******** RAN1#116 ******** Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. Note: The use of SRS for MIMO resource is transparent to UE. ******** RAN1#118 ******** Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

Based on the above agreements, our understanding is that the AI/ML model in the gNB uses UL SRS-based measurements as model input for inference purposes (other than for other procedures such as training and monitoring). And, as per Proposal 3 above and based on RAN1 agreements, the purpose for which information are provided and used shall not be indicated in RAN3 specifications.  RAN1 made a conclusion in RAN1#118 meeting that the existing procedures can be reused for the SRS configuration, even though the details of the existing parameters (e.g., values) can be further discussed.

Proposal 8: The AI/ML model in the gNB for Case 3a uses SRS-based measurements as model input also for inference purposes, by reusing the existing procedures for the SRS configuration.

With regards to the AI/ML model output for Case 3a, i.e., information that could be reported from the NG-RAN to the LMF, RAN1 agreed that at least LOS/NLOS indicator and/or timing information are supported for reporting.

******** RAN1#116 ******** Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

For both pieces of information RAN1 also considered the existing methods already specified, that is, soft/hard indicator for the LOS/NLOS indicator, and UL RTOA (Uplink Relative Time of Arrival) or gNB Rx-Tx time difference for the timing information.

In the last two meetings RAN1 agreed further aspects related to the content of the measurement report from gNB to LMF for Case 3a, as follows:

******** RAN1#118bis ******** Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. ******** RAN1#119 ******** Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements ******** RAN1#120 ******** Conclusion For AI/ML based positioning Case 3a, regarding the time stamp in a measurement report from gNB to LMF, - Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective.

The first agreement above from RAN1 is related to the presence (mandatory/optional) of components (Information Elements, IEs) that constitute the measurement report from gNB to LMF in case timing information (either UL RTOA or gNB Rx-Tx difference) is reported as part of the AI/ML model output for Case 3a. In the RAN1 agreement there is also a Note stating that the final decision of “mandatory” or “optional” presence of each field is up to RAN3.

By checking the latest NRPPa specification [6], the following can be excerpted:

#### 9.1.4.4	MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |

#### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

Based on the current NRPPa specification, in our understanding the presence of IEs (timing information, quality of the timing information, time stamp) in the measurement report from gNB to LMF in case timing information (either UL RTOA or gNB Rx-Tx difference) is reported is already aligned with the presence as suggested in the RAN1 agreement. Therefore, we think RAN3 should confirm what RAN1 already indicated for the presence of IEs in the measurement report for Case 3a, which has no additional standard impact on NRPPa. This is also motivated by the following agreement and common understanding from the RAN3#126 meeting [4]:

Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used.

Proposal 9: RAN3 to confirm the RAN1 agreement that, when timing information is reported for Case 3a, the presence of IEs in the measurement report from gNB to LMF is as in legacy NRPPa, that is:

- Timing information: mandatory IE

- Quality of the timing information: optional IE

- Time stamp: mandatory IE.

Another FFS from RAN3#126 meeting is related on whether the LMF should be made aware of the fact that measurements reported by the gNB are generated by an AI/ML model or they are generated as in legacy positioning [4]:

FFS if LMF should be aware that the reported measurements are AI/ML generated.

In light of the approach used so far by RAN3 to strive to reuse the existing procedures and IEs for positioning measurements, we believe that it would be useful for the LMF to be able to distinguish whether the reporting is related to predicted information. We think that a simple flag “predicted” introduced in the existing TRP Measurement Result IE could serve the purpose, as it is not only a simple solution but also more flexible in case RAN1 decides in the future to include additional predicted information in the reporting to the LMF.

Proposal 10: RAN3 to agree to introduce a flag in the existing TRP Measurement Result IE to allow the LMF to distinguish whether the measurement report from the gNB is related to predicted information. The flag can be an ENUMERATED IE type with codepoint “true”.

## 2.3 Model monitoring considerations

Concerning the AI/ML model monitoring, in RAN3#125bis a Working Assumption (WA) based on what RAN1 identified as Option A for the model monitoring metric calculation was reached [5]:

Case 3a agreements: - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

For model monitoring in Case 3a, RAN1 identified two options:

- Option A, where the NG-RAN (hosting the AI/ML model) performs the monitoring metric calculation – this is the option covered in the WA from the last RAN3 meeting;

- Option B, where the LMF performs the monitoring metric calculation for the model hosted in NG-RAN.

In RAN3#126 meeting concluded this discussion by agreeing to turn the above reported WA into an agreement [2]:

Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

On top of that, we think RAN3 should agree on a general principle concerning the location of the model monitoring function with respect to the entity which is in charge of the AI/ML model inference or, in general, of taking a proper “monitoring decision” for the model based on inference results (e.g., (de-)activation, fallback to legacy positioning methods, etc.). In our view, it would be better that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation, which is also the entity which has knowledge of the model being used. This because in our understanding the “monitoring decision” should be based on the model-specific monitoring metric calculated by considering two main components, that is, the inference results (i.e., model output) and the ground-truth labels.

Proposal 11: RAN3 to agree the general principle that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation and takes the corresponding monitoring decision.

A TP for the AI/ML BLCR to TS 38.455 is in the Annex.

# Conclusions

Model training considerations

Proposal 1: RAN3 to agree to take the call flow in Figure 1 for the “opportunistic” data collection in gNB as the baseline for normative work.

Proposal 2: RAN3 to discuss the following options to achieve a common understanding of what “proactive” data collection is:

- Option 1: gNB selects the UE;

- Option 2: LMF selects the UE.

Proposal 3: RAN3 to select Option 2 (UE selection by LMF) as the baseline for the normative work related to the “proactive” data collection in the gNB.

Proposal 4: RAN to agree that, for case 3a data collection, Part A is known internally by the gNB and is not signalled outside the gNB.

Proposal 5: For Case 3a, the signalling procedure from LMF to gNB for transferring Part B information of the training data sample shall fulfil the following:

- Provisioning of Part B ensures and preserves the corresponding user data privacy requirements.

- Purpose, such as “training data collection”, will not necessarily be specified in RAN3 specification.

Proposal 6: For Case 3a, there is no RAN3 impact for Part A of the training data sample to be made available in the NG-RAN (if AI/ML model training is located in the NG-RAN).

Proposal 7: For AI/ML assisted positioning Case 3a the AI/ML model training can also be located in the OAM other than in the gNB.

Model inference considerations

Proposal 8: The AI/ML model in the gNB for Case 3a uses SRS-based measurements as model input also for inference purposes, by reusing the existing procedures for the SRS configuration.

Proposal 9: RAN3 to confirm the RAN1 agreement that, when timing information is reported for Case 3a, the presence of IEs in the measurement report from gNB to LMF is as in legacy NRPPa, that is:

- Timing information: mandatory IE

- Quality of the timing information: optional IE

- Time stamp: mandatory IE.

Proposal 10: RAN3 to agree to introduce a flag in the existing TRP Measurement Result IE to allow the LMF to distinguish whether the measurement report from the gNB is related to predicted information. The flag can be an ENUMERATED IE type with codepoint “true”.

Model monitoring considerations

Proposal 11: RAN3 to agree the general principle that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation and takes the corresponding monitoring decision.

A TP for the AI/ML BLCR to TS 38.455 is in the Annex.

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#107

- R3-252001, (TP for AI/ML BLCR to TS 38.455) Discussion on RAN3 impacts for Direct AI/ML positioning (Case 3b), Huawei

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (v18.0.0), Rel-18

- 3GPP TS 38.300, NR and NG-RAN Overall Description; Stage 2, Rel-18

- 3GPP TS 38.455, NG-RAN; NR Positioning Protocol A (NRPPa), Rel-18

# Annex – TP for the AI/ML BLCR to TS 38.455

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | Ignore |
| >Predicted Indication (NAME FFS) | C-ifMeasurements |  | ENUMERATED(true,…) | If present, it indicates predicted TRP Measurement Result | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

| Condition | Explanation |
| --- | --- |
| ifMeasurements | This IE shall be present if the Measured Results Value Time of the Measured Result Item IE is set to either “UL RTOA Measurement” or “gNB Rx-Tx Time Difference”. |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

*** skip unmodified text ***

id-SRSPosPeriodicConfigHyperSFNIndex

id-PredictedIndication(NAME-FFS)

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- P

PathlossReferenceInformation ::= SEQUENCE {

pathlossReferenceSignal					PathlossReferenceSignal,

iE-Extensions					ProtocolExtensionContainer { { PathlossReferenceInformation-ExtIEs } } OPTIONAL,

...

}

*** skip unmodified text ***

PointA  ::= INTEGER (0..3279165)

PredictedIndication(NAME-FFS) ::= ENUMERATED (true,...)

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- T

TAC ::= OCTET STRING (SIZE(3))

*** skip unmodified text ***

TRPMeasurementQuantities ::= SEQUENCE (SIZE (1..maxnoPosMeas)) OF TRPMeasurementQuantitiesList-Item

TRPMeasurementQuantitiesList-Item ::= SEQUENCE {

tRPMeasurementQuantities-Item		TRPMeasurementType,

timingReportingGranularityFactor	INTEGER (0..5) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRPMeasurementQuantitiesList-Item-ExtIEs}}		OPTIONAL,

...

}

TRPMeasurementQuantitiesList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ID id-TimingReportingGranularityFactorExtended	CRITICALITY ignore EXTENSION TimingReportingGranularityFactorExtended PRESENCE optional},

...

}

*** skip unmodified text ***

TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoPosMeas)) OF TrpMeasurementResultItem

TrpMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				TrpMeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

TrpMeasurementResultItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSResourcetype	CRITICALITY ignore EXTENSION SRSResourcetype PRESENCE optional}|

{ ID id-ARP-ID		CRITICALITY ignore EXTENSION ARP-ID 	PRESENCE optional}|

{ ID id-LoS-NLoSInformation		CRITICALITY ignore EXTENSION LoS-NLoSInformation		PRESENCE optional }|

{ ID id-Mobile-TRP-LocationInformation	CRITICALITY ignore EXTENSION Mobile-TRP-LocationInformation	PRESENCE optional }|

{ ID id-MeasuredFrequencyHops			CRITICALITY ignore EXTENSION MeasuredFrequencyHops					PRESENCE optional }|

{ ID id-AggregatedPosSRSResourceID-List	CRITICALITY ignore EXTENSION AggregatedPosSRSResourceID-List	PRESENCE optional }|

{ ID id-MeasBasedOnAggregatedResources			CRITICALITY ignore EXTENSION MeasBasedOnAggregatedResources PRESENCE optional }|

{ ID id-PredictedIndication(NAME-FFS)	CRITICALITY ignore EXTENSION PredictedIndication(NAME-FFS)	PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

*** skip unmodified text ***

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-SRSPosPeriodicConfigHyperSFNIndex							ProtocolIE-ID ::= 165

id-PredictedIndication(NAME-FFS)								ProtocolIE-ID ::= aaa

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>